3GPP TSG-RAN WG3 Meeting #119b-e	R3-231963

Online, 17th – 26th April, 2023

Title: 	(TP to 38.473) Introduction on NR Redcap enhancement

Source: 	Huawei, ZTE, Nokia, Nokia Shanghai Bell

Agenda item:	21.2

Document Type:	Discussion

- Introduction

Include the following agreement into the TS38.473

F1AP agreements:

Extend the NR Paging eDRX Information for RRC INACTIVE IE F1AP 9.3.1.259:

- add new codepoints in the NR Paging eDRX Cycle Inactive IE: ENUMERATED (hfquarter, hfhalf, hf1, …,hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024)

- add the NR Paging Time Window IE ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32,…)

- Text proposal for TS 38.473

------Start of the first change-------

#### 9.3.1.259	NR Paging eDRX Information for RRC INACTIVE

This IE indicates the NR Paging eDRX parameters for RRC_INACTIVE as defined in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR Paging eDRX Cycle Inactive | M |  | ENUMERATED (hfquarter, hfhalf, hf1, …, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024) | TeDRX,RAN defined in TS 38.304 [24]. Unit: [number of hyperframes]. |  |  |
| NR Paging Time Window Inactive | O |  | ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32,…) | Unit: [1.28 second]. | YES | ignore |

-----------------Start of the next Change-------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

//skip the unchanged part

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-NRPaging-Time-Window-Inactive,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

maxNRARFCN,

maxnoofErrors,

Editor’s Note:	FFS if the NRPaging-time-Window defined in ASN.1 can be re-used instead of a new NRPaging-Time-Window-Inactive

//skip the unchanged part

NRPagingeDRXInformation ::= SEQUENCE {

nrpaging-eDRX-Cycle-Idle		NRPaging-eDRX-Cycle-Idle,

nrpaging-Time-Window			NRPaging-Time-Window												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {NRPagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Idle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4,

hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NRPaging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...,

s17, s18, s19, s20, s21,

s22, s23, s24, s25, s26,

s27, s28, s29, s30, s31, s32

}

NRPagingeDRXInformationforRRCINACTIVE ::= SEQUENCE {

nrpaging-eDRX-Cycle-Inactive		NRPaging-eDRX-Cycle-Inactive,

iE-Extensions			ProtocolExtensionContainer { { NRPagingeDRXInformationforRRCINACTIVE-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformationforRRCINACTIVE-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRPaging-Time-Window-Inactive		CRITICALITY ignore EXTENSION NRPaging-Time-Window-Inactive	PRESENCE optional},

...

}

NRPaging-eDRX-Cycle-Inactive ::= ENUMERATED {

hfquarter, hfhalf, hf1,

...,

hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024

}

NRPaging-Time-Window-Inactive ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

s17, s18, s19, s20, s21, s22,

s23, s24, s25, s26, s27, s28, s29,

s30, s31, s32, ...

}

NonDynamic5QIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			INTEGER (1..127)				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NonDynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

//skip the unchanged part

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-NRPaging-Time-Window-Inactive					ProtocolIE-ID ::= XXX

END

-- ASN1STOP

------End of the changes-------