3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231969

Electronic, 17 - 26 April 2023

Agenda item:	26.2

Source:	Nokia, Nokia Shanghai Bell, CATT, Huawei, Ericsson, Samsung, ZTE, Qualcomm Incorporated

Title:	(TP for TS 38.413 BL CR) 5GS network timing synchronization status and reporting

Document for:	Discussion and Decision

# 1	Introduction

At RAN#99, a new WID on NR Timing Resiliency and URLLC enhancements was approved in [1]. It includes the following objective:

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

A text proposal for TS 38.413 is provided in Annex A, reflecting the outcome of RAN3#119bis-e discussion as summarized in the SoD [2].

# References

- RP-230754 New WID on NR Timing Resiliency and URLLC enhancements, Nokia, Nokia Shanghai Bell

- R3-231899 Summary of Offline Discussion for CB # URLLC_RANenh

# Annex	A: Text Proposal for TS 38.413

First Modification

#### 9.3.1.220	Time Synchronisation Assistance Information

This IE indicates 5G access stratum time distribution parameters as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Time Distribution Indication | M |  | ENUMERATED (enabled, disabled, …) |  | - |  |
| Uu Time Synchronisation Error Budget | C-ifEnabled |  | INTEGER (1..1000000, …) | Expressed in units of 1ns. | - |  |
| Clock Quality Reporting Control Information | O |  | 9.3.1.x1 |  | Yes | ignore |

| Condition | Explanation |
| --- | --- |
| C-ifEnabled | This IE shall be present if the Time Distribution Indication IE is set to “enabled”. |

Next Modification

#### 9.3.1.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >>[FFS] |  |  |  |  |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

#### 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

#### 9.3.1.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [9].

Editor’s Note: The non-UE associated NGAP procedure(s) used to convey this IE towards the AMF is FFS.

Editor’s Note: This IE may be further refined based on SA2 and RAN3 progress.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | 9.3.1.x4 |  |
| Clock Accuracy | O |  | 9.3.1.x5 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x4	Clock Frequency Stability

This IE indicates the clock frequency stability as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

#### 9.3.1.x5	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >choice1 |  |  |  |  |
| >>[FFS] |  |  |  |  |

#### Editor’s Note: Encoding of the Clock Accuracy IE is to be decided by RAN3 and should allow for different RAN implementations (e.g., CHOICE structure). Details FFS.

End Modification