3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231982

Electronic meeting, 17 Apr – 26 Apr, 2023

Title: 	(TP to TS 38.470 BL CR) Multicast Reception for RRC_INACTIVE state UEs

Source: 	Huawei, Lenovo, Nokia, Nokia Shanghai Bell

Agenda item:	15.3

Document Type:	other

# 1. Introduction

Follow the discussion of CB: # MBS2_RRCInactive, this paper provides the TP to 38.470, on the new SIB.

# Annex E. TP to TS 38.470

-----------------Start of the Changes-------------------

### 5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The gNB-DU is responsible for the encoding of the NR-MIB message. In case broadcast of SIB1 and other SIBs is needed, the gNB-DU is responsible for the encoding of the SIB1 message, SIB10, SIB12, SIB13, SIB14, SIB15, SIB17, SIB18, SIB20 and SIBX, and the gNB-CU is responsible for the encoding of other SIBs. The gNB-DU may re-encode SIB9. The gNB-DU is responsible for the generation of the SystemInformation message.

NOTE:	The SIB19 is generated by the gNB-DU.

The gNB-CU is responsible for receiving the positioning assistance information from LMF, e.g. the positioning related SIBs. The gNB-CU notifies gNB-DU about the SIBs, and the gNB-DU signals them directly.

To support Msg3 based on-demand SI as described in TS 38.331 [11], the gNB-CU can confirm the received SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested other SIs.

-----------------End of the Change-------------------