3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231994

was R3-231396

Electronic meeting, 17 Apr – 26 Apr, 2023

Title: 	(TP to TS 38.401BL CR) MT-SDT in disaggregated architecture

Source: 	Huawei, ZTE, Nokia, Nokia Shanghai Bell, Google, LG Electronics, CATT, Lenovo, Ericsson

Agenda item:	20.2

Document Type:	Other

# Introduction

In this document provide the TP for TS 38.401 BL CR, takes into account the inputs from R3-231396 and R3-231785.

# TP to TS 38.401 BL CR

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1a-0.	The gNB-CU-UP receives only DL SDT data for the UE in RRC Inactive on NG-U interface.

1a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If supported, the gNB-CU-UP may include the MT-SDT information in the DL DATA NOTIFICATION message.

NOTE: The gNB-CU-CP may request the gNB-CU-UP to provide such MT-SDT information before step 1a-0, via BEARER CONTEXT SETUP REQUEST message or BEARER CONTEXT MODIFICATION REQUEST message.

1b. The gNB-CU-CP receives DL signalling over NGAP.

2.	After 1a or 1b, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT information may be included in the PAGING message.

3.	The gNB-DU sends the Paging message, and may include the MT-SDT  indicator to the UE.

4.If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

- the UE indicates the RRC Resume Request is for MT-SDT, which may be without UL data.