3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231999

E-meeting, 17 – 26 April, 2023

Agenda item:	10.2.3

Source:	Nokia, Nokia Shanghai Bell, Huawei, ZTE, Intel

Title:	(TP for SON BL CR to TS 38.473) TP for RACH optimisation

Document for:	Text Proposal

# 1	Introduction

This is TP for RACH optimisation following discussions at RAN3#119bis-e.

# 2	References

[1]		R3-231126, (BLCR to 38.473) Addition of SON features enhancement

# TP for BL CR to TS 38.473

This TP is based on [1]

<<< start of changes >>>

## 8.2	Interface Management procedures

### 8.2.x		RACH Indication

#### 8.2.x.1		General

This procedure is initiated by the gNB-DU to inform the gNB-CU about the occurrences of successful random access procedures in the gNB-DU.

The procedure uses non-UE associated signalling.

#### 8.2.x.2		Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure.

The gNB-DU initiates the procedure by sending the RACH INDICATION message to the gNB-CU.

The RACH INDICATION message contains information concerning one or more successful random access procedures occurring at the gNB-DU and not known to the gNB-CU as described in TS 38.401 [4].

Upon reception of the RACH INDICATION message, the gNB-CU may trigger retrieval of RA Reports from the UE.

#### 8.2.x.3 	Abnormal Conditions

Not applicable.

<<< next change >>>

#### 9.2.1.x	RACH INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about one or more random access procedures performed at the gNB-DU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RA Report List |  | 1 |  |  | YES | reject |
| >RA Report List Item |  | 1..<maxnoofRAReports> |  |  | - | - |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum number of RA Report indications in the message. Value is FFS. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

[snip]

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

UlTxDirectCurrentMoreCarrierInformation,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

RAReportList

FROM F1AP-IEs

[snip]

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-UlTxDirectCurrentMoreCarrierInformation,

id-SRSPosRRCInactiveQueryIndication,

id-RAReportList,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

[snip]

<<< skip unchanged >>>

-- **************************************************************

--

#### -- RACH Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- RACH Indication

--

-- **************************************************************

RachIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{ RachIndication-IEs}},

...

}

RachIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-RAReportList				CRITICALITY reject	TYPE RAReportList						PRESENCE mandatory	},

...

}

<<< next change >>>

### 9.4.5	Information Element Definitions

<<< skip unchanged >>>

MIMOPRBusageInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RAReportList ::= SEQUENCE (SIZE(1..maxnoofRAReports)) OF RAReportList-Item

RAReportList-Item ::= {

gNB-CU-UE-F1AP-ID					GNB-CU-UE-F1AP-ID,

iE-Extensions						ProtocolExtensionContainer { { RAReportList-ExtIEs} } OPTIONAL,

...

}

RAReportList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RANAC ::= INTEGER (0..255)

<<< next change >>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

[snip]

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

maxnoofRAReports						INTEGER ::= 256 –- value is FFS

[snip]

id-measurementPreconfiguration				ProcedureCode ::= 76

id-measurementActivation					ProcedureCode ::= 77

id-QoEInformationTransfer					ProcedureCode ::= 78

id-PDCMeasurementTerminationCommand			ProcedureCode ::= 79

id-PDCMeasurementFailureIndication 			ProcedureCode ::= 80

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

id-RachIndication							ProcedureCode ::= 99 -- assigned by MCC

[snip]

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-UlTxDirectCurrentMoreCarrierInformation			ProtocolIE-ID ::= 689

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 690

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= 901 -- assigned by MCC

id-RAReportList										ProtocolIE-ID ::= 900 -- assigned by MCC

END

<<< end of changes >>>