3GPP TSG-RAN WG3 Meeting #119bis-e	R3-232000

Online, 17 – 26 Apr, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0287 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Multicast session establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Lenovo, ZTE, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the RAN3#119 meeting, the GTP-UL TEID info in the broadcast session establishment procedure had been deleted to align with stage 3 specification. However, for NG-U tunnel establishment procedure of multicast session, the current version of NGAP also does not specify the GTP UL TEID for NG-U unicast transport in Distribution Setup procedure. Therefore, the GTP UL TEID for NG-U unicast transport in steps 4/5 of Multicast session establishment procedure should be removed . - To provide the 5GC NG-U TNL address for multicast transport from gNB-CU-CP to gNB-CU-UP, the MC BEARER CONTEXT MODIFICATION REQUEST message should be used. In the current TS 38.401, the steps 5a/5b added in the RAN3#119 meeting shall include the NG-U TNL, which should be corrected to address for multicast transport instead of GTP UL TEID. And also steps 11/12 in the callflow is updated to only include the F1-U TNL DU info and remove the NG-U TNL info, while the description of steps 11/12 still include the NG-U TNL redundantly. Thus, the description of steps 11/12 should be aligned. - Wording issues, e.g. change "DU" to "gNB-DU". - Redundancy between step 10 and step 11/12. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the GTP UL TEID for NG-U unicast transport in steps 4/5 of Multicast session establishment procedure - Update the wording of "5GC UL TEID" to "5GC NG-U TNL address for multicast transport" in steps 5a/5b, and remove the wording of "and the 5GC NG-U TNL address " in steps 11/12 - Change "DU" to "gNB-DU". - Remove the redundancy between step 10 and step 11/12. - Typos fixed. Impact Analysis: This CR has an isolated impact towards the previous version of the specification because it only modifies stage 2 description of the MBS multicast context establiment function. This is CR has an impact under functional point of view. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between the call flow of MBS multicast context establiment and the NGAP specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-1: updated based on comments received during the meeting. |  |  |  |  |  |  |  |  |

#### 8.15.1.2	Multicast MBS Session Context Establishment

Figure 8.15.1.2-1 illustrates an exemplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for Multicast MBS Session Context Establishment with a UE joining an active multicast session as the first UE joining in its serving gNB.

Figure 8.15.1.2-1: Multicast MBS Session Context establishment

0.	For a certain multicast MBS session (TMGI), which is currently active, no multicast MBS session context is yet established in the gNB.

1.	The UE joins the multicast session.

NOTE 1:	NAS related details for PDU Session signalling are not shown in Figure 8.15.1.2-1.

2/3. The gNB-CU-CP establishes the multicast bearer context at the gNB-CU-UP. For unicast NG-U transport the GTP DL TEID is retrieved.

4/5. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast NG-U transport, DL GTP TEID is provided to the 5GC. For multicast NG-U transport, multicast address information is retrieved from the 5GC. Multicast session QoS parameters are provided by the 5GC.

5a/5b. The gNB-CU-CP triggers the setup of MRB resources via the E1AP MC Bearer Context Modification procedure. For multicast NG-U transport the 5GC shared NG-U multicast address information is provided to the gNB-CU-UP. Multicast session QoS parameters are provided to the gNB-CU-UP.

6/7.	The gNB-CU-CP establishes the Multicast Context at the gNB-DU, providing MRB configuration. It may contain MBS Area Session ID information.

8/9. The gNB-CU-CP retrieves the MRB configuration for the joined UE from the gNB-DU via F1 UE Context Management procedures.

NOTE 2:	The interactions between the gNB-CU-CP and gNB-CU-UP for UE specific Berarer Context management before and after step 8/9 are not illustrated in this Figure.

10.	The gNB-DU triggers the establishment of F1-U resources, which are established either per gNB-DU or per cell or per MBS Area Session ID.

11/12. The gNB-CU-UP side of the F1-U is established by means of the E1AP MC Bearer Context Modification procedure, providing the gNB-DU side F1-U TNL to the gNB-CU-UP, which provides the gNB-CU-UP side F1-U TNL address in return.

13.	The gNB-CU-UP side F1-U TNL address is provided to the gNB-DU.

14.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

15.	The gNB successfully terminates the NGAP procedure for establishing the multicast session context.

16.	The gNB-CU-CP RRC-configures each UE which has joined the multicast group.

17.	The multicast MBS media stream is provided to the UEs.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU- CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.