3GPP TSG-RAN WG3 #119bis_e                                      R3-232072

17th–26th April 2023

Online

Agenda item:	11.2

Source: 	ZTE, CMCC, Huawei, China Telecom, China Unicom

Title: 	TP to BL CRs for TS 38.300 for INACTIVE/IDLE QoE, new service types, and high mobility scenario

Document for:	Discussion and Approval

- 1 Introduction

This contribution provides TP for BLCR of TS 38.300.

- 2 TP for TS 38.300

<<<<<< START OF CHANGES >>>>>>

- 21	Application Layer Measurement Collection

- 21.1	Overview

The QoE Measurement Collection function enables collection of application layer measurements from the UE. The supported service types are:

-	QoE Measurement Collection for DASH streaming services;

-	QoE Measurement Collection for MTSI services;

-	QoE Measurement Collection for VR services;

For DASH, MTSI, VR, the QoE measurement collection is supported in RRC_CONNECTED state only.

QoE measurements for the application sessions for broadcast are supported in RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE states. The QoE measurements for the application sessions for MBS multicast are supported in RRC_CONNECTED state only. Both signalling based and management based QoE measurement collection are supported.

NOTE:	The naming QoE Measurement is used in NG, Xn, and interfaces between the OAM and the gNB. In the Uu interface, the naming application layer measurement is used and it is equal to QoE Measurement.

--- SKIP UNCHANGED PART ---

- 21.2.4	QoE Measurement Handling in RRC_IDLE and RRC_INACTIVE States

If the UE is in RRC_INACTIVE state, the QoE measurement configuration is stored in the UE Inactive AS context.

If the UE enters RRC_IDLE state, the UE releases all application layer measurement configurations except for the application layer measurement configurations pertaining to the application sessions for MBS broadcast.

--- SKIP UNCHANGED PART ---

- 21.3	QoE Measurement Continuity for Mobility

The QoE measurement collection continuity for intra-system intra-RAT mobility is supported, with the Area Scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the Area Scope. A UE continues an ongoing QoE measurement even if it leaves the Area Scope, unless the network indicates to the UE to release the QoE configuration.

For the RRC_CONNECTED state mobility, the source gNB may transmit the QoE measurement configuration(s) and/or the information related to the configuration(s) of a specific UE to the target gNB via XnAP or NGAP. For signalling-based QoE, the service type, QoE reference, MCE IP address, measurement configuration application layer id, MDT alignment information, area scope, slice support list for QMC and measurement status are passed to the target gNB. For management-based QoE, the service type, measurement configuration application layer id, MCE IP address and QoE measurement status are passed to the target gNB. For RRC_INACTIVE state mobility, QoE measurement configuration(s) of a specific UE can be retrieved from the gNB hosting the UE context when it resumes to the RRC_CONNECTED state. Multiple sets of QoE measurement configurations should be supported during mobility.

For signalling based QoE, at handover to a target gNB that supports QoE, the target gNB decides which of the application layer measurement configurations should be kept or released, e.g. based on application layer measurement configuration information received from the source gNB in Xn/NG signalling.

For QoE measurements pertaining to application sessions for MBS broadcast, measurement collection may continue during RRC_INACTIVE and RRC_IDLE states, and the measurement results, if collected, are sent via QoE reports when the UE returns to the RRC_CONNECTED state.

Upon UE’s transition from RRC_IDLE to RRC_CONNECTED state, the gNB serving the UE should ensure that it does not release an already configured signaling based QoE measurement configuration for the sake of configuring a management-based QoE measurement configuration.

When the UE resumes the connection with a gNB that does not support QoE, the UE releases all application layer measurement configurations.

--- SKIP UNCHANGED PART ---

- 21.x QoE Measurement Collection in High Mobility Scenarios

QoE measurements can be confined to high mobility state of the UE and/or to HSDN cells.

<<<<<< END OF CHANGES >>>>>>