3GPP TSG-RAN WG3 #119bis-e	R3-231900

Online, April 17 - 26, 2023

Agenda Item:	13.1

Source:	Xiaomi (Moderator)

Title:	Summary of CB: # IAB1_General

Document for:	Discussion

Introduction

This paper captures the following CB discussion:

CB: # IAB1_General - Continue discussions and converge on NGAP Initial UE message to include an optional “mobile IAB-node indication” - Discuss and converge on TAC update at the mIAB-DU - Discuss and converge on SA2’s request for positioning of onboard UEs via NRPPs using TRPs of the mobile IAB-node - Discuss and converge, if possible, on handling of the ULI for the UEs served by mIAB node (content, signaling and nodes involved) - Discuss Mobile TRPs and what information is needed to be exchanged to support them (moderator - Xiaomi) Summary of offline disc R3-231900

The CB has the following phases:

Phase I：Converge on open issues. Deadline is Wednesday, April 19, 2023, 18:00 UTC.

Phase II：Work on the CRs/LS If needed.

The following contributions are included in this CB:

| R3-231307 | Workplan for Rel-18 mobile IAB (Qualcomm Inc. (Rapporteur)) | Work Plan |
| --- | --- | --- |
| R3-231308 | Discusssion on issues related to SA2 VMR (Qualcomm Inc.) | discussion |
| R3-231356 | Discussion on SA2 issues and mobile IAB authorization (ZTE) | discussion |
| R3-231482 | Discussion on UE positioning and additional ULI for VMR (Huawei) | discussion |
| R3-231522 | Discussion on support of MBSR (Xiaomi) | discussion |
| R3-231523 | Support of MBSR (Xiaomi, Ericsson, Qualcomm, CATT) | CR1153r, TS 38.473 v17.4.1, Rel-18, Cat. B |
| R3-231532 | Discussion of SA2 FS_VMR Solutions (Ericsson) | discussion |
| R3-231533 | (draftCR TS 38.305) Introduction of MBSR (Ericsson, Xiaomi, Qualcomm Inc., CATT) | draftCR |
| R3-231534 | (CR TS 38.455) Support for MBSR Location Information (Ericsson, Xiaomi, Qualcomm Inc., CATT) | CR0101r, TS 38.455 v17.4.0, Rel-18, Cat. B |

For the Chairman’s Notes

Workplan

Proposal 0: R3-231307 workplan is “noted”.

Mobile IAB indication over NGAP

Proposal 1: RAN3 agrees that the IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5.

Proposal 2: RAN3 to decide whether to support NGAP indication to AMF for mobile IAB-authorization based on progress in SA2.

Mobile IAB-node involved positioning

Proposal 3, WA: RAN3 assumes that LMF obtains an updated location and velocity information and location time stamp of the mobile IAB-node by performing either option 1 or option 2 during UE positioning, SA2’s progress on option 1 and option 2 should also be considered.

Proposal 4, WA: if Network Assisted procedure is used (i.e. UL related positioning is performed), LMF may obtain an updated location and velocity information of the mobile IAB-node in UL measurement result related message(s) in NRPPa and F1AP. FFS on which message(s) is used and whether time stamp for the location of mobile IAB-node is needed

Proposal 5, RAN3 agrees to introduce a new TRP type for mobile IAB-node in NRPPa/F1AP.

Proposal 6, if option 1 is supported, the NRPPa/F1AP TRP information exchange procedure can be used to trigger MO-LR procedure of IAB-MT to obtain the location and velocity information of mobile IAB-node as well as the timestamp.

Proposal 7: if GPSI is available, mobile IAB-MT’s UE ID (i.e. GPSI) can be included in NRPPa/F1AP TRP information response message so that LMF can perform MT-LR procedure to obtain mobile IAB-node’s location.

Proposal 8, Checking the security issue of GPSI with SA3 is out of RAN3 scope.

Proposal 9, the issue for the case that “LMF that performs the location estimation of the mobile IAB-node can be different than the LMF that performs the location estimation of the target UE” is not in RAN3 scope.

Additional ULI

Proposal 10: RAN3 agrees that the IAB-DU’s donor-CU includes at least the NCGI of the cell serving IAB-MT as an additional ULI together with UE ULI over NGAP.

Proposal 11, RAN3 to select between the following options for IAB-DU’s donor CU to obtain the IAB-MT’s serving cell ID and TAI (FFS) in case IAB-MT and IAB-DU are connected to different IAB-donors

-           Option A, the cell ID and TAI (FFS) of IAB-MT’s serving cell is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU.

-           Option B, the cell ID and TAI (FFS) of IAB-MT’s serving cell is passed from IAB-DU to IAB-DU’s donor CU.

To be continued:

RAN3 to discuss whether to include the TAI and the time stamp of the location information of IAB-MT’s serving cell into the new IAB-MT User Location Information IE.

Discussion - Phase I

The workplan in R3-231307 was floated on the reflector several before submission and no comments were received. The Moderator proposes that the workplan is marked as “noted”.

Proposal 0: Workplan in R3-231307 to be marked as “noted”.

Q0: Do you agree with this proposal? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| CATT | Yes |  |
| Nokia | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| Xiaomi | Yes |  |

Summary: No comments received, all companies agree to note the workplan.

Proposal: R3-231307 workplan is “noted”.

In RAN3 119 meeting, the following agreements and TBC were captured.

RAN3#119:

Reply LS on FS_VMR solutions review to SA2 agreed in R3-231011

With respect to mobile IAB, for issues concerning the control of UE access to MBSR using CAG function no enhancement is needed and no replies are foreseen from RAN3 on this matter.

NGAP Initial Context Setup Request, UE Context Modification Request and HO Request to include an IE with code points “mobile-IAB authorized”, “mobile-IAB not-authorized”.

To be continued: NGAP Initial UE message to include an optional “mobile IAB-node indication”.

In the Reply LS to SA2 R3-231011, RAN3 agreed to further discuss point#2 (TAC issue), point#6 (positioning issue) and point#7 (additional ULI), according to the content in R3-231011.

RAN3 would like to thank SA2 for their Reply LS on FS_VMR solutions review (R3-230032/ S2-2211437). Regarding SA2’s requests on points #1, #2, #6, and #7, RAN3 provides the following replies: - For point#1 (regarding KI#1): RAN3 is still discussing procedures and configurations for mobile IAB and has so far not identified any new OAM configuration parameters. RAN3 will provide information on additional parameters to be configured on the mobile IAB by OAM, if any, when these discussions have been finalized. - For point#2 (regarding KI#3): Regarding TAC, RAN3 has agreed: Capture on stage 2 that the TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location. It needs to be further discussed how the mobile IAB-DU’s TAC/RANAC is changed and what Stage 3 impacts are (if any). RAN3 will inform SA2 on further progress regarding TAC for mobile IAB, if any. - For point#6 (regarding KI#5): RAN3 needs to conduct further discussions to converge on a solution. RAN3 will inform SA2 for any progress. - For point#7 (regarding KI#6): RAN3 believes that the functionality requested to provide UE location in point#7 can be accommodated within Rel-18.

In this AI, companies provided discussion papers, CRs and draft LS on the following issues

- “mobile IAB-node indication” in NGAP initial UE message;

- TAC update at the mIAB-DU, for point#2 (regarding KI#3);

- MBSR (i.e. mobile TRP or mobile IAB-node) involved positioning, for point#6 (regarding KI#5);

- Additional ULI, for point#7 (regarding KI#6);

For point#2 (TAC issue), the details had been discussed in previous RAN3 meetings and there’re some agreements in AI13.3, and most companies provided their views in AI13.3, so it is suggested to discuss TAC issue in AI13.3. If there’re some progresses needed to be captured in the LS (if any) to SA2, we can further discuss the details in 2nd round according to the discussions.

In Phase 1, the plan is to converge on solutions for mobile IAB-node indication, MBSR involved positioning, additional ULI.

In phase 2, the plan is to prepare the CRs/TP/LS (if needed) according to the 1st round discussion.

Mobile IAB-node indication in NGAP

This issue is discussed in paper R3-231308 (Qualcomm), R3-231356 (ZTE) and R3-231532(E///) in AI 13.1, and also discussed in paper R3-231442(Lenovo) in AI 13.2.

As stated in many papers, SA2 has already captured the following normative text in TS 23.501, vs. 18.0, section 5.35A.1:

For a MBSR node, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization.

Based on the above text, R3-231308 (Qualcomm), R3-231356 (ZTE), R3-231532(E///) and R3-231442(Lenovo) proposed to include mobile IAB-node indication” in NGAP Initial UE message.

In addition, R3-231356 (ZTE) indicates that RAN2 has no consensus on whether to include the mobile IAB-node indication in Msg5, which seems not aligned with SA2, it is proposed RAN3 to send LS to RAN2 about RAN3 discussion progress. But according to RAN2's latest agreement in RAN2’s Monday online session, RAN2 agreed to introduce mobile IAB-node indication in Msg5, so there’s no need to send the LS.

#### Q1: Proposals for mobile IAB-node indication:

Proposal 1-1: RAN3 agrees to include “mobile IAB-node indication” in NGAP Initial UE message so that the AMF can perform mobile IAB authorization.

Proposal 1-2: RAN3 agrees that the IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5.

Do you agree with these proposals? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | Please add “an optional” in P1-1. |
| Qualcomm | P1-1: Yes P1-2: Yes | We need to align with SA2 spec. Also, on Monday, RAN2 agreed to support mobile IAB-node indicator in Msg.5 |
| Huawei | P1-1: see comment P1-2: Ok | For P1-1, as discussed in last meeting, we do not see the real benefit on introducing such indication. Some contributions propose this just because SA2 spec has already captured “When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413”, but this indication in NGAP is just assumed by SA2, which still to be confirmed by R3. It does not make sense to use SA2’s assumption as a reason for R3 discussion here. |
| CATT | Yes | Mobile IAB-node indicator should be included in Msg.5. |
| Nokia | P1-1: no P1-2: yes | There is no clear benefit for P1-1. Similar discussion in NCR and concluded there is no such need. |
| ZTE | P1-1: Yes P1-2: Yes |  |
| Samsung | P1-1: Yes P1-2: Yes |  |
| Lenovo | P1-1: yes P1-2: yes | To keep similar with R16 fixed IAB and to introduce these two indicators. |
| Xiaomi | P1-1: need more discussion P1-2: yes | After further checking, we think the necessity of this mobile IAB indication may need more discussion. |

Summary: all companies agree P1-2, although RAN2 already agreed it, it’s nice for RAN3 to have the agreement for stage2 text, 6 companies agree P1-1 since it’s already captured in TS 23.501, but 2 companies think that this indication is not needed, since the AMF anyway will connect UDM to obtain the subscription information (including mobile IAB authorized) without this indication. So, the moderator proposes the following way forward.

Proposal 1: RAN3 agrees that the IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5.

2nd round discussion:

Proposal 2, RAN3 to discuss whether the AMF can perform mobile IAB authorization without mobile IAB indication from gNB:

- If yes, RAN3 sends LS to SA2 to notify RAN3’s understating.

- If no, RAN3 agrees to include “mobile IAB-node indication” in NGAP Initial UE message.

- If not sure, RAN3 send LS to check with SA2.

(if LS to SA2 is needed, suggest QC to prepare it)

MBSR involved positioning for point#6 (regarding KI#5)

The support of MBSR involved positioning is the requested by SA2’s LS R3-230032/ S2-2211437, and RAN3 agreed to further discuss the solutions and impacts according to the Reply LS in R3-231011.

- For point#6 (regarding KI#5): RAN3 needs to conduct further discussions to converge on a solution. RAN3 will inform SA2 for any progress.

In this AI, R3-231308 (Qualcomm), R3-231482 (HW), R3-231356 (ZTE), R3-231522(Xiaomi) and R3-231532(E///) provided discussion papers on their understandings on SA2’s solution and potential RAN3 impacts. And there’re some CR/TP/LS provided as well.

### 3.2.1 Confirm the understandings on SA2’s stage2

In the discussion papers, all the companies discussed this issue based on the stage2 procedure captured in the SA2’s agreed CR S2-2301478 “MT-LR procedure for when a MBSR is involved in the location of a UE” quoted as follows (the RAN3 related impacts are highlighted in blue):

And below are some relevant texts for RAN3 discussion:

4.	LMF derives if any MBSR(s) is involved in the positioning of the target UE based on the cell-ID used for positioning measurements in the step 3. The AMF serving the target UE may indicate that the serving cell is an MBSR (if applicable). When the MBSR was integrated as a TRP (IAB-DU) with a gNB, the LMF may determine from in a TRP information exchange procedure that the cell-ID belongs to the MBSR and/or the UE-ID (GPSI) associated with MBSR. As the MBSR can be mobile the LMF may need to determine an updated location of the MBSR by either performing step 5-7 (option 1) or performing step 8-10 (option 2) if option 1 is not feasible. If several MBSRs were derived, then step 5-7 or step 8-10 are be performed for each MBSR.

<Omitted part>

11.	[Conditional] The LMF performs one of the positioning procedures with the target UE described in clauses 6.11.1 and 6.11.2. To reduce the timing offset of the positioning measurements, the UE positioning procedure may be scheduled with the same scheduled location time as the MBSR positioning in step 6 or 9. If Network Assisted procedure is used, as in 6.11.2, the NG-RAN may provide the MBSR updated location and velocity information to the LMF as part of the NRPPa procedure.

To make the discussion simple, it is suggested that RAN3 have common understandings on the SA2’s stage2 flow first, and then discuss the spec impacts and detail signalling design.

According to the discussion papers, the following understandings need to be confirmed with the group.

Understanding 1: LMF needs to be aware of the TRP is MBSR (i.e. mobile TRP) before UE positioning [according to the description in step 4, LMF needs to know whether the TRP is mobile before UE positioning, and then perform either option 1 or option 2 during UE positioning]

Understanding 2, LMF obtains an updated location and velocity information of the MBSR by performing either option 1 or option 2 during positioning, which means both options needs be supported. [according to the description in step 4, if the TRP involved in the positioning is MBSR, LMF performs either option 1 or option 2. However, R3-231482 (HW) argues that there’re some issues for option 1 in case of inter-donor DU migration and partial migration, it is moderator’s understanding that the issue can be solved by OAM or other means, which needs to be further discussed in 3.2.4, and SA2 is aware of this issue, so it is clearly stated in SA2’s CR that if option 1 is not feasible option 2 is used, which means the option 1 needs to be supported even through it’s not feasible sometimes.]

Understanding 3, If Network Assisted procedure is used (i.e. UL related positioning is performed), LMF obtains an updated location and velocity information of the MBSR as part of NRPPa procedure. [according to the description in step 11, if RAN assisted procedure is used, i.e. LMF decides to use UL related positioning methods for UE positioning, which means the TRPs involved in this UE positioning will measure the UL SRS and report the measurement results to LMF, LMF will calculate the UE location based on the measurement results, so providing the updated location and velocity information of the involved MBSRs along with the measurement results can help LMF to calculate the UE location accurately]

#### Q2: Understandings on SA2 stage2 procedure for MBSR involved positioning:

- Understanding 2-1: LMF needs to be aware of the TRP is MBSR (i.e. mobile TRP) before UE positioning.

- Understanding 2-2, LMF obtains an updated location and velocity information of the MBSR by performing either option 1 or option 2 during positioning (option 2 can be performed if option 1 is not feasible), which means both options needs be supported by specifications.

- Understanding 2-3, If Network Assisted procedure is used (i.e. UL related positioning is performed), LMF obtains an updated location and velocity information of the MBSR as part of NRPPa procedure.

Do you agree with these understandings? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | P2-1: Yes P2-2: Yes P2-3: Yes | On P2-2: R3-231482 is right in that the mobile TRP may move away during the UE positioning session. In this case, the LMF does not obtain UE positioning data for this TRP anymore. This situation is equivalent to the scenario, where the UE moves out of the coverage range of a stationary TRP during a positioning session. If any of this happens, the LMF can always perform Option 1 to obtain an update oPn mobile TPRs. |
| Huawei | Yes for 2-1 Others please see comment | For 2-2: Option 1 has some issues during IAB mobility (for DU migration and partial migration), which still need further discussion (please refer to our answer to Q4 and the Q6 in 3.2.5), not sure both options can be supported at current stage. So, we suggest option 1 should pending the progress on these issues. For 2-3: Does this mean that the step 11 can be used standalone, and no need to introduce option 1/2? This is not indicated in SA2’s LS. In addition, it is unclear in the procedures When and how the MBSR obtain the location and velocity to be reported in the measurement response? More clarification from SA2 is expected on how to work with such understanding 2-3. |
| CATT | Yes |  |
| Nokia | 2-1: yes 2-2: yes 2-3: yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| Xiaomi | Yes to all |  |

Summary: all companies aligned on the first understanding, which can be further captured in 3.2.2, no proposal for this section. For understanding 2 and 3, all the companies except one have the same understandings on the SA2’s stage procedure, one company have concerns on the feasibility of option 1 during DU-migration and partial migration, which may need more discussion and SA2’s further inputs, so the moderator think at least we can have the following WAs for further discussion.

Proposal 3, WA: RAN3 understand that LMF obtains an updated location and velocity information of the MBSR by performing either option 1 or option 2 during UE positioning (option 2 can be performed if option 1 is not feasible), SA2’s progress on option 1 and option 2 should also be considered.

Proposal 4, WA: RAN3 understand that If Network Assisted procedure is used (i.e. UL related positioning is performed), LMF obtains an updated location and velocity information of the MBSR as part of NRPPa procedure, SA2’s progress on this understanding should also be considered.

### 3.2.2 New TRP type

To support the above understanding 2-1, it is proposed to introduce a new TRP type in NRPPa/F1AP in the following papers R3-231308 (Qualcomm), R3-231482 (HW), R3-231356 (ZTE), R3-231522(Xiaomi) and R3-231532(E///). If this can be agreed, RAN3 can further discuss the corresponding stage2 CR as proposed in R3-231533(Ericsson, Xiaomi, Qualcomm Inc., CATT) and stage 3 CRs as proposed in R3-231534(Ericsson, Xiaomi, Qualcomm Inc., CATT) for NRPPa and R3-231523(Xiaomi, Ericsson, Qualcomm, CATT) for F1AP.

#### Q3: Proposals about new TRP type for MBSR:

Proposal 3-1, RAN3 agrees to introduce a new TRP type for MBSR in NRPPa/F1AP.

Proposal 3-2, if P3-1 is agreed, RAN3 agrees to capture the stage2 description as proposed in R3-231533.

Stage 2 CR in R3-231533

MBSR (Mobile base Station Relay): mobile IAB-node as defined in TS 23.273 [35]. A MBSR can be a RP, TP or TRP

Proposal 3-3, if P3-1 is agreed, RAN3 agrees to add a new codepoint (e.g. “mobile TRP” or “MBSR”) in the TRP Type IE in the TRP Information IE in NRPPa/F1AP as proposed in R3-231534 and R3-231523.

- Stage 3 CR for NRPPa in R3-231534 and F1AP R3-231523

| >>TRP type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mbsr) | TS 38.305 [18] | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

[Moderator’s note: companies are invited to provide the views on the naming of this new mobile TRP type, as some companies suggested that whether we need to align the naming in RAN specifications, either mbsr or mobile trp.]

Do you agree with these proposals? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Huawei | Ok for P3-1, See comment | For P3-2: about the terminology, in RAN WID, we use the terminology mobile IAB, rather than MBSR. Not sure we will use MBSR in our RAN specs. For P3-3：Suggest to use “mobile-trp” which looks more general as new code point, as in our TP(R3-231482). |
| CATT | Yes | So, is “mbrs” is preferred for the new TRP type? |
| Nokia | 3-1: yes 3-2: with comments 3-3: yes | Same view as Huawei. Prefer simply say the TRP type is mobile trp? |
| ZTE | Yes | Prefer to use “mobile TRP” as the new TRP type in RAN3. |
| Samsung | Yes |  |
| Lenovo | Yes for P3-1 | For P3-2, prefer not to use the MBSR in RAN specs, and mobile IAB-node is better. For P3-3, prefer mobile trp. |
| Xiaomi | Yes |  |

Summary: all the companies are fine with the proposal, just have some naming issue to be solved in 2nd round.

Proposal 5, RAN3 agrees to introduce a new TRP type for MBSR in NRPPa/F1AP.

2nd round:

- Work on the TP/CR to capture the above agreement, based on Stage 2 CR in R3-231533 (E///), NRPPa CR R3-231534(E///) and F1AP CR R3-231523(Xiaomi)

- Discuss the naming issue.

### 3.2.3 Location and velocity information of MBSR

To support option 1, it is requested to provide location and velocity information including the corresponding timestamp via NRPPa/F1AP TRP exchange procedure in R3-231308 (Qualcomm), R3-231482 (HW), R3-231356 (ZTE), R3-231522(Xiaomi) and R3-231532(E///).

As stated in SA2’s stage2 procedure, the location and velocity information are obtained via MO-LR procedure in step 6, which is triggered by the TRP information request message in step 5, and the information obtained in step 6 is provided to LMF via TRP information response message in step 7, this indicates that the information transferred over NRPPa/F1AP should refer to the location and velocity information in LPP spec as MBSR MBSR is considered as a UE from the LMF perspective. Few companies may have different views on whether the location information and timestamp should refer to the existing IEs in NRPPa/F1AP or the IEs in LPP, and some companies think that the existing NG-RAN Access Point Position is designed for static TRP which does not cover all the types of coordinates for a MBSR’s location info, and the existing timestamp IE is SFN type for measurement timing control, which is not suitable for this case the above understandings are discussed in R3-231308 (Qualcomm), R3-231522(Xiaomi) and R3-231532(E///), the detail signalling reference can refer to R3-231532(E///).

In addition, papers in R3-231308 (Qualcomm), R3-231522(Xiaomi) and R3-231532(E///) think that the location and velocity information should also be introduced in measurement related messages, this is to support the step 11 in SA2’s stage procedure (i.e. understanding 2-3)

#### Q4: Proposals about location and velocity information over NRPPa/F1AP:

Proposal 4-1, RAN3 agrees that the location and velocity information of MBSR as well as the timestamp should be provided via NRPPa/F1AP TRP exchange procedure.

Proposal 4-2, if P4-1 is agreed, RAN3 agree to introduce a new type of TRP information (i.e. mobile TRP location info including location, velocity and timestamp) in NRPPa/F1AP spec to request MBSR’s location and velocity information via MO-LR procedure.

Proposal 4-3, RAN3 agrees that location and velocity information of MBSR as well as the timestamp should be provided in measurement result IE when UL related positioning is performed.

[Moderator’s note: the stage2 and stage3 details can be discussed in 2nd round if the above proposals are agreeable]

Do you agree with these proposals? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | P4-1: Yes P4-1: Yes P4-3: Yes |  |
| Huawei | See comments | There are some issues regarding the feasibility of option1. • Issue 1: In the DU migration case, if the mIAB-DU migrates to a new F1 terminating CU, how the LMF can find the new F1 terminating CU? • Issue 2: In the partial migration case, how to perform UE location, if MT and DU connects to different donor? Which LMF (e.g. the LMF serve the F1 terminating donor, or the LMF serve the non-F1 terminating donor) to be used for the UE location service? So, these proposals pending the confirmation of the feasibility of option 1 after solving the above issues. In addition, if supporting option 1, another question for the content is the location information Geographical Coordinates has already been included in the TRP information, why need to add location information again? |
| CATT | Yes |  |
| Nokia | Yes with comments | For location info, this may be already covered by the Geographical Coordinates IE. For velocity, it is unclear how this can be used by LMF. Does it mean in case a low (or 0) velocity, the LMF can use previous knowledge of the TRP’s location information? But then this may be covered by the “mobile TRP” in case IAB moves, or normal “Trp” in case IAB stops or move at low speed. For HW comments, in case of DU migration, the AMF will receive the new ULI for the UE and AMF provide the serving cell ID to LMF, so LMF can know target CU. Of course, there is a corner case that LMF initiate a position procedure and UE is HO to another gNB/AMF at the same time, but this also happens in normal system. In partial migration, the AMF (UE) always contact the CU who terminating the UE’s RRC, i.e. the F1-terminating CU. So no issue. |
| ZTE | P4-1: Yes P4-1: Yes P4-3: Yes |  |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| Xiaomi | Yes to all |  |

Summary: majority companies agree with the proposal, one company still doubt option1, one company have some stage3 comments, which may be fixed in the next phase. So, the moderator proposes the following proposals with condition to solve the concerns.

Proposal 6, if option 1 is supported, the NRPPa/F1AP TRP exchange procedure can be used to trigger MO-LR procedure of IAB-MT (i.e. MBSR) to obtain the location and velocity information of MBSR as well as the timestamp. FFS on the stage3 details.

Proposal 7, WA: If UL related positioning is performed, the location and velocity information of MBSR as well as the timestamp can be provided in measurement result IE.

### 3.2.4 MBSR UE ID

To support option 2, it is requested to provide MBSR’s UE ID via NRPPa/F1AP TRP exchange procedure before UE positioning, which is discussed in R3-231308 (Qualcomm), R3-231482 (HW), R3-231356 (ZTE), R3-231522(Xiaomi) and R3-231532(E///).

In addition, R3-231532(E///) thinks that the need for the mIAB-MT UE ID is common to both Option 1 (for the LMF to be aware of the IAB-MT UE ID associated to a specific TRP before UE positioning) and Option 2 (to trigger the MT-LR procedure) agreed by SA2.

According to SA2’s conclusion in TR 23.700-05-i00 and SA2’s agreed CR S2-2301478, GPSI is agreed to be introduced as MBSR’s UE ID for this issue, majority of companies in RAN3 proposed to introduce GPSI as MBSR’s UE ID to support SA2’s conclusion, but R3-231482 (HW) thinks that SUPI can also be considered as GPSI may not be available sometimes, R3-231482 (HW) also mentioned security issue of using GPSI/SUPI and would like to check with SA3.

It is moderator’s understanding that RAN3 can follow SA2’s decision as SA2 already had a comprehensive discussion on which ID to be used. In addition, the moderator thinks that NG-RAN node knows SUPI may have security issue, instead of GPSI.

#### Q5: Proposals about MBSR’s UE ID over NRPPa/F1AP:

Proposal 5-1, RAN3 agrees to include MBSR’s UE ID (i.e. GPSI) in NRPPa/F1AP TRP information response message so that LMF can perform LT-LR procedure to obtain MBSR’s location.

[Moderator’s note: companies are invited to provide views on either GPSI or SUPI is used as MBSR’s UE ID]

Proposal 5-2, RAN3 discuss whether to check with SA3 on the security issue.

Do you agree with these proposals? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | P5-1: agree P5-2: see comment | P5-2: in our understanding, the security concern is only related to the SUPI. In any case, given that SA2 agreed on GPSI, we should follow their conclusion. |
| Qualcomm | P5-1: Yes P5-2: No | On 5-1: In S2-2301478, SA2 refers to SUPI or GPSI at one place, and to GSPI only at another place. Indeed, GPSI may not always be available. We may want to ask SA2 if they have considered that GPSI is not always available, and if SUPI should be supported, too. On 5-2: In this procedure, the UE ID (GPSI or SUPI) is passed over a secure link. This is different from passing IMSI over an unsecured link as it was the case in 4G. Therefore, we cannot claim that there is no obvious security issue here. Also, RAN3 performs this effort based on SA2’s request. RAN3 can assume that SA2 has done their homework. If SA2 is concerned about security issues, they themselves can ask SA3. There is no need for RAN3 to babysit SA2 in properly assessing security issues with SA3. If a company in RAN3 is concerned about security issues in SA2’s efforts, they may want to discuss the matter with their SA2 delegate and take it to SA2. |
| Huawei | See comment | As we stated in our paper, the GPSI is not mandatory to be assigned to a UE because it is only “needed” for “addressing a 3GPP subscription in different data networks outside of the 3GPP system”, especially when the UE is an IAB-MT which may not have any requirement of communication with a data network outside 3GPP. Not sure SA2 has confirmed that the IAB-MT must have GPSI? Better to check with SA2 as suggested by QC. If using SUPI, this will cause some privacy issue. So, our suggestion is: Check with SA2, whether they have considered that the GPSI is not mandatory, and how to proceed if GPSI is not assigned to a mobile IAB-MT. |
| CATT | P5-1: Yes P5-2: Should respect to SA2’s conclusion |  |
| Nokia | 5-1: yes 5-2: no | 23.501 states: GPSIs are public identifiers used both inside and outside of the 3GPP system. |
| ZTE | P5-1: Yes P5-2: No | P5-1: we should follow SA2’s conclusion. P5-2: we believe that SA3 can do it by itself if it’s needed. |
| Samsung | P5-1: agree P5-2: see comment | P5-2: share the similar view with Ericsson. |
| Lenovo | P5-1: Yes P5-2: No | For P5-2, it can be discussed in SA3 if needed. |
| Xiaomi | P5-1: Yes P5-2: No |  |

Summary: two companies mentioned that the GPSI may not always be available, one company still have concern on security issue, so, the moderator proposes the following:

Proposal 8: if GPSI is available, MBSR’s UE ID (i.e. GPSI) can be included in NRPPa/F1AP TRP information response message so that LMF can perform LT-LR procedure to obtain MBSR’s location.

Proposal 9, it is not RAN3’s responsibility to check the security issue of GPSI with SA3.

2nd round:

- Proposal 10, discuss whether RAN3 need to check SA2 about the concerns on GPSI availability and security.

### 3.2.5 Issues in case of inter-donor DU migration and partial migration

It was observed that there may be some issues in case of inter-donor DU migration and partial migration, as mentioned in paper R3-231482 (HW), R3-231522(Xiaomi) and R3-231532(E///).

Before we discuss this issue, one thing needs to be clarified is that the selected MBSRs for a UE positioning may be or may not be the served MBSR of the UE, this understanding is based on the description in step 4 in SA2’s CR, i.e. “If several MBSRs were derived, then step 5-7 or step 8-10 are be performed for each MBSR”.

Based on the above understanding, one of the issues discussed in the above papers is that the TRP information (e.g. cell ID and the hosted gNB as well as the TRP ID) of a MBSR may change after inter-donor DU migration, the LMF may select a MBSR for UE positioning with outdated information.

And the following solutions had been discussed in the papers:

- Option A: LMF can initiate new TRP information exchange procedure by receiving the notification from OAM

- Option B: LMF can initiate new TRP information exchange procedure by receiving the notification from UE-AMF as it is aware of the cell ID change.

But it is the moderator’s understanding that both solutions are not that perfect, the OAM-based solution may not update the information in time, and the UE AMF-based solution requires there is at least one UE served by the MBSR so that the UE AMF can trigger the notification.

Another issue discussed in R3-231482 (HW), if IAB-MT and IAB-DU connects to different donors, the LMF that connects to UE-AMF may be different from the LMF that connects to MBSR-AMF, it is the moderator’s understanding that it’s always the LMF that connects to UE AMF (i.e. F1-terminating donor) to manage the UE location service, but the scenario may be possible, it seems an inter-LMF issue, which should be discussed in SA2.

#### Q6: Proposals about issues in case of inter-donor migration and partial migration:

Proposal 6-1, the TRP information of MBSR including cell ID and TRP ID changes after inter-donor DU migration.

Proposal 6-2, RAN3 discuss whether OAM-based trigger or UE-AMF-based trigger for LMF initiating new TRP information exchange procedure is enough or not.

Proposal 6-3, if IAB-MT and IAB-DU connect to different donors, RAN3 observes that the LMF that connects to UE AMF may be different from the LMF that connects to MBSR AMF, the corresponding discussion is not in RAN3’s scope.

Do you agree with these proposals? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | OK to all, but we prefer the UE-AMF-based trigger. |  |
| Qualcomm | P6-1: No. See comment P6-2: No. See comment. P6-3: No. See comment. | On P6-1: It is incorrect to state that the TRP info changes after DU migration. Instead, the TRP disappears. This implies that there is no TRP info available anymore for this TRP ID. On P6-2: When the LMF requests a positioning measurement using an MBSR-type TRP, which has disappeared due to DU migration, it simply won’t get anything reported for this TRP ID. In this case, it can proactively rerun the TRP Info Exchange for MBSR-type TRPs. There is no need for OAM-based or UE-AMF-based triggers. On P6-3: As agreed in last meeting, it is assumed that there is inter-donor IP connectivity. Hence, if donor 1 can connect to LMF, then donor 2 can connect to this LMF, too. |
| Huawei | See comments | Rewording for P6-1: The cell ID and TRP ID included in the TRP information of MBSR including cell ID and TRP ID changes after inter-donor DU migration. P6-2: The OAM based triggering of a new TRP information exchange procedure is not timely, during the IAB mobility. While for the UE-AMF based triggering, this will be triggered when the UE has location service, then the solution will not be time efficient for UE’s location service. Because LMF should initiate TRP information exchange to of NG-RAN nodes first, to get the updated information of the TRP when mIAB-DU connects to new donor CU. After that, it can initiate the UE positioning procedure. Another solution can be the source CU provides information on the target CU to the LMF based on the request from LMF, after the DU perform migration. Anyway, we need to discuss how to handle this issue if support option 1. Suggest rewording the P6-2 as: RAN3 to discuss how to update the mobile TRP information towards LMF, in case of mIAB-DU migration. For P6-3: the proposal is fine, but suggest to inform SA2 about this issue, if not in RAN3 scope. |
| CATT | P6-1: No P6-2: prefer UE-AMF-based trigger P6-3: See comment | On P6-1: The TRP information of MBSR may not be changed after DU migration. On P6-3: We think the mIAB-MT’s CU and mIAB-DU’s CU should connect to the same AMF(s). So, there is no issue. |
| Nokia | P6-1: No. See comment P6-2: No. See comment. P6-3: out of RAN3 scope | On P6-1: agree with QC. On P6-2: First, this is not in RAN3 scope on how LMF is triggered. Second, we agree with QC. LMF know a TRP is mobile TRP, so it should know this TRP may be gone. A LMF can initiate the position procedure using a TRP that is already gone, but it is ok, since LMF will not get any report. so it can rerun TRP information exchange procedure. BTW, LMF need to be OAM triggered when a new TRP is added, or an existing TRP is gone. This is just like normal system when operator deploy a new cell/TRP, or remove an existing cell/TRP. On P6-3: This is not in RAN3 scope. |
| ZTE | P6-1: yes with rewording P6-2: ok P6-3: yes | P6-1: According to TS 38.455, the TRP ID IE is used to identify a TRP uniquely within an NG-RAN node. We are not sure whether TRP ID should change after DU migration. we suggest the following rewording: Proposal 6-1, the TRP information including cell ID reported by the mobile IAB node changes after inter-donor DU migration. P6-3: it’s not in RAN3 scope. And this issue should have been discussed in SA2 based on S2-2301478/TS 23.273. - The LMF that performs the location estimation of the MBSR can be different than the LMF that performs the location estimation of the target UE (not shown in figure 6.1.4-1). |
| Lenovo | OK for P6-2 and P6-3. | P6-1: agree with QC. |
| Xiaomi | Yes to all |  |

Summary: companies have different views, but the moderator thinks that RAN3 need more discussion and also need to consider SA2’s progress, so the following proposals are for further discussion.

Proposal 11, RAN3 to discuss whether the cell ID and TRP ID included in the TRP information of MBSR including cell ID and TRP ID changes after inter-donor DU migration.

Proposal 12, RAN3 to discuss the following options to trigger LMF initiating new TRP information exchange procedure to obtain the latest TRP information after inter-donor DU migration.

- Option 1, OAM can notify LMF

- Option 2, UE AMF can notify LMF

- Option 3, LMF can trigger it by itself by knowing the TRP disappear

- Option 4, the source CU can notify LMF

Proposal 13, the issue for the case that “LMF that performs the location estimation of the MBSR can be different than the LMF that performs the location estimation of the target UE” is not in RAN3 scope.

Additional ULI

### 3.3.1 Providing additional ULI to UE AMF

The support of additional ULI is the requested by SA2’s LS R3-230032/ S2-2211437, and RAN3 agreed to further discuss the solutions and impacts according to the Reply LS in R3-231011.

- For point#7 (regarding KI#6): RAN3 believes that the functionality requested to provide UE location in point#7 can be accommodated within Rel-18.

In this AI, R3-231482 (HW), R3-231356 (ZTE), and R3-231532(E///) provided their views on the potential issues and RAN3 impacts, in addition, R3-231309 (QC) and R3-231524(Xiaomi) also provided the views about this issue in AI 13.2.

According to the discussions in the above papers, most companies think that the IAB-DU’s donor-CU (i.e. the serving gNB of the UE) should provide the IAB-MT’s ULI along with UE ULI over NGAP, R3-231532(E///) thinks that since RAN3 has agreed to pursue a dynamic TAC solution, the TAC of the mIAB-DU will reflect the location of the mIAB node, and thus no ULI enhancements are needed, but this does not against to provide the serving cell ID of IAB-MT.

Regarding the detail information in the additional ULI, based on the contributions, no companies against to provide the serving cell ID of the IAB-MT. But companies may have different views on whether to provide the TAC and time information.

#### Q7: Proposals about providing additional ULI over NGAP

Proposal 7-1, RAN3 agrees that the IAB-DU’s donor-CU includes ULI of the IAB-MT as an additional ULI together with UE ULI over NGAP.

Proposal 7-2, RAN3 agree that at least the serving cell ID of the IAB-MT is included in the additional ULI, FFS on TAC and time information.

Do you agree with these proposals? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | P7-1: OK P7-2: a modification is needed | P7-2: - If, as P7-2 says, mIAB-MT’s ULI is sent together with UE’s ULI, this means that the Age of Location (“time information” as the Moderator calls it) may be already present, so we do not need any special agreement for the AoL. - TAI of the mIAB-MT’s serving cell is also needed. |
| Qualcomm | P7-1: Yes P7-2: Yes + TAC + time | On P7-2: In NGAP, ULI includes TAC and timestamp. We shouldn’t be too skimpy here. |
| Huawei | P7-1: agree P7-2: see comment | P7-2: the additional ULI includes the cell ID and the TAI of the IAB-MT’s serving cell. The Age of location which is a timestamp of the location information will be carried in the UE’s ULI, and if the additional ULI is included in same message, there is no need to include the same timestamp again in the additional ULI. suggest the following version: The Additional ULI includes the cell ID and TAI of the mIAB-MT’s serving cell. |
| CATT | P7-1: Yes P7-2: Yes + TAC + time |  |
| Nokia | P7-1: Yes P7-2: Yes + TAC + time |  |
| ZTE | P7-1: No P7-2: Yes | P7-1: RAN3 has already agreed that the TAC of the mIAB-DU will reflect the location of the mIAB node, so there is no need to include MT’s TAI as additional ULI. So we suggest the following rewording: Proposal 7-1, RAN3 agrees that the IAB-DU’s donor-CU includes NCGI of the cell serving IAB-MT as an additional ULI together with UE ULI over NGAP. |
| Samsung | P7-1: Yes P7-2: Yes |  |
| Lenovo | P7-1: Yes P7-2: Yes + TAC + time |  |
| Xiaomi | Yes, at least NCGI |  |

Summary: all the companies agree to introduce at least the serving cell ID of IAB-MT as an additional ULI of the UE over NGAP. Other IEs may need more discussion. So, the following proposal and way forward are proposed:

Proposal 14: RAN3 agrees that the IAB-DU’s donor-CU at least includes the NCGI of the cell serving IAB-MT as an additional ULI together with UE ULI over NGAP.

2nd round:

Work on the CR for NGAP based on R3-231482 (HW)
Proposal 15, RAN3 to discuss whether TAI of the IAB-MT should be included in the additional ULI of UE based on the discussion in CB 13.2

- If the TAI broadcasted by IAB-DU is reflected by the TAI of IAB-MT, then the TAI of the IAB-MT is not needed for the UE’s additional ULI.

- If the TAI broadcasted by IAB-DU is not reflected by the TAI of IAB-MT, then the TAI of the IAB-MT is needed for the UE’s additional ULI.

Proposal 16, RAN3 to discuss whether the time stamp of IAB-MT’s ULI is the same as the time stamp of UE’s ULI

- If yes, RAN3 agree to include the additional time stamp in the additional ULI.

- If no, RAN3 agree to reuse the existing time stamp of UE’ULI in the additional ULI (i.e. no additional IE)

### 3.3.2 IAB-MT and IAB-DU are connected to different IAB-donors

For the scenarios where IAB-MT and IAB-DU are connected to different IAB-donors, the IAB-DU’s donor-CU needs to know the serving cell ID of IAB-MT, the following options are proposed in the related papers:

- Option A, the serving cell ID is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU (along with the the gNB-ID of the mIAB-MT’s CU, which already agreed in RAN3 previous meeting)

- Option B, the serving cell ID is passed from IAB-DU to IAB-DU’s donor CU via F1AP message.

#### Q8: Proposal about providing additional ULI in case of the IAB-MT’s donor and IAB-DU’s donor are different

Proposal 8, RAN3 discuss the following options for IAB-DU’s donor to obtain the IAB-MT’s serving cell ID in case IAB-MT and IAB-DU are connected to different IAB-donors

-	Option A, the serving cell ID of IAB-MT is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU

-	Option B, the serving cell ID of IAB-MT is passed from IAB-DU to IAB-DU’s donor CU

Do you agree with this proposal? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, but see comment | The additional ULI should include the NRCGI and TAI of the mIAB-MT’s cell. The above proposal refers only to the latter. If RAN3 agrees that the mIAB-DU inherits the TAI of the mIAB-MT’s parent cell, then mIAB-DU’s CU knows the TAI of mIAB-MT’s parent cell automatically. If RAN3 agrees something else, the proposal above should also include the TAI of mIAB-MT’s parent cell. |
| Qualcomm | Option B, but see comment | In both options, the servicing cell ID of the MT is passed to the mIAB-DU’s CU. The only difference is that in option A, it is passed via Xn, while in option B, it is passed via the IAB-node. Option B has the following advantages: - Works when Xn is not available. Info can be piggybacked onto existing RRC and F1AP signaling. |
| Huawei | Agree |  |
| CATT | Yes, Option A | The Option A is slightly preferred to Option B. The serving cell ID of mIAB-MT together with with the the gNB-ID of the mIAB-MT’s CU can be introduced in the Xn Transport Migration Modification Request message. |
| Nokia | Option B |  |
| ZTE | Agree | Option A is preferred. |
| Samsung | Option A |  |
| Lenovo | Agree | And prefer option B |
| Xiaomi | Option A |  |

Summary: companies views are diverse, this issue can be further discussed in the next meeting with more analysis.

Proposal 17, RAN3 discuss the following options for IAB-DU’s donor CU to obtain the IAB-MT’s serving cell ID and TAI (FFS) in case IAB-MT and IAB-DU are connected to different IAB-donors

-	Option A, the serving cell ID and TAI (FFS) of IAB-MT is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU

-	Option B, the serving cell ID and TAI (FFS) of IAB-MT is passed from IAB-DU to IAB-DU’s donor CU

E-mail discussion over reflector

Workplan

Proposal 0: R3-231307 workplan is “noted”.

Mobile IAB indication over NGAP

Proposal 1: RAN3 agrees that the IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5.

2nd round:

Proposal 2, RAN3 to discuss whether the AMF can perform mobile IAB authorization without mobile IAB indication from gNB:

- If yes, RAN3 sends LS to SA2 to notify RAN3’s understating.

- If no, RAN3 agrees to include “mobile IAB-node indication” in NGAP Initial UE message.

- If not sure, RAN3 send LS to check with SA2.

(if LS to SA2 is needed, suggest QC to prepare it)

MBSR involved positioning

Proposal 3, WA: RAN3 understand that LMF obtains an updated location and velocity information of the MBSR by performing either option 1 or option 2 during UE positioning (option 2 can be performed if option 1 is not feasible), SA2’s progress on option 1 and option 2 should also be considered.

- E///:

  - the location time stamp is also obtained

  - the sentence in the parentheses should be removed, as it is not accurate, we do not need to capture it since it is anyway outside our turf

  - so:

    - Proposal 3, WA: RAN3 understand that LMF obtains an updated location and velocity information and location time stamp of the MBSR by performing either option 1 or option 2 during UE positioning (option 2 can be performed if option 1 is not feasible), SA2’s progress on option 1 and option 2 should also be considered.

[Xiaomi] OK

Proposal 4, WA: RAN3 understand that if Network Assisted procedure is used (i.e. UL related positioning is performed), LMF obtains an updated location and velocity information of the MBSR as part of NRPPa procedure, SA2’s progress on this understanding should also be considered.

- E///: rewording:

  - Proposal 4, WA: RAN3 understand that if Network Assisted procedure is used (i.e. UL related positioning is performed), LMF obtains an updated location and velocity information and location time stamp of the MBSR as part of NRPPa and F1AP procedure, SA2’s progress on this understanding should also be considered.

- [Xiaomi] OK

- [QC] Agree with E///.

Proposal 5, RAN3 agrees to introduce a new TRP type for MBSR in NRPPa/F1AP.

2nd round:

- Work on the TP/CR to capture the above agreement (i.e. a new TRP type), based on Stage 2 CR in R3-231533 (E///), NRPPa CR R3-231534(E///) and F1AP CR R3-231523(Xiaomi)

- Discuss the naming issue.

Proposal 6, if option 1 is supported, the NRPPa/F1AP TRP exchange procedure can be used to trigger MO-LR procedure of IAB-MT (i.e. MBSR) to obtain the location and velocity information of MBSR as well as the timestamp. FFS on the stage3 details.

Proposal 7, WA: If UL related positioning is performed, the location and velocity information of MBSR as well as the timestamp can be provided in measurement result IE.

- E///: rewording:

  - Proposal 7, WA: If UL related positioning is performed, the location and velocity information of MBSR as well as the timestamp can be provided in measurement result IE in NRPPa/F1AP.

- [Xiaomi] OK

- [QC] Agree with E///.

Proposal 8: if GPSI is available, MBSR’s UE ID (i.e. GPSI) can be included in NRPPa/F1AP TRP information response message so that LMF can perform LT-LR procedure to obtain MBSR’s location.

Proposal 9, it is not RAN3’s responsibility to check the security issue of GPSI with SA3.

2nd round:

- Proposal 10, discuss whether RAN3 need to check SA2 about the concerns on GPSI availability and security.

Proposal 11, RAN3 to discuss whether the cell ID and TRP ID associated with the included in the TRP information of MBSR are changed including cell ID and TRP ID changes after inter-donor DU migration.

- [Nokia]: Not ok. IAB-DU’s cell ID and TRP ID are unique per its connected CU. In case DU migration, the source IAB-DU and target IAB-DU have separate cell ID and TRP ID related to the connected donor-CU. There is NO change on cell ID or TRP ID.

- E///: Agree with P11. When the cell-ID and TRP changes, the LMF an inquire the donor to obtain all TRP information, including whether the TRP is mobile or not. This is captured in TS 38.455, clause 8.2.8.2:

  - “If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message”

- [Xiaomi] to Nokia: no sure get the point, but the Proposal needs rewording, hope it would be OK after the update, as the donor-CU of MBSR is changed, the cell ID and TRP ID associated with the MBSR is assigned by the new donor-CU, which means they are changed.

- [Nokia-2]: For each TRP, LMF stores information including TRP ID, NCGI ID, etc. The TRP is identified by the TRP ID (unique per gNB) and/or NCGI/gNB ID which can identify the gNB. The MBSR has 2 logical IAB-DUs (and associated TRPs). From LMF perspective, after DU migration, LMF just see one TRP (from source donor) left, and new TRP (from target donor) joined. There is No “Change” cell ID/TRP ID for a TRP. This is similar to a TRP is removed in normal gNB, and another new TRP is added in another gNB. Current spec allows a LMF to retrieve the information of TRPs from gNB. I do not see any issue in current standard. So P11 is not needed.

- [Xiaomi-2] the issue is just as you mentioned here, how LMF see one TRP left, how LMF knows that the new TRP is replace the old TRP that left. And the proposal is quite open, it doesn’t mean RAN3 agree this, but companies have different understandings, it’s worth to further discuss.

- [QC] Agree with Nokia

Proposal 12, RAN3 wait for SA2’s progress on how RAN3 to discuss the following options to trigger LMF initiating new TRP information exchange procedure to obtain the latest TRP information after inter-donor DU migration.

2nd round:

Discuss whether to notify SA2 about RAN3’s understanding.

- Option 1, OAM can notify LMF

- Option 2, UE AMF can notify LMF

- Option 3, LMF can trigger it by itself by knowing the TRP disappear

- Option 4, the source CU can notify LMF

- [Nokia]: not ok. LMF is a CN node. Why should RAN3 need to discuss how to trigger a CN node initiate a procedure? This is in SA2 scope, but not RAN3.

- E///: Perhaps there is no need for this proposal. Option 1 and 2 are already in TS 38.305 section 8.10.3.2.1 and TS 23.273 says that (UE) AMF can send a positioning request to LMF to trigger positioning. Option 3 is left to implementation. We do not need to specify such information in RAN3. Option 4 contradicts the NRPPa principles because it is always the LMF that triggers an NRPPa transaction and should not be considered.

- [Xiaomi]:Nokia is right, this is in SA2’s scope, but the issue may have RAN3 impacts, so we can wait for SA2’s progress on this issue, the proposal is updated.

- [Xiaomi2] reply to E///, it’s already updated according to nokia’s comment, the intention is to reflect the discussion.

- [QC] Agree with Nokia. We don’t need to discuss this. Also, we don’t support option 4

Proposal 13, the issue for the case that “LMF that performs the location estimation of the MBSR can be different than the LMF that performs the location estimation of the target UE” is not in RAN3 scope.

Additional ULI

Proposal 14: RAN3 agrees that the IAB-DU’s donor-CU includes at least the NCGI of the cell serving IAB-MT as an additional ULI together with UE ULI over NGAP.

2nd round:

Work on the CR for NGAP based on R3-231482 (HW)

Proposal 15, RAN3 to discuss whether TAI of the IAB-MT should be included in the additional ULI of UE based on the discussion in CB 13.2.

- If the TAI broadcasted by IAB-DU is reflected by the TAI of IAB-MT, then the TAI of the IAB-MT is not needed for the UE’s additional ULI.

- If the TAI broadcasted by IAB-DU is not reflected by the TAI of IAB-MT, then the TAI of the IAB-MT is needed for the UE’s additional ULI.

[ZTE] why RAN3 needs to discuss this issue? no matter whetherTAI broadcast by IAB-DU is the same as the TAI of the cell serving the MT, the TAI is not needed based on the following RAN3 agreement:

Proposal 16, RAN3 to discuss whether the time stamp of IAB-MT’s ULI is the same as the time stamp of UE’s ULI

- If yes, RAN3 agree to include the additional time stamp in the additional ULI.

- If no, RAN3 agree to reuse the existing time stamp of UE’ULI in the additional ULI (i.e. no additional IE)

Proposal 17, RAN3 discuss the following options for IAB-DU’s donor CU to obtain the IAB-MT’s serving cell ID and TAI (FFS) in case IAB-MT and IAB-DU are connected to different IAB-donors

-           Option A, the serving cell ID and TAI (FFS) of IAB-MT is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU

-           Option B, the serving cell ID and TAI (FFS) of IAB-MT is passed from IAB-DU to IAB-DU’s donor CU

Discussion - Phase II

After 1st round discussion, the following agreements are stable, which can be agreeable.

Proposal 1: RAN3 agrees that the IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5.

Proposal 3, WA: RAN3 understand that LMF obtains an updated location and velocity information and location time stamp of the MBSR by performing either option 1 or option 2 during UE positioning, SA2’s progress on option 1 and option 2 should also be considered.

Clarification from the moderator on HW’s concern on P4: the current wording is exact the same as what described in SA2’s CR (latest agreed version S2-2304678r02) as below, just added some RAN3 specific wording and spec.

“If Network Assisted procedure is used, as in 6.11.2, the NG-RAN may provide the MBSR updated location and velocity information to the LMF as part of the NRPPa procedure”

In addition, the moderator already considered HW’s concern, as all the companies except one agree P4, so it’s “WA” instead of “agreement”. This WA can help companies for further discussion.

Proposal 4, WA: RAN3 understand that if Network Assisted procedure is used (i.e. UL related positioning is performed), LMF obtains an updated location and velocity information and location time stamp of the MBSR as part of NRPPa and F1AP procedure

Proposal 5, RAN3 agrees to introduce a new TRP type for MBSR in NRPPa/F1AP.

Proposal 6, if option 1 is supported, the NRPPa/F1AP TRP information exchange procedure can be used to trigger MO-LR procedure of IAB-MT (i.e. MBSR) to obtain the location and velocity information of MBSR as well as the timestamp.

Proposal 7, WA: If UL related positioning is performed, the location and velocity information of mobile IAB if available can be provided in measurement result IE in NRPPa/F1AP.

Proposal 8: if GPSI is available, MBSR’s UE ID (i.e. GPSI) can be included in NRPPa/F1AP TRP information response message so that LMF can perform MT-LR procedure to obtain MBSR’s location.

Proposal 9, it is not RAN3’s responsibility to check the security issue of GPSI with SA3.

Proposal 13, the issue for the case that “LMF that performs the location estimation of the MBSR can be different than the LMF that performs the location estimation of the target UE” is not in RAN3 scope.

Proposal 14: RAN3 agrees that the IAB-DU’s donor-CU includes at least the NCGI of the cell serving IAB-MT as an additional ULI together with UE ULI over NGAP.

Proposal 17, RAN3 selects the following options for IAB-DU’s donor CU to obtain the IAB-MT’s serving cell ID and TAI (FFS) in case IAB-MT and IAB-DU are connected to different IAB-donors

-	Option A, the cell ID and TAI (FFS) of IAB-MT’s serving cell is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU

-	Option B, the cell ID and TAI (FFS) of IAB-MT’s serving cell is passed from IAB-DU to IAB-DU’s donor CU

If you still have any concerns on the above proposals, provide your views?

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine with the content of the proposals. For P17, based on Chair’s recent policies, we may want to reword: “Proposal 17: RAN3 to select one of the following two options….” |
| Huawei | On P4 and P7: Looks redundant? Both looks not agreeable. It is still unclear in the procedures when and how the MBSR obtain the location and velocity to be reported in the measurement response, even with the text copied by the moderator. Besides, the text quoted by moderator from SA2 CR not include timestamp, which is included in the two proposals. On P6: Based on the request on the TRP information of the mobile IAB’s TRP (including the TRP ID, and request for the location iformation), the mobile IAB will provide updated location information, and it can initiate MO-LR procedure to obtain the location of the IAB-MT by internal signaling between IAB-DU and IAB-MT. The internal signaling should be up to implementation, no Stage 3 impact is foreseen. So, we suggest to change “FFS on the stage3 details.” to be “No stage 3 impact is expected” P17: agree with QC’s suggestion. |
| Ericsson | P6: We propose to delete the part related to stage3. It is unnecessary. |
| ZTE | P17: suggest to replace “the serving cell ID and TAI (FFS) of IAB-MT” with “the cell ID and TAI (FFS) of the cell serving IAB-MT”. It’s not appropriate to use “TAC of IAB-MT”. Proposal 17, RAN3 selects the following options for IAB-DU’s donor CU to obtain the cell ID and TAI (FFS) of the cell serving IAB-MT in case IAB-MT and IAB-DU are connected to different IAB-donors - Option A, the cell ID and TAI (FFS) of the cell serving IAB-MT is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU - Option B, the cell ID and TAI (FFS) of the cell serving IAB-MT is passed from IAB-DU to IAB-DU’s donor CU |
| Nokia | Ok. ZTE’s modification to P17 is ok. |

Tentative Summary:

Mobile IAB indication for authorization

After further checking SA2’s discussion, the moderator found that the mechanism for MBSR authorization is still not stable, and there are two issues to be considered:

| Issues | Analysis | Result |
| --- | --- | --- |
| 1. whether an indication is needed | AMF will not always communicate with UDM for subscription information (the procedure is optional in TS 23.502) -> AMF may not perform mobile IAB authorization without specific indication. UDM will not provide everything to AMF (too much subscription information), the information provided is based on the request ->AMF may not get the authorized information without specific indication in the request | A specific indication is needed |
| 2. if the indication is needed, whether to reuse existing IAB indication or by other means | According to their EN in chapter 5.35A in TS 23.501, the issue is still pending to SA2’s discussion, and there’s no progress for this meeting. Editor's note: It is FFS whether the MBSR indication is a new IE or part of UE 5G MM capabilities. Editor's note: Whether existing IAB-Operation allowed indication and IAB authorized indication can be reused for MBSR will be determined. | Introduction of mobile IAB indication in NGAP is still FFS |

According to the above updated information, the moderator added option 4 for this issue, considering SA2’s conclusion is unclear right now.

### Proposal 2, RAN3 to discuss whether the AMF can perform mobile IAB authorization without mobile IAB indication from gNB:

- Option 1, If yes, RAN3 sends LS to SA2 to notify RAN3’s understating.

- Option 2, If no, RAN3 agrees to include “mobile IAB-node indication” in NGAP Initial UE message.

- Option 3, If not sure (i.e. no consensus), RAN3 send LS to check with SA2.

- Option 4 wait for SA2’s conclusion.

(if LS to SA2 is needed, suggest QC to prepare it)

Which option is preferred? Comments?

| Company | Option | Comments |
| --- | --- | --- |
| Xiaomi | Prefer option 4, option 3 is also ok | Even SA2 themselves haven’t figured out how the MBSR authorization work, we may need to wait for their conclusion. But if companies want to further check with SA2, sending LS is also fine, but we may need to wait for their conclusion as well. |
| Qualcomm | Option 4 | SA2 should make more progress. RAN3 can always add an mIAB-indicator in later meeting. We should have a proposal here: Proposal 2’: RAN3 to decide if to support NGAP indication to AMF for mobile IAB-authorization based on progress in SA2. |
| Huawei | Option 4 |  |
| Ericsson | 4 |  |
| ZTE | Option 4 | Based on moderator’s observation, according to the EN in chapter 5.35A in TS 23.501, SA2 is still discussing whether to reuse existing IAB authorized indication for MBSR. But the TP for 38.413 on Mobile IAB Authorized IE has been agreed unseen in R3-231955 during last RAN3 online session. How should we handle it? Shall we still wait SA2 progress on it? Editor's note: It is FFS whether the MBSR indication is a new IE or part of UE 5G MM capabilities. Editor's note: Whether existing IAB-Operation allowed indication and IAB authorized indication can be reused for MBSR will be determined. |
| Nokia | 4 |  |
| CATT | Option 4 |  |

Summary: seems the following can be agreed:

Proposal 2: RAN3 to decide if to support NGAP indication to AMF for mobile IAB-authorization based on progress in SA2.

MBSR involved positioning

### Naming of the new TRP type and stage3 details

Since P5 “RAN3 agrees to introduce a new TRP type for MBSR in NRPPa/F1AP.” can be agreed, we can work the TP in 2nd round, but companies have different views on the naming.

The following options can be further considered, and please companies also consider below information for down-selection

- The new TRP type should be associated with MBSR in the RAN specifications

- The mobile TRP or the TRP provided by MBSR can include mobile TP, mobile RP, mobile TRP.

| Options | Possible impacts |
| --- | --- |
| Option 1 | Stage 3: new codepoint: MBSR Stage 2: clarify the relation between MBSR and mobile IAB, clarify MBSR including mobile TP, RP and TRP. |
| Option 2 | Stage 3: new codepoint: mobile TP, mobile RP, mobile TRP Stage 2, clarify the relation between MBSR and mobile TRP |
| Option 3 | Other suggestions are welcomed. |
| … | … |

Which option is preferred? Comments?

| Company | Option | Comments |
| --- | --- | --- |
| Xiaomi | Option 1 or 2 |  |
| Qualcomm | Option 3 | Stage 3: Codepoint: “Mobile IAB” NRPPa and F1AP are both in RAN3 realm. We therefore should use RAN3 terminology. We have never used the term “MBSR” but always “mobile IAB”. Stage 2: Clarity that mobile IAB-DU can include mobile TP, RP, TRP. We should not discuss the relation between MBSR and mobile IAB. We don’t even know this relation. RAN3 has only discussed mobile IAB. |
| Huawei | See comment | We prefer mobile TRP as new codepoint, which looks more general. MBSR is not a popular terminology used in RAN. And a question for clarification: Why need 3 types for option 2? For stage 2 CR, is this really necessary? It seems that the stage 2 will be captured in SA2’s spec. The IAB-MT just works as normal UE no matter using which positioning method. So, maybe nothing to be captured. |
| Ericsson | 1 |  |
| ZTE | 3 | The term “MBSR” is not used in RAN3 and should be avoided. Either “mobile IAB” or “mobile TRP” is ok. |
| Nokia | 3 | This is TRP type, so it should be “mobile TRP”. |
| CATT | 3 | New Codepoint should be “mobile TRP” and clarify in stage-2 spec that mobile IAB-node can be a mobile TRP. |

Summary: no consensus, no proposal, discuss this next meeting.

### LS to SA2

During 1st round, company have concerns on SA2’s stage2 procedure, and would like to check with SA2.

What the following issue you would like to confirm with SA2?

| No. | Possible concerns to check with SA2 |
| --- | --- |
| 1 | Whether SA2 is aware of GPSI availability |
| 2 | Whether SA2 is aware of GSPI security |
| 3 | Whether SA2 is aware of Inter-donor migration issue |
| … | … |

Which of the above issues should be checked with SA2? Comments?

| Company | Option | Comments |
| --- | --- | --- |
| Xiaomi | None | We think SA2 is aware of these, no need to check. GPSI can be used in NRPPa/F1AP anyway, it’s already agreed in SA2. |
| Qualcomm | None | SA2 is aware of these issues. No babysitting needed. |
| Huawei | 4 | Clarification on how the P4 related method works, and ask them the question related to P12. |
| Ericsson | Nada |  |
| ZTE | None |  |
| Nokia | No | It is not in RAN3 scope. |
| CATT | None |  |

Summary: no need LS to SA2, no proposal, company can still to clarify the issue by contribution-driven.

### Controversial issue

During 1st round, the following proposals are controversial, the moderator updated the wording and try to reflect the discussion.

Proposal 11, RAN3 to discuss whether the cell ID and TRP ID associated with the MBSR are changed after inter-donor DU migration.

Proposal 12, RAN3 wait for SA2’s progress on how to trigger LMF initiating new TRP information exchange procedure to obtain the latest TRP information after inter-donor DU migration.

Do you agree with above proposals? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Qualcomm | No | Propose the following: Proposal: The cell ID and TRP ID associated with the mIAB-DU cease to exist after the mIAB-DU’s F1AP association has been released, e.g., due to DU migration. |
| Huawei | no | P11: Why need discuss this, isn’t this obviously correct? P12: At least, we can tell SA2 on the issue we found here. |
| Ericsson | No | The proposals seem to be unnecessary. |
| ZTE | P11: No P12: Yes | P11: not sure about the intention of this proposal, is there any RAN3 impact? P12: this issue has already been identified and discussed in SA2, as captured in section 6.1.4 in TS 23.273: - When the MBSR is integrated or fully migrated to a new gNB, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a gNB is mobile and its MBSR IAB UE ID (GPSI) via a TRP information exchange towards the gNB with the Cell ID of the TRP. The LMF may also performs NRPPa TRP information exchange procedure before the positioning procedure for a target UE, e.g. to determine the position capability of NG-RAN. Editor's note: The trigger of LMF to initiate the TRP information exchange needs further discussion for the case of MBSR's inter-CU mobility.. |
| Nokia | No | P11: as we commented early, there is No TRP change. LMF just see one TRP left, and another new TRP joined. |
| CATT | P11: No P12: Yes | P11 is unnecessary. |

Summary: issue is not acknowledged, no proposal, company can still to clarify the issue by contribution-driven.

Additional ULI

In the 1st round discussion, all companies agreed to introduce additional ULI over NGAP but have different views on the detail IEs. CGI should be considered. FFS on TAI and time stamp.

- For point#7 (regarding KI#6), SA2 would like to clarify that additional information besides existing ULI from donor-gNB (as defined in Rel-17) is needed, so that the 5GC can understand that the existing ULI cannot be used directly. This is crucial for the support of services that rely on the cell ID to infer the UE locations, e.g. emergency services. Therefore, SA2 would like to request RAN3 to either confirm the support of additional ULI for the UE serviced by an MBSR or provide an alternative solution.

Regarding TAI, one company thinks SA2 haven’t ask RAN3 to provide TAI according to their LS in R3-230032/ S2-2211437 as below, so their view is that only CGI of IAB-MT is useful, TAI of IAB-MT seems not useful for the core network.

### 5.35A.6	Providing cell ID/TAC of MBSR for services

Editor's note:	The description of how to provide the MBSR cell ID/TAC for certain services will be enhanced if needed based on RAN WG work progress.

The TAC and cell ID broadcasted by the MBSR cell(s) are configured as specified in TS 38.470 [165].

When a UE is served by a MBSR cell, the IAB-donor-CU may provide 'Additional ULI', User Location Information, in N2 messages. The 'Additional ULI' is the ULI of the IAB-UE. The CN may consider the 'Additional ULI' when it manages the UE location related functions (e.g. Mobility Restrictions).

On the other hand, after check SA2’s progress, the following text was agreed in CR S2-2304182r01. The text indicates that the CN may use additional ULI for mobility restrictions, and TAC is part of the mobile restriction information “Service Area Information” in TS 38.413. This means TAI of IAB-MT is needed as additional ULI.

Thus, we only need to discuss whether the TAI of UE is the same as TAI of IAB-MT, but according to the discussion in CB: IAB3, it’s hard to conclude whether TAI of the IAB-MT is the same as broadcasted by IAB-DU this meeting, so it’s suggested to postpone the discussion next meeting.

Proposal 15, RAN3 discuss whether to include the TAC of IAB-MT in the additional ULI based on the conclusion of dynamic TAC.

Do you agree with this proposal? Comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Qualcomm |  | SA2 explicitly states: “SA2 would like to request RAN3 to either confirm the support of additional ULI for the UE serviced by an MBSR or provide an alternative solution”. This implies that RAN3 should include the ULI of the IAB-MT or provide an alternative solution. Nobody has proposed an alternative solution that provides the information carried in the MT’s ULI the AMF. This means that we go for the MT’s ULI. Why do we need more discussion? Proposal 15: The mIAB-DU’s CU to include the mIAB-MT’s ULI into location updates for UEs connected to the mIAB-DU. |
| Huawei |  | We think the TAC is necessary. Also ok to discuss it later if no consensus at this meeting. |
| Ericsson | See comment | The wording does not sound like a proposal but like an FFS. Anyway, ULI is ULI, and we agreed to include mIAB-MT’s ULI, while the above SA2 excerpt states that “The 'Additional ULI' is the ULI of the IAB-UE”. Hence the TAC seems to be needed. |
| ZTE | No | We don’t need this proposal. Whether to include the TAI of the cell serving IAB-MT is irrelevant to the conclusion of whether DU’s broadcast TAC is the same as the TAC of the cell serving the mIAB-MT. In the reply LS from RAN3 R3-226048, RAN3 doubted the necessity of introding additional ULI considering that RAN3 is discussing whether the TAC broadcast by the mobile IAB-cell should change with the IAB-node’s movement to reflect the IAB-node’s actual location. RAN3’s feedback on point #7: RAN3 believes that the feasibility of the signaling enhancement proposed by SA2 would need further assessment for the scenarios, where IAB-MT and IAB-DU are connected to different IAB-donors. RAN3 is currently discussing whether the TAC broadcast by the mobile IAB-cell should change with the IAB-node’s movement to reflect the IAB-node’s actual location. In this context, RAN3 will discuss if a signaling enhancement as proposed by SA2 would be needed. RAN3 will provide feedback as this discussion progresses.. And then, in the reply LS from SA2 R3-230032, SA2 replied that additional ULI is needed for the support of services that rely on the cell ID to infer the UE locations. In other words, the including of TAI is not validated in the latest SA2 LS. For point#7 (regarding KI#6), SA2 would like to clarify that additional information besides existing ULI from donor-gNB (as defined in Rel-17) is needed, so that the 5GC can understand that the existing ULI cannot be used directly. This is crucial for the support of services that rely on the cell ID to infer the UE locations, e.g. emergency services. Therefore, SA2 would like to request RAN3 to either confirm the support of additional ULI for the UE serviced by an MBSR or provide an alternative solution.. On the other hand, we can look at the motivation of additional ULI, which is related to Key Issue#6. As captured in TR 23.700, the point of Key Issue#6 is that the serving cell ID/TAC of the UE may not always accurately reflect the location of the UE. In this context, as RAN3 has already agreed that the TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location, we believe there is no motivation to include TAI info as additional ULI together with UE ULI, no matter whether the DU’s broadcast TAI is the same as its parent cell. |
| Nokia | No | This is to be continued, and shall be in blue text, rather agreement. |
| CATT | See comment | We believe it should go to an agreement rather than FFS that the mIAB-MT’s ULI is indicated in addition to the UE’s ULI. To support that, TAC is necessary to be included in the mIAB-MT’s ULI. Note that the TAC broadcasted by IAB-DU can be different with that of the mIAB-MT’s serving cell, i.e., different with the TAC indicated in the mIAB-MT’s ULI. What TAC should be broadcasted by mIAB-DU is discussed in CB MobEnhance, it is another issue. |

Summary: some companies still think the TAI of IAB-MT’s serving cell can be reflected by existing IE e.g. TAI of UE’s serving cell. This issue can be TBC for next meeting.

RAN3 to discuss whether to include the TAI of IAB-MT’s serving cell into the new IAB-MT User Location Information IE.

Regarding time stamp, companies are invited to provide views on whether the time stamp of IAB-MT’s ULI is the same as the time stamp of UE’s ULI.

Proposal 16, RAN3 to discuss whether the time stamp of IAB-MT’s ULI is different from the time stamp of UE’s ULI?

- Option 1, If yes, include the additional time stamp in the additional ULI.

- Option 2, If no, not include the time stamp in the additional ULI

Which option is preferred? Comments?

| Company | Option | Comments |
| --- | --- | --- |
| Xiaomi | Option 1 | The time stamp reflects the time of HO, in partial migration, the IAB-MT performs HO, the ULI of IAB-MT including the time stamp will be changed, but the UE may not perform HO, which means the time stamp of IAB-MT’ULI can not be reflected by the time stamp of UE’s ULI |
| Qualcomm | These are not separate options since they refer to different conditions. | mIAB-MT’s timestamp should always be included. We should not spend time on minuscule optimizations. |
| Huawei | Option 2 | In our view, the additional ULI includes the current CGI/TAI of IAB-MT’s serving cell when RAN providing the UE’s ULI, the time stamp (Age of Location IE) already included in the User location information IE is enough. |
| Ericsson | 1 | Same reasons as Xiaomi’s. |
| ZTE | Option 1 | MT’s ULI and UE’s ULI may be determined at different time. |
| Nokia | 2 | This maybe not necessary. CN only need to know the UE’s location when the UE’s ULI is generated. It is enough as long as it is a valid IAB-MT’s ULI when sending the UE’s ULI. For Option 1, Please clarify the different behavior to the AMF, e.g. AMF receive the IAB-MT’s TAC+NCGI with timestamp 10:00 in one case, and same TAC+NCGI with timestamp 10:05 in another case. |
| CATT | Option 2 | Not clear why another time stamp is necessary given there is already time stamp in the UE’s ULI. |

Tentative Summary: some companies think UE’s timestamp is enough, but some companies think the time stamp of IAB-MT’s ULI is needed. This issue can be TBC for next meeting,

RAN3 to discuss whether to include the time stamp of IAB-MT’s serving cell into the new IAB-MT User Location Information IE.

Summary

Mobile IAB indication over NGAP

Proposal 1: RAN3 agrees that the IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5.

Proposal 2: RAN3 to decide whether to support NGAP indication to AMF for mobile IAB-authorization based on progress in SA2.

Mobile IAB-node involved positioning

Proposal 3, WA: RAN3 assumes that LMF obtains an updated location and velocity information and location time stamp of the mobile IAB-node by performing either option 1 or option 2 during UE positioning, SA2’s progress on option 1 and option 2 should also be considered.

Proposal 4, WA: if Network Assisted procedure is used (i.e. UL related positioning is performed), LMF may obtain an updated location and velocity information of the mobile IAB-node in UL measurement result related message(s) in NRPPa and F1AP procedure. FFS on which message(s). FFS on whether time stamp for the location of mobile IAB-node is needed

Proposal 5, RAN3 agrees to introduce a new TRP type for mobile IAB-node in NRPPa/F1AP.

Proposal 6, if option 1 is supported, the NRPPa/F1AP TRP information exchange procedure can be used to trigger MO-LR procedure of IAB-MT to obtain the location and velocity information of mobile IAB-node as well as the timestamp.

Proposal 7: if GPSI is available, mobile IAB-MT’s UE ID (i.e. GPSI) can be included in NRPPa/F1AP TRP information response message so that LMF can perform MT-LR procedure to obtain mobile IAB-node’s location.

Proposal 8, Checking the security issue of GPSI with SA3 is out of RAN3 scope.

Proposal 9, the issue for the case that “LMF that performs the location estimation of the mobile IAB-node can be different than the LMF that performs the location estimation of the target UE” is not in RAN3 scope.

Additional ULI

Proposal 10: RAN3 agrees that the IAB-DU’s donor-CU includes at least the NCGI of the cell serving IAB-MT as an additional ULI together with UE ULI over NGAP.

Proposal 11, RAN3 to select between the following options for IAB-DU’s donor CU to obtain the IAB-MT’s serving cell ID and TAI (FFS) in case IAB-MT and IAB-DU are connected to different IAB-donors

-           Option A, the cell ID and TAI (FFS) of IAB-MT’s serving cell is passed from the IAB-MT’s donor-CU to IAB-DU’s donor CU.

-           Option B, the cell ID and TAI (FFS) of IAB-MT’s serving cell is passed from IAB-DU to IAB-DU’s donor CU.

To be continued:

RAN3 to discuss whether to include the TAI and the time stamp of the location information of IAB-MT’s serving cell into the new IAB-MT User Location Information IE.

References

[1]  RP-221815, WID on Mobile IAB for NR, 3GPP TSG RAN#96, Budapest, Hungary, June 2022