3GPP TSG-RAN WG3 #119bis-e						       R3-232116

17th – 26th April 2023

Online

Agenda Item:	13.4

Source:	ZTE (moderator)

Title:	        Summary of CB # IAB4_MitInt

Document for:	Approval

Introduction

This paper is for the following offline discussion:

CB: # IAB4_MitInt - Discuss and converge on the use case of PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs - Is the use case in scope of RAN3 work? - Should the case of no Xn between F1-terminating donor and the MT’s target donor be considered? - Should the cases of centralized and distributed PCI assignment be differentiated? (moderator - ZTE) Summary of offline disc R3-231903

This discussion has two phases:

Phase 1: Converge on potential proposals. Please give your feedback before Wednesday, 19th April, 2023, 12:00 UTC.

Phase 2: Further discussion on the identified issues. Please give your feedback before Monday April 24, 18:00 UTC.

The following contributions will be discussed in this CB:

| R3-231359 | Discussion on PCI collision avoidance for mobile IAB (ZTE) | discussion |
| --- | --- | --- |
| R3-231443 | PCI collision mitigation of mobile IAB-node mobility (Lenovo) | discussion |
| R3-231473 | Mobile IAB interference mitigation (Nokia, Nokia Shanghai Bell) | discussion |
| R3-231485 | PCI collision for mobile IAB (Huawei) | discussion |
| R3-231526 | Discussion on mitigation of interference (Xiaomi) | discussion |
| R3-231537 | PCI Collision Avoidance for Mobile IAB-Nodes (Ericsson) | discussion |
| R3-231720 | Discussion on mitigation of interference (Samsung) | discussion |

For the Chair’s Notes

Propose to capture the following Agreements:

…

Proposal 4: RAN3 to discuss how to avoid PCI collision in the scenario with Xn between IAB-DU’s donor and IAB-MT’s donor.

Proposal 4a: RAN3 to discuss how to avoid PCI collision in the scenario without Xn between IAB-DU’s donor and IAB-MT’s donor if the scenario is supported.

Proposal 5: RAN3 to discuss whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report.

PHASE 1: Discussion

During RAN3#117bis-e meeting, the PCI collision issue was discussed and the following agreements were reached:

PCI Space Partitioning is performed by OAM and up to implementation. As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message. PCI-change on the IAB-node can be supported via handover of connected UEs between cells using old and new PCI, respectively. FFS for the PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs. PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node.

PCI collision detection/prediction during partial migration without Xn connection

During RAN3#119, the following agreements were achieved regarding the case in absence of Xn connection, which means support of partial migration when there is no Xn between F1-terminating donor and the MT’s target donor needs to be considered.

RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available. For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF.

- In [R3-231359 ZTE], it is observed that in the scenario when partial migration is executed and there is no Xn connection between the F1-terminating donor and the MT’s target donor, there is some problem for the F1-terminating IAB-donor to detect/predict PCI collision.

The reason is that the F1-terminating donor may be far away from the MT’s target donor and is not able to obtain the PCIs used by the serving cells and neighbouring cells of the MT’s target donor considering that there is no Xn connection between the two donors.

Furthermore, it is observed that PCI collision cannot be predicted/avoided based on UE measurement report. In UE report method, UEs served by the mobile IAB node report the detected PCI and corresponding NCGI after the mobile IAB node moves into the coverage of the cell with the same PCI. In this situation, the UE report method doesn’t help since PCI collision is detected after the collision occurs, which is too late.

- In [R3-231443 Lenovo], it is suggested that in case of partial migration without Xn connection, F1-terminating IAB-donor-CU can detect the potential PCI collision based on measurement report from served UEs.

- In [R3-231526 Xiaomi], it is suggested that this issue can be solved based on the existing signalling, i.e. measurement information reported from the UE or neighbour information transferred from the non-F1 terminating donor.

- In [R3-231485 HW], it is observed that measurement report from served UEs may not work well, i.e. the UE may not be able to report the cell information (if has same PCI with the mobile IAB-DU) correctly, because this cell will cause big interference to the UE and it is not distinguishable to UE. And it is suggested that PCI collision can be detected by the mobile IAB-node in the absence of Xn interface. And it is suggested that a list of PCIs can be pre-configured to the mobile IAB node, and the mobile IAB node select a new one if detects potential confliction. In the moderator’s view, it’s not clear how could the mobile IAB node detects PCI collision by itself in the absence of Xn interface.

- In [R3-231720 SS], it is suggested that the source IAB donor CU should inform the target IAB donor CU of PCI collision, and the target IAB donor CU decides the new PCI and forwards it to the source IAB donor CU for PCI reconfiguration of mobile IAB cell. However, in the moderator’s view, it’s not clear how could the source donor detect/predict the PCI collision for the mobile IAB node.

- In [R3-231537 Ericsson], It is observed that PCI collision detection by the mIAB-MT node is not always possible, and PCI collision can be predicted by sharing the information about configured PCIs between IAB-donors. In the moderator’s view, it’s not clear how could information about configured PCIs be shared between IAB-donors in absence of Xn connection.

- In [R3-231473 Nokia], it is observed that existing method (e.g., UE ANR reporting) can be used,  and the RAN serving the mobile IAB-nodes will be aware of the mobile IAB locations and the mobility history at least on the cell resolution which will be in normal cases enough to detect or estimate the probability of colliding PCIs of two mobile IAB-nodes. And it is suggested that no further enhancements are needed for PCI collision detection/avoidance.

Q1: Do you agree that partial migration when there is no Xn between F1-terminating donor and the MT’s target donor needs to be considered based on RAN3 agreements?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Yes, but nothing new is needed | See our answer to Q2. |
| Qualcomm | See comment | The only thing that matter here is the MT and DU are connected to separate CUs without Xn. The UEs can be configured to include CGI-InfoNR in the measurement results, which allows the DU’s CU to derive that the UE sees two cells with same PCI but different NGCI. The DU’s CU can then reconfigure the mIAB-DU-cells PCI to a value that has not be reported by the mIAB-DU’s UEs. This procedure can be autonomously applied by the DU’s CU without need for Xn. |
| Lenovo | Yes, but | Agree to consider the scenario, but this can be handled by F1-terminating IAB-donor-CU based on measurement report from served UEs. |
| Nokia | No | If no Xn, then no partial migration. What is the scenario for partial migration without Xn? Please clarify the scenario. |
| Xiaomi | No | Nokia has a good point, and the agreement above doesn’t mean the scenario will be supported. |
| Samsung | See comment | Even though there is no Xn between F1-terminating donor and the MT’s target donor, the PCI collision can also be detected by UE measurement report. However, at this time PCI collision has already occurred and UEs have be impacted. In order to avoid/mitigate impact on UEs, it is better to consider Xn between F1-terminating donor and the MT’s target donor during multiple partial migration for PCI collision prediction. |
| Huawei | See the comments | First, the scenario where there is no Xn between F1-terminating donor and the MT’s target donor will appear if we do nothing on the MT or DU’s target CU selection. Therefore, we propose to consider how to avoid the scenario of no Xn between F1-terminating donor and MT’s target donor in the target donor selection stage, as analyzed in our discussion paper in AI 13.2. If the Xn interface is ensured, there is no issue for the PCI collision detection, because the two CUs can exchange information about the served cells, and can detect the potential PCI collision in advance, using existing mechanism. Second, if we do not have consensus on selecting target CU with Xn for the consecutive partial migration, there is issue if no Xn interface between F1-terminating donor and the MT’s target donor. The issue is: F1 terminating CU has no idea about the info of cells near the mIAB-node, and cannot detect the PCI collision. |
| ZTE | Yes | RAN3 agreements implies that partial migration when there is no Xn between F1-terminating donor and the MT’s target donor needs to be supported. |

Summary:

8 companies provided feedback on Q1.

6/8 companies agree that the scenario of partial migration without Xn between F1-terminating donor and the MT’s target donor needs to be supported in R18. 3/6 companies think this scenario can be supported via current UE measurement report while 3/6 companies think current UE measurement report mechanism cannot solve the problem since it cannot help to avoid PCI collision.

2/8 companies doubts RAN3 agreements doesn’t imply that partial migration without Xn between F1-terminating donor and the MT’s target donor is to be supported.

1/8 company thinks this scenario will appear if we do nothing on the MT or DU’s target CU selection and proposes to consider how to avoid this scenario. If the scenario cannot be avoided, it needs to be discussed. The moderator believes this can be discussed in AI 13.2.

Reply to nokia: partial migration is designed when there is Xn in R17. In R18, RAN3 is discussing how partial migration is supported without Xn. Please refer to the RAN3 agreements copied in the below. So it’s incorrect to say “If no Xn, then no partial migration.”

RAN3#117 agreement: RAN3 to discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn and/or inter-donor IP routability. RAN3#119 agreement: RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available. For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF.

Based on the discussion, the moderator emphasize that one of the objective of R18 IAB WID is to avoid potential reference and control signal collisions. That means the PCI collision needs to be predicted in advance rather than detected after the PCI collsion occurs.

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

So the moderator proposes:

Proposal 1: RAN3 to discuss how to avoid PCI collision in the scenario when there is no Xn between DU’s donor and the MT’s donor.

Q2: During partial migration without Xn connection between F1-terminating donor and MT’s target donor, which of the following options is used for PCI collision detection/prediction for mobile IAB node? Please explain how it works in detail.

- Option 1: based on UE measurement report

- Option 2: detected by the mobile IAB-node itself, please explain how could mobile IAB-node detects the PCI collision.

- Option 3: by the F1-terminating donor, please explain how could the F1-terminating donor be aware of the PCIs used by the serving cells and neighbouring cells of the MT’s target donor in absence of Xn connection between the two donors.

- Option 4: by MT’s target donor

| Company | Option 1/2/3/4 | Comments |
| --- | --- | --- |
| Ericsson | 1 and 3 | Option 3 can be used when there is XnAP connectivity between the mIAB-MT’s CU and mIAB-DU’s CU. If there exists XnAP connectivity between mIAB-DU’s CU and mIAB-MT’s CU, the PCI collision can be predicted by sharing the PCI information between donors. For the case when there is no XnAP connectivity, Option 1 can be used. Option 1 is legacy – the UEs can read PCIs of surrounding cells and report them to the mIAB-DU’s donor. If there is no XnAP connectivity between donors, the mIAB-DU’s donor can, detect PCI collision, based on UE reports. The F1-terminating donor CU can then change the PCI of the DU cell depending upon whether already standardized distributed or centralized PCI allocation schemes are in use by the operator. In Option 1, the CU can memorize that there has been a PCI collision, together with the location information of the UE and mIAB-node. This knowledge, from that point on, de facto enables PCI prediction - later on, if needed, the CU can use this knowledge to pre-emptively reconfigure the PCI every time one of its mIAB nodes traverses the area where the PCI collision occurred. Of course, Option 1 can be also used when XnAP connectivity exists. These two options are sufficient. |
| Qualcomm | 1 and 3 | The main point here is that MT and DU connect to different CUs without Xn. Option 1: The UE measurement reports provide info on cells with same PCI but different NCGI. Option 3: The F1-terminating donor is the donor receiving these measurement reports and it is in the position to reconfigure the mIAB-DU’s cell PCI in case there is a collision. Option 2: Yes, the IAB-node performs the L3 measurements it reports so it is also in the position to derive information on potential PCI collisions. However, it does not have any means to mitigate such collisions. Option 4: The MT’s donor receives MT’s measurement reports. However, the MT’s measurement report may not be indicative about PCI collisions experienced by the mIAB-DU’s cells. Further, the MT’s donor wouldn’t be able to mitigate such collisions since it cannot reconfigure the DU’s cell PCI. |
| Lenovo | Option 1 |  |
| Xiaomi | Option 1 and 3 | We share similar view with E/// |
| Samsung | 1 and 3 | For option 1, PCI collision can be detected by UE measurement report when there is no Xn between F1-terminating donor and the MT’s target donor. However, PCI collision has already occurred and UEs have been impacted. Even though F1-terminating donor can memorize the information on PCI collision reported by UEs, PCIs of surrounding cells reported by UE may also be changed in the future. For option 3, share the same view with Ericsson, which is that the PCI collision can be predicted by sharing the PCI information between donors when Xn interface exists. |
| Huawei | Option 2 | Option 1 does not work, because the UE served by the mIAB cannot distinguish two cells based on measurement, because the two cells’ signal will cause strong interference to each other, UE may even not able to decode the serving cell signaling correctly if suffering strong interference. For option 2, the mobile IAB-node detects the PCI collision based on the MT’s measurement, considering that the co-located DU’s transmission will not impact the IAB-MT’s measurement due to proper duplex design. The IAB-MT can measure a neighbor cell, and know the cell’s PCIs. When the MT find a neighbor cell’s PCI is same as its co-located DU’s cell, the MT can detect the PCI collision. Option 3 and 4 has similar problem, because there is no Xn interface between the two CUs, neither of the two donors can detect the PCI collision. |
| ZTE | Option 4 | Option 1 cannot work to predict the PCI collision. In UE report method, UEs served by the mobile IAB node report the detected PCI and corresponding NCGI after the mobile IAB node moves into the coverage of the cell with the same PCI. In this situation, the UE report method doesn’t help since PCI collision is detected after the collision occurs, which is too late. On QC’s comment, we would like to clarify that option 4 is not based on UE measurement report. MT’s source donor can send the PCI used by the mobile IAB node to MT’s target donor, e.g., in the handover request message for MT HO. MT’s donor can predict the PCI collision based on the received PCI in the HO request message. And MT’s donor can reconfigure PCI for the mobile IAB node if needed via the HO cmd message. |

Summary:

7 companies provided feedback on Q2.

5/7 companies think option1 (i.e. based on UE measurement report) can be used to detect/predict PCI collision for partial migration when there is no Xn connection. Specifically, one company suggests that the CU can memorize that there has been a PCI collision, together with the location information of the UE and mIAB-node. In moderator’s view, the PCI collision may occure between mobile IAB nodes rather than between mobile IAB node and stationary gNB. For PCI collision between mobile IAB nodes, it cannot be predicted by history information and location information.

3/7 companies believe option 1 cannot work.

1/7 company think option 2 (i.e. detected by the mobile IAB-node itself) can be used in this scenario.

1/7 company think option 4(i.e. detected by MT’s target donor after receiving PCI used by mIAB cell in HO request for MT HO) can work in this scenario.

Based on the discussion, the moderator proposes:

Proposal 2: RAN3 to discuss whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report in the scenario when there is no Xn between DU’s donor and the MT’s donor. FFS on PCI collision detection/prediction by the MT’s donor or mobile IAB-node.

Q3: During partial migration without Xn connection between F1-terminating donor and MT’s target donor, which of the following options is used to reconfigure the PCI for mobile IAB cell upon PCI collision?

- Option 1: the mobile IAB node select a new one from the pre-configured PCI list

- Option 2: reconfigured by the F1-terminating donor via F1

- Option 3: reconfigured by the MT’s target donor via RRC

| Company | Option 1/2/3 | Comments |
| --- | --- | --- |
| Ericsson | Option 2 | The DU cells are under the control of the F1-terminating donor CU so it should be in charge of reconfiguring the PCIs through legacy procedures. Allowing the mIAB-node of the mIAB-MT’s donor to reconfigure the mIAB-node’s PCI can lead to a domino effect of PCI collisions in the surrounding network, forcing other mIAB-nodes and gNBs nearby to change their PCIs. Hence, Options 1 and 3 should be avoided. |
| Qualcomm | Option 2 | Agree with Ericsson |
| Lenovo | Option 2 | Agree with Ericsson |
| Xiaomi | Option 2 + OAM | Agree with E///’s view on option2, we also think the PCI can reconfigured by OAM |
| Samsung | See comment | Because F1-terminating donor may be far away from MT’s target donor and have no Xn with MT’s target donor and neighbor donors of MT’s target donor, it may have no idea about the PCI assignment of surrounding donors of MT’s target donor. Even though PCI collision can be detected by F1-terminating donor based on UE measurement report, MT’s target donor has a better acknowledge about the surrounding donors of itself. If F1-terminating donor reconfigures PCI for mIAB directly, PCI collision may happen again when mIAB moves into surrounding donors of MT’s target donor. Therefore, a better way is that MT’s target donor decides the new PCI and informs F1-terminating donor of it, then F1-terminating donor reconfigures this new PCI for mIAB. |
| Huawei | Option 1 | The available PCI list for the mobile IAB node can be pre-configured on it. If the mobile IAB-node detects the PCI collision, it can select a non-collided PCI from the PCI list to replace the collided one, and inform that to the F1 terminating donor. |
| ZTE | Option 3 | In the scenario when there is no Xn between DU’s donor and the MT’s donor, if the PCI collision is predicted by the MT’s donor as discussed in Q2, the new PCI should be configured by MT’s donor. On Ericsson’s comment, there is no domino effect if the PCI is configured by MT’s donor since the MT’s donor is quite aware of the PCIs used in surrounding cells of the mobile IAB cell. And it’s quite common to configure DU parameters via RRC. For instance, in R16, IAB node’s BAP address, IP address and default BAP configuration are configured via RRC. |

7 companies provided feedback on Q3.

4/7 companies supports option 2 (reconfigured by the F1-terminating donor via F1).

2/7 companies suggest that MT’s target donor is preferred to reconfigure PCI for mobile IAB cell in the scenario when there is no Xn between DU’s donor and the MT’s donor since MT’s target donor has a better acknowledge about the surrounding donors of the mobile IAB node.

1/7 company supports option 1 (reconfigured by the mobile IAB node) and suggests that the available PCI list for the mobile IAB node can be pre-configured.

In the moderator’s view, the PCI reconfiguration is some kind of dependant on how PCI collision is detected/predicted. So the moderator sugguest to have no proposal on PCI reconfiguration at this stage and wait for the progress on the PCI collision prediction in the scenario when there is no Xn between DU’s donor and the MT’s donor.

PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs

In [R3-231443 Lenovo][R3-231526 Xiaomi], it is observed that legacy PCI Optimization Function cannot work when the mobile IAB-node and the IAB-donor-CU are served by different OAMs.

In [R3-231443 Lenovo], the following enhancements are proposed to solve this issue:

For centralized PCI assignment in split gNB architecture, IAB-donor-CU and mobile IAB-node have following enhancements to legacy PCI Optimization Function if mobile IAB-node and IAB-donor-CU are served by different OAMs. - IAB-donor-CU informs the PCI collision to mobile IAB-node - Mobile IAB-node reports the new PCI to IAB-donor-CU For distributed PCI assignment in split gNB architecture, mobile IAB-node may report the PCI list assigned by its OAM to IAB-donor-CU, and the IAB-donor-CU may select a new PCI value from the PCI list to upon PCI collision, if mobile IAB-node and IAB-donor-CU are served by different OAMs.

In [R3-231526 Xiaomi], the following enhancements are proposed to solve this issue:

If it’s the mobile IAB node (or it’s OAM) to assign the new PCI, the PCI collision detection indication and the cell configuration of the neighbours should be transferred from IAB-donor of mobile IAB-DU to the mobile IAB-node. If it’s IAB-donor of mobile IAB-DU (or it’s OAM) to assign the new PCI, we think the pre-configurations PCI list in the mobile IAB-node and the IAB-node movement info should be transferred from the mobile IAB-node to IAB-donor of mobile IAB-DU.

In [R3-231537 Ericsson], it is proposed that PCI reconfiguration for the scenario where the IAB-donor and the mIAB-node connect to different OAMs is out of RAN3 scope. And it is observed that it is always assumed that the DU and CU can be connected to different OAMs in our standardization work.

In [R3-231473 Nokia], it is observed that from RAN3 perspective, the IAB node and IAB-donor may use different OAMs, just like normal gNB-DU and gNB-CU that may use different OAMs. And it is proposed that no enhancement is needed when the IAB node and IAB-donor use different OAMs.

Q4-1: Do you agree that normal DU and CU can be connected to different OAMs, and existing mechanism is sufficient for PCI reconfiguration in normal split gNB architecture where DU and CU are connected to different OAMs? If no, please provide the reason.

Q4-2: Do you agree that the case where IAB-donor and IAB-node are connected to different OAMs is the same as normal split gNB architecture where DU and CU are connected to different OAMs? If no, please provide the reason.

Q4-3: Do you agree that existing mechanism is sufficient for PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs? If no, please provide the reason and the solution.

Q4-4: Do you agree that the PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs is out of RAN3 scope?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Q4-1: agree Q4-2: Agree Q4-3: Agree Q4-4: Agree |  |
| Qualcomm | Q4-1: Agree Q4-2: Agree Q4-3: see comment Q4-4: see comment | On Q4-3: PCI reconfiguration should avoid UE RLF. We agreed that this can be achieved by handing over UEs between cells with old and new PCIs, respectively. This is straightforward but we may not want to close the door yet on further discussion. On Q4-4: We cannot simply state that it is out of scope. RAN3 cannot assume that two RAN nodes are supported by the same OAM. |
| Lenovo | See comments | For Q4-1 and Q4-2, agree that CU and DU can connect to different OAMs, but the existing mechanism may be only work well for the case if CU and DU connect to the same OAM. For example, in centralized PCI assignment in split gNB architecture, it’s the CU to detect the PCI collision and reconfigure the PCI, and CU reports the collision to its OAM. However, if CU and DU subject to different OAM, it’s better to report the collision to the OAM of DU to request a new PCI, rather than to report the collision to the OAM of CU. Agree with QC for Q4-3 and Q4-4 to keep them open. |
| Nokia | Agree all | Please note current system already support DU and CU from different vendors and using different OAM. |
| Xiaomi | See comments | Share view with Lenovo. We think the most difference between current system and mobile IAB involved network is that the PCI conflict can be very dynamic and frequent for the latter case, we need a better mechanism to avoid PCI collision. In general, we prefer to keep Q4-3 and Q4-4 open. |
| Samsung | Q4-1: agree Q4-2: Agree Q4-3: Agree Q4-4: Agree |  |
| Huawei | Agree all | No enhancement is needed. |
| ZTE | Agree to all |  |

Summary:

8 companies provided feedback on Q4.

5/8 companies agree to all the potential agreements, i.e. existing mechanism is sufficient for PCI reconfiguration in normal split gNB architecture where DU and CU are connected to different OAMs and existing mechanism is also sufficient for PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs.

1 company agree to the first 2 of them and think we sould leave the issue for mobile IAB node open at current stage.

2 companies think existing mechanism may only work well for the case if CU and DU connect to the same OAM, which implies that there is an isue for existing mechanism in split gNB architecture when gNB-CU and gNB-DU connect to the different OAMs.

Based on the disucussion, the moderator feels that we may not be able to achieve consensus on the OAM related aspects in RAN3 and suggests to send an LS to SA5 for confirmation.

Proposal 3: RAN3 to send an LS to SA5 with the following questions:

- Is current PCI Optimisation Function applicable for the scenario when gNB-CU and gNB-DU connect to the different OAMs.

- For mobile IAB node, is the current PCI Optimisation Function applicable for the scenario when IAB donor CU and mobile IAB-DU connect to different OAMs.

Others

Q5: Is there any other issue to be discussed for interference mitigation in mobile IAB scenario?

| Company | Comments |
| --- | --- |

PHASE II: Convergence of PH1

Avoidance of PCI collision

Based on the following RAN3 agreements, RAN3 needs to discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn.

RAN3#117 agreement: RAN3 to discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn and/or inter-donor IP routability. RAN3#119 agreement: For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF.

That implies the scenario when there is no Xn between DU’s donor and the MT’s donor needs to be supported and discussed. On the other hand, it should be noted that one of the objectives of R18 IAB WI is to avoid potential reference and control signal collisions. That means the PCI collision needs to be predicted in advance so that it can be avoided rather than detected after the PCI collsion occurs.

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

On the other hand, the following agreements were achieved in RAN3#117 bis meeting, i.e. PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node. In moderator’s understanding, if there is Xn between DU's donor CU and MT's target donor CU, DU's donor CU can detect PCI collision after receiving gNB ID of MT's target donor from MT's source donor CU, since DU's donor can exchange serving cell and neighbour cell information with MT's target donor via existing Xn signaling. In this way, PCI collision can only be detected by F1-terminating donor after receiving gNB ID of MT's target donor from MT's source donor CU after completion of IAB-MT HO. That means PCI collision cannot be avoided in this method.

As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message. PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node.

As discussed above, RAN3 needs to discuss how to avoid PCI collision considering the scenario with/without Xn between DU’s donor and the MT’s donor. So the following proposal was made after phase 1 discussion.

Proposal 4: RAN3 to discuss how to avoid PCI collision considering the scenario with/without Xn between DU’s donor and the MT’s donor.

Q6: Do you agree with P4, if not, please provide your reason/revision.

| Company | Yes/no | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei | No | If there is Xn interface, the F1 terminating CU can detect the PCI collision after obtain cells information served by the MT’s donor, the existing mechanism works well. The problem only exists when the Xn interface is absent. In such scenario, UE served by mobile IAB node may suffered strong interference due to PCI collision, and the two cells using same PCI cannot be distinguished by the UE. Suggest the following rewording: Proposal 4: RAN3 to discuss how to avoid PCI collision considering the scenario with/without Xn between DU’s donor and the MT’s donor, if supporting partial migration in such scenario. |
| Nokia |  | Let’s first discuss “with Xn”. The “No Xn” is lower priority. |
| Xiaomi |  | Agree with Nokia |
| Lenovo | No | In case having Xn between DU’s donor and the MT’s donor, there is no problem with the legacy mechanism. And in case without Xn between DU’s donor and the MT’s donor, it can be discussed later after we have confirmed to support this case, anyway, this case can be resolved based on the UE’s measurement report. |
| Samsung | Yes |  |
| Qualcomm | Yes | It seems companies have not converged if there are only issues in absence of Xn or also issues in presence of Xn. This topic is an explicit objective in the WID. This implies that we discuss it. Based on how P4 is presently worded, it would be a “To be continued:…” in blue. |
| Ericsson | No | There are no issues when XnAP connection is present. Wrt Moderator’s explanation, even if the mIAB-DU’s CU finds out the gNB-ID of mIAB-MT’s target after successful HO, it is still not too late. If it is not too late for TMM procedure, why would it be for PCI change? The PCI change procedure is not expected to impact the BH configuration under mIAB-DU’s CU. The case when XnAP connection is not present can be discussed once we dig into that scenario and start discussing partial migration therein. |

Summary:

8 companies provided feedback on Q6.

For the scenario with Xn:

5/8 companies agree to discuss this issue while 3/8 companies disagree to discuss and think PCI collision can be detected via existing mechanism. But the mechanism provided by the oponent companies can only detect PCI collision but cannot avoid PCI collision. In moderator’s view, there is enough support to discuss this issue.

For the scenario without Xn:

All 8 companies agree to discuss this issue, wherein 2 companies think this issue can be discussed after we confirmed that the scenario with Xn is supported, wherein 2 companies think the scenario without Xn should be low priority as previously agreed in RAN3.

Based on the discussion, the following proposal is made:

Proposal 4: RAN3 to discuss how to avoid PCI collision considering the scenario with Xn and without Xn between DU’s donor and the MT’s donor, the scenario without Xn is discussed only if the scenario is supported.

During phase 1 discussion, some companies think legacy UE measurement report can be used to detect/predict PCI collision, especially in the scenario when MT and DU are connected to separate CUs without Xn. And one company suggests that the CU can memorize that there has been a PCI collision, together with the location information of the UE and mIAB-node. While some companies believe that legacy UE measurement report cannot work to avoid PCI collision. In moderator’s view, the PCI collision may occure between mobile IAB cells rather than between mobile IAB cell and stationary gNB. For PCI collision between mobile IAB cells, it cannot be predicted by history information and location information. So the following proposal was made after phase 1 discussion.

Proposal 5: RAN3 to discuss whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report.

Q7: Do you agree with P5, if not, please provide your reason/revision.

| Company | Yes/no | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei | No | If we have P4, seems no need this P5. |
| Nokia | Yes |  |
| Xiaomi | Yes | Ok for further discussion |
| Lenovo | Yes |  |
| Samsung | Yes |  |
| Qualcomm | Yes | Applies independent of support of Xn. |
| Ericsson | Disagree | However, we can look at this once we start working on the no-XnAP case. As of today, we think that UE measurement reports can do the job. |

Summary

8 companies provided feedback on Q7.

6/8 agree with this proposal.

1 company think this proposal has been covered by P4. In moderator’s view, P5 is needed since it reflects the current discussion status that some companies think legacy UE measurement report is sufficient for PCI collision between mobile IAB cells.

1 company disagree but think can discuss it for the scnario without Xn.

Based on the discussion, the moderator propose:

Proposal 5: RAN3 to discuss whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report.

PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs

Based on phase 1 discussion, most companies believe that existing PCI Optimization Function is sufficient for PCI reconfiguration in normal split gNB architecture where DU and CU are connected to different OAMs and in case of IAB-donor and IAB-node with different OAMs. 2 companies think existing mechanism may only work well for the case if CU and DU connect to the same OAM, which implies that there is an isue for existing mechanism in split gNB architecture when gNB-CU and gNB-DU connect to the different OAMs.

During the phase 1 discussion, some company suggests to send an LS to SA5 to ask whether current PCI Optimisation Function is applicable for the scenario when IAB donor CU and mobile IAB-DU connect to different OAMs. SA2 also discussed OAM connectivity issue in the R18 VMR SI/WI and sent an LS to SA3 and SA5 to ask how could the MBSR access the serving PLMN OAM server securely. In moderator’s view, maybe we can wait for the progress in SA2/SA3/SA5.

On the other hand, RAN3 has agreed that F1-terminating donor can reconfigure PCI for the cell of mobile IAB-DU. And RAN3 is discussing whether PCI of mobile IAB cell can be reconfigured by other menas to avoid PCI collision as in section 3.1/4.1. That means even if current PCI Optimisation Function is not applicable for the scenario when IAB donor CU and mobile IAB-DU connect to different OAMs, other means can be used for PCI collision avoidance.

Q8: Do you agree to send an LS to SA5 to ask whether current PCI Optimisation Function is applicable for the scenario when IAB donor CU and mobile IAB-DU connect to different OAMs., if not, please provide your reason/revision.

| Company | Yes/no | Comments |
| --- | --- | --- |
| ZTE | No | The LS seems to be not needed for now. |
| Huawei | No | Not sure why to ask SA5 on PCI collision problem. In addition, the current CU-DU architecture support different OAM also, we see nothing special for mobile IAB. |
| Nokia | No | Same as current system that gNB-DU and gNB-CU may come from different vendor and use different OAM. |
| Xiaomi | Maybe No | Maybe we go to a wrong direction, we think the differences between mobile IAB and legacy system is the frequent and unpredictable PCI collision considering the mobile IAB is moving. |
| Lenovo | No | The LS is not need. Agree with Xiaomi that we may need to do some enhancements for the PCI reconfiguration when IAB-donor-CU and mobile IAB-DU connect to different OAM, especially for frequent and unpredictable PCI collision considering the mobile IAB is moving. |
| Samsung | No | Agree with Nokia. |
| Qualcomm | No | The present PCI coordination function relies on OAM providing a list of PCIs to a gNB, and the gNB can decide which one to pick. This always works. The question is if it resolves PCI collision problems due to IAB-node mobility, which is a complete novelty and has never been addressed by SA5. For that purpose, we need a centralized dynamic inter-OAM coordination function which can obtain measurement reports from all gNB’s and can perform PCI reconfiguration across multi-vendor OAMs on short time scale. RAN3 may want to ask themselves why we would need such a centralized dynamic inter-vendor-OAM coordination function. If this function were available, we wouldn’t need the CU-CP and we wouldn’t RAN3. |
| Ericsson | No | Same view as Nokia. |

Summary

8 companies provided feedback on Q7.

All companies disagree to send an LS to SA5, so there is no proposal here.

References

R3-231359	Discussion on PCI collision avoidance for mobile IAB (ZTE)

R3-231443	PCI collision mitigation of mobile IAB-node mobility (Lenovo)

R3-231473	Mobile IAB interference mitigation (Nokia, Nokia Shanghai Bell)

R3-231485	PCI collision for mobile IAB (Huawei)

R3-231526	Discussion on mitigation of interference (Xiaomi)

R3-231537	PCI Collision Avoidance for Mobile IAB-Nodes (Ericsson)

R3-231720	Discussion on mitigation of interference (Samsung)