3GPP TSG-RAN WG3 #119bis-e 					                                                      R3-232123

17th – 26th April 2023

Online

Agenda item:	17.2

Source:	Nokia, Nokia Shanghai Bell, Ericsson, Samsung

Title:	(TP for BL CR 38.300) data forwarding in HO with time-based trigger condition

Document for:	Approval

# Text Proposal for BL CR 38.300

##### 16.14.3.2.2	Conditional Handover

The same principle as described in 9.2.3.4 applies to NTN unless hereunder specified.

NTN supports the following additional trigger conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]:

-	The RRM measurement-based event A4;

-	A time-based trigger condition;

-	A location-based trigger condition.

A time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12].

It is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event.

When time-based trigger condition is used, the source NG-RAN node should start the data forwarding at the time indicated to the UE.