3GPP TSG-RAN WG3 Meeting #119bis-e                                                               R3-232125

17th – 26th April 2023, Online

Agenda item:	12.2.1

Source:	CMCC, CATT, Nokia, Nokia Shanghai Bell, Lenovo, Ericsson, Huawei, Samsung

Title:	(TP for AI&ML BLCR to TS38.300) Stage 2 updates on the new procedures of AI/ML for RAN

Document for:	Discussion and Decision

1	Introduction

The TP is to capture the agreements on the new procedures of AI/ML for RAN, based on the CB: # AIRAN1_Stage2 [1].

2	TP to 38.300

******************************Start of  Change*****************************************

X.X AI/ML for NG-RAN

X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

Signalling procedures used for the exchange of AI/ML related information are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

For the scenario where AI/ML model training is located in the OAM, the inputs will be sent from NG-RAN directly to OAM (which is in the scope of SA5).

For the Mobility Optimization use case, the AI/ML predictions – e.g., cell-based UE trajectory predictions – are transferred to target gNB via the Handover Request to provide information for subsequent mobility decision.

For the Energy Saving use case, the metric of Energy Cost (EC) is introduced as the AI/ML metric to be shared among gNBs; the EC is a value at gNB level and can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighbour gNB related to either an additional load or the current load.

UE performance feedback reporting is used to reflect the performance of UEs after handover to assist the evaluation of AI/ML related decision. It is configured through AI/ML Information request/response procedure (FFS on the name), while actual reporting is performed through AI/ML Information update procedure (FFS on the name).

Solutions for AI/ML information exchange over the NG interface are not considered in this release of the specification.

******************************End of  Change*****************************************

4	Reference

[1] R3-232122 Summary of CB: # AIRAN1_Stage2