3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231190

# E-meeting,  , 17. - 26.04.2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0711 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of the tabular and ASN.1 definitions for the Resource Status Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The gNB-CU-UP UE ID is mandatory in the RESOURCE STATUS UPDATE message, but optional in the ASN.1. The correct is the tabular definition – for an UE-associated signalling both IDs shall always be included. Also, the criticality is different in ASN.1 and in the tabular, but here the ASN.1 version may be assumed correct. On the other hand, the HW Capacity information is optional in tabular definition of the message, while mandatory in ASN.1 – again, the tabular definition is correct definition considering intented functionality (the HW Capacity can be explicitly requested, so possibly may not be needed to be signalled). |  |  |  |  |  |  |  |  |
| Summary of change: |  | A correction in ASN.1 may be considered non-bakward compatible and it may trigger recording a trasfer syntax error in the case where the presence is changed from ‘mandatory’ to ‘optional’, but since the criticality of this IE is ‘ignore’, it will not fail the procedure. Therefore, it is proposed to correct the error in the ASN.1. Impact assessment towards the previous version of the specification (same release): Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because we modify the presence of IEs with assigned criticality ignore. This CR has an impact under protocol & functional point of view. The impact can be considered isolated because the change affects Resource Status Reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Discrepancy between the tabular and the ASN.1 definitions remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.22, ASN.1 (9.4.4) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CR 0055 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change, ommited text not changed

#### 9.2.1.22	RESOURCE STATUS UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | Ignore |  |
| Transaction ID | M |  | 9.3.1.53 |  | YES | Reject |  |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,…) | Allocated by gNB-CU-CP | YES | Reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | Ignore |
| TNL Available Capacity Indicator | O |  | 9.3.1.72 |  | YES | Ignore |
| HW Capacity Indicator | O |  | 9.3.1.73 |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

Next change, ommited text not changed

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ResourceStatusUpdateIEs } },

...

}

ResourceStatusUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject		TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID		CRITICALITY reject		TYPE INTEGER (1..4095, ...)		PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID		CRITICALITY ignore		TYPE INTEGER (1..4095, ...)		PRESENCE mandatory}|

{ ID id-TNL-AvailableCapacityIndicator		CRITICALITY ignore		TYPE 							TNL-AvailableCapacityIndicator	PRESENCE optional}|

{ ID id-HW-CapacityIndicator				CRITICALITY ignore		TYPE 							HW-CapacityIndicator			PRESENCE optional},

...

}

Remaining text not changed