3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231253

# Online, 17th – 26th April 2023

Agenda Item:	15.3

Source:	Ericsson

Title:	Discussions and proposals concerning Rel-18 work on multicast reception in RRC_INACTIVE

Document for:	Discussions & Approval

# 1	Introduction

The document contains discussions on RAN3 aspects on support of multicast reception by UEs in RRC_INACTIVE state.

It contains review of information received by SA2 in R3-230789 [1] and various attachments [1a]-[1k].

Annex A contains a draft reply LS to SA2 on aspects for multicast reception in RRC_INACTIVE.

Annexes B-E contain Text Proposals for TSs 38.300, 38.401, 38.413 and 38.423 following discussions, mainly on information contained in [1].

# 2	Discussion

## 2.1	Comments on 
CR0159r7 TS 23.247 (S2-2303395) " Support RRC_INACTIVE UE receiving multicast MBS data "
and
CR0168r6 TS 23.247 (S2-2303396) "Support of RRC Inactive state reception for MBS session"

We have discussed at the last meeting and during some offline discussions before the current meeting the principles upon which 5G was designed, especially the fact that the possible RRC states for UEs in CM-CONNECTED (RRC_CONNECTED, RRC_INACTIVE) are supposed to be transparent to the 5GC and under sole responsibility of the NG-RAN.

The 5GC is only indirectly concerned about the RRC state of UEs in CM-CONNECTED when it needs to know whether, and if applicable, when UEs are reachable for reacting sufficiently prompt on requests from the 5GC (for signalling or data provision), however, in principle there is no need for 5GC to know explicitly how RRC states are applied in NG-RAN; and, for sure there is no need to allow 5GC the control of the RRC state.

Therefore, in Rel-18, MBS “assistance” information that is not able to sufficiently abstract RRC state handling (an NG-RAN internal realisation) from traffic/service requirements, and therefore directly prescribes which RRC state to be applied by the NG-RAN, cannot be regarded as following 5G system design principles.

It needs to be reminded that such fundamental change of system design principles would need consensus, and can neither be changed by means of majority only nor can it be changed by a single TSG alone, given the responsibilities for system design matters in various TSGs.

When SA2 started to work on Rel-18 and system aspects of MBS reception in RRC_INACTIVE, there was rather little to do for SA2 and it was rather surprising to see the level of (NG-RAN internal) details being introduced by the various CRs (see applicable attachments to [1]). The only impact identified by SA2 was the provision of “assistance” information in order to assist NG-RAN in determining by which means certain UEs should be served. However, the definition of the “assistance” information turned into a control of the RRC state, which is undisputably not aligned with principles agreed and specified in Rel-15 for 5G.

As outlined in the Annexes below, the following specification text is proposed for (RAN) stage-2 and stage 3:

Stage 2 (38.300):

#### 16.10.5.2	Configuration

A UE can receive data of MBS multicast session in RRC_CONNECTED or RRC_INACTIVE state. If the UE which joined a multicast session is in RRC_CONNECTED state and when the multicast session is activated, the gNB may send RRCReconfiguration message with relevant MBS configuration for the multicast session to the UE.

As specified in TS 23.247 [45], the gNB may receive from the 5GC MBS Assistance Information for a UE, which assists the gNB in configuring the UE properly. The MBS Assistance Information indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception.

Stage 3 (NGAP/XnAP):

The MBS Assistance Information IE indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception, as specified in TS 38.300 [8].

NOTE:	One possible realization may be to keep the UE for which this parameter is received in RRC_CONNECTED when receiving MBS session data.

Observation 1:	The definition of the MBS Assistance Information as per CR159r7 [1c] and CR0168r6 [1d] does actually not “assist” NG-RAN in determining the proper configuration but rather “controls” it – which is not in line with basic system design principles and agreed and specified worksplit between NG-RAN and 5GC. Changing system design principles and worksplit between 5GC and NG-RAN is only possible based on consensus by all responsible TSGs and WGs.

Proposal 2:	SA2 has to be requested to replace text introduced per CR159r7 [1c] and CR0168r6 [1d] regarding the MBS Assistance Information by a definition that abstracts and generalises respective traffic/service characteristics.

## 2.2	Comments on 
CR179r8 TS 23.247 (S2-2303846) "Mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state"

This CR contains system aspects of a UE, which is configured to receive multicast data in RRC_INACTIVE, and which is currently receiving multicast data, moves (a) within its configured RNA and (b) outside its configured RNA.

From a system point of view, the 5GC does normally not notice a UE’s RRC state transition while in CM-CONNECTED. The 5GC may configure NG-RAN to report the UE’s availability for exchange of user data or signalling. However, the level of abstraction/generalisation that is necessary for 5GC to ensure system functions performing properly requires only to know whether (and, if applicable, when) UEs are reachable for reacting sufficiently prompt on requests from the 5GC. SA2 should be liaised to remove those details (not to speak that RAN WGs should first progress on these matters before SA2 continues specification work.)

Observation 3:	CR179r8 (S2-2303846) contains stage-2 description on mobility in RRC_INACTIVE with regards to its configured RNA, which is an NG-RAN internal aspect and to be specified entirely in RAN TSs.

Proposal 4:	We propose to feedback to SA2 to remove details on mobility in RRC_INACTIVE with regards to its configured RNA.

Further, for Rel-17, the extent and the means of how MBS session related information is exchanged between 5GC and gNBs and between gNBs was agreed. For support of mobility, RAN3 distinguished between MBS session information needed at the target (new) gNB at handover preparation (UE context retrieval) during active multicast sessions and MBS session information necessary during inactive MBS sessions.

CR179r8 [1h] contains details of how the MBS Assistance Information as part of UE Context transfer/establishment during handover and UE Context retrieval is provided to the target/new gNB. It is expected that the MBS Assistance Information is useful at the target/new gNB irrespective the MBS session state and may influence the gNB’s decision whether to send joined UEs to RRC_INACTIVE at all.

Observation 5:	MBS Assistance Information may be useful at mobility towards a target/new gNB irrespective the MBS session state. It should be provided via Xn signalling during UE context transfer to the target gNB (Xn handover) or to the new gNB (RRC connection resume). At NG/N2 handover, as for PDU Session context setup, it is expected to be provided by the SMF to the target gNB.

Proposal 6:	We propose to feedback to SA2 that RAN3 foresees MBS Assistance Information to be provided at mobility to the target gNB (Xn handover) or to the new gNB (RRC connection resume) by means of Xn signalling irrespective the MBS session state. At NG/N2 based handover MBS Assistance Information is expected to be provided by the SMF to the target gNB.

Rel-17 specifies interaction with non-supporting gNBs by means of “individual” multicast data delivery methods. This includes communication between NG-RAN and 5GC on whether PDU Session context information was stored within the UE Context in NG-RAN and whether the gNB supports MBS at all.

An additional level of MBS support is introduced in Rel-18, as gNBs supporting MBS do not necessarily support MBS reception in RRC_INACTIVE as well. We would not expect any explicit signalling towards the 5GC distinguishing these different levels of MBS support, mainly, because control of RRC_INACTIVE should be regarded transparent to the 5GC.

Proposal 7:	We propose to feedback to SA2 that RAN3 does not see the necessity of explicitly informing the 5GC about the NG-RAN node’s support of MBS reception in RRC_INACTIVE.

## 2.3	Comments on current status of discussions in RAN2

RAN2 made some progress at RAN2#121 in the February/March 2023 meeting. In particular the following agreements is notable from RAN3 point of view:

Serving cell will not provide the PTM configuration of neighbour cells from other gNBs.

FFS whether the network can provide PTM configuration for intra-gNB cells.

This has to be read together with another agreements

If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured.

When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration. Other dedicated RRC messages will not be used to provide PTM configuration for MBS multicast for INACTIVE.

Along latest agreements in RAN2 one can deduce that there is (exactly) one dedicated RRC signalling means to configure the UE with the PTM configuration to be applied in RRC_INACTIVE, which is the RRCRelease message. It also appears as if updating of that configuration by means of MCCH is possible. Further, the UE will not receive PTM configuration of neighbour cells from other gNBs (neither via dedicated signalling nor via MCCH). Provision of intra-gNB neighbour cells’ PTM configurations is FFS.

Proposal 8: 	Acknowledge that, given current RAN2 agreements, it can be deduced that Xn signalling for exchange of neighbour cells’ PTM configuration is not necessary.

During the past meetings we have presented the observation that the approach to allow UEs to receive multicast data in RRC_INACTIVE stems from scenarios where the number of UEs participating in a multicast group exceed the number of UEs able to be served within a cell in RRC_CONNECTED. It can be deduced that scenarios where cells serve only UEs in RRC_INACTIVE are not possible, or, at least, protocol options should not be developed to optimally support those scenarios. This also means that in Rel-18, still, the preferred RRC state a UE should be in is RRC_CONNECTED.

Proposal 9: 	Acknowledge that the main uses case targeted in Rel-18 for support of multicast data reception in RRC_INACTIVE are scenarios where the number of served multicast group members exceed a cell’s capacity serving UEs in RRC_CONNECTED. As long as the number of served group members in a cell are not near that limit, these UEs are served in RRC_CONNECTED.

Along the current status of RAN2 discussions it is uncertain whether the PTM configuration to be used for multicast reception in RRC_INACTIVE may contain PTM configuration of neighbouring intra-gNB cells, i.e. contain more than one PTM configuration. Given that speculations are allowed that such will be not be agreed in RAN2, UEs would need to be configured by the network upon mobility to a neighbouring cell. This configuration may be provided via dedicated signalling, for which the UE would need to return to RRC_CONNECTED, which we would prefer, i.e. mobility in typical cases is performed in RRC_CONNECTED, or via MCCH after cell-reselection. The latter possibility is probably necessary for very extreme congestion scenarios where not even connected mobility is possible to be provide to all UEs. However, the support of such scheme is not yet fully agreed in RAN2 and needs further input for further discussions in RAN3.

Observation 10: 	Mobility schemes for support of multicast reception in RRC_INACTIVE still need input from RAN2.

On F1 aspects, one important input from RAN2 would be whether PTM resources configured for receptions in RRC_INACTIVE may be different PTM resources configured for reception in RRC_CONNECTED. Configuration of MBS session resources and the applicability / extendibility of protocol means introduced in Rel-17 is dependent on respective RAN2 decisions.

What can be deduced from RAN2 decisions is that an MBS Session Context may exist in the gNB-DU for inactive multicast MBS Sessions, as RAN2 concluded that the the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration.

Further questions to be answered would be: should a UE context be kept in gNB-DU for UEs released with ptm configuration? By which means should the gNB-CU retrieve the PTM configuration from the gNB-DU before releasing the UE to INACTIVE? But all that can only be actually further progressed if RAN2 made progress on more fundamental questions.

Observation 11: 	Detailled protocol work for F1 needs more progress from RAN2.

# 3	Conclusion and Proposals

We have been discussing information received from SA2 on current status of Rel-18 discussions on multicast reception in RRC_INACTIVE with the following observations and proposals:

Observation 1:	The definition of the MBS Assistance Information as per CR159r7 [1c] and CR0168r6 [1d] does actually not “assist” NG-RAN in determining the proper configuration but rather “controls” it – which is not in line with basic system design principles and agreed and specified worksplit between NG-RAN and 5GC. Changing system design principles and worksplit between 5GC and NG-RAN is only possible based on consensus by all responsible TSGs and WGs.

Proposal 2:	SA2 has to be requested to replace text introduced per CR159r7 [1c] and CR0168r6 [1d] regarding the MBS Assistance Information by a definition that abstracts and generalises respective traffic/service characteristics.

Observation 3:	CR179r8 (S2-2303846) contains stage-2 description on mobility in RRC_INACTIVE with regards to its configured RNA, which is an NG-RAN internal aspect and to be specified entirely in RAN TSs.

Proposal 4:	We propose to feedback to SA2 to remove details on mobility in RRC_INACTIVE with regards to its configured RNA.

Observation 5:	MBS Assistance Information may be useful at mobility towards a target/new gNB irrespective the MBS session state. It should be provided via Xn signalling during UE context transfer to the target gNB (Xn handover) or to the new gNB (RRC connection resume). At NG/N2 handover, as for PDU Session context setup, it is expected to be provided by the SMF to the target gNB.

Proposal 6:	We propose to feedback to SA2 that RAN3 foresees MBS Assistance Information to be provided at mobility to the target gNB (Xn handover) or to the new gNB (RRC connection resume) by means of Xn signalling irrespective the MBS session state. At NG/N2 based handover MBS Assistance Information is expected to be provided by the SMF to the target gNB.

Proposal 7:	We propose to feedback to SA2 that RAN3 does not see the necessity of explicitly informing the 5GC about the NG-RAN node’s support of MBS reception in RRC_INACTIVE.

Final proposal:	It is proposed to reply to SA2 on aspects for multicast reception in RRC_INACTIVE as shown in Annex A, as well as to agree on TPs outlined in Annexes B-E.

The following observations and proposals have been developed for NG-RAN internal work, focussing on recent RAN2 agreements:

Proposal 8: 	Acknowledge that, given current RAN2 agreements, it can be deduced that Xn signalling for exchange of neighbour cells’ PTM configuration is not necessary.

Proposal 9: 	Acknowledge that the main uses case targeted in Rel-18 for support of multicast data reception in RRC_INACTIVE are scenarios where the number of served multicast group members exceed a cell’s capacity serving UEs in RRC_CONNECTED. As long as the number of served group members in a cell are not near that limit, these UEs are served in RRC_CONNECTED.

Observation 10: 	Mobility schemes for support of multicast reception in RRC_INACTIVE still need input from RAN2.

Observation 11: 	Detailled protocol work for F1 needs more progress from RAN2.

# 4	References

[1]	R3-230789 "LS on the open issues related to RAN WGs in 5MBS_Ph2", LS in from SA2#155, received at RAN3#119 with the following attachments:

[1a]	S2-2301598 "Coexistence with existing power saving mechanisms for capability-limited devices" SA2 agreed CR0162 TS 23.247, attached to LS in [1]

[1b]	S2-2301599 "Support MBS for UEs using power saving functions" SA2 agreed CR TS 23.247, attached to LS in [1]

[1c]	S2-2303395 "Support RRC_INACTIVE UE receiving multicast MBS data" SA2 agreed CR0159r7 TS 23.247, attached to LS in [1]

[1d]	S2-2303396 "Support of RRC Inactive state reception for MBS session" SA2 agreed CR0168r6 TS 23.247, attached to LS in [1]

[1e]	S2-2303397 "KI#1, adding RRC inactive network functionality of NF description" SA2 agreed CR0171r5 TS 23.247, attached to LS in [1]

[1f]	S2-2303401 "Introducing functionality of resource efficiency in MOCN Network Sharing" SA2 agreed CR0160r6 TS 23.247, attached to LS in [1]

[1g]	S2-2303402 "On resource efficiency for MBS reception in RAN sharing scenario" SA2 agreed CR0165r3 TS 23.247, attached to LS in [1]

[1h]	S2-2303846 "Mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state" SA2 agreed CR0179r8 TS 23.247, attached to LS in [1]

[1i]	S2-2303871 "On Clarifying the Scenario Considering the Power Saving Mechanism" SA2 agreed CR0164r6 TS 23.247, attached to LS in [1]

[1j]	S2-2303876 "Support of MBS multicast reception by UEs in RRC_INACTIVE state" SA2 agreed CR0149r8 TS 23.247, attached to LS in [1]

[1k]	S2-2303897 "KI#2, adding MOCN network sharing for the same content" SA2 agreed CR0176r9 TS 23.247, attached to LS in [1]

[2]	TR 23.700-47 "Study on architectural enhancements for 5G multicast-broadcast services; Phase 2"

# Annex A	Draft reply LS to SA2 on aspects regarding multicast reception in RRC_INACTIVE

3GPP TSG-RAN WG3 Meeting #119bis-e	R3-23xxxx

Online, 17th – 26th April 2023

Title:	[DRAFT] Reply LS on the open issues related to RAN WGs in 5MBS_Ph2

Response to:	LS on the open issues related to RAN WGs in 5MBS_Ph2 from SA2 (S2-2303407/R3-230789)

Release:	REL-18

Work Item:	NR_MBS_enh-Core, 5MBS_Ph2

Source:	Contact Company (will be RAN3)

To:	SA2, RAN2, SA, RAN

Cc:

Contact Person:

#### Name:

Tel. Number:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA2 for their LS on the open issues related to RAN WGs in 5MBS_Ph2 from SA2 which was received in S2-2303407/R3-230789 containing numerous attachments which indeed have impact on NG-RAN functionality.

RAN3 would like to provide the following feedback:

CR0159r7, CR0168r6, CR0171r5, CR0179r8 and CR0149r8 on multicast reception in RRC_INACTIVE including the introduction of MBS Assistance Information:

-	There is no consensus in RAN3 to modify currently existing basic system design principles which do not allow the 5GC to influence the NG-RAN’s decisions regarding RRC state handling directly. Hence, it is only permissible to define the MBS Assistance Information, as provided by 5GC, in a way that generalises and abstracts the service and traffic characteristics.

-	RAN3 requests SA2 to replace current definition of the MBS Assistance Information as shown in the relevant RAN3 CRs by the following (quote from Baseline CR for TS 38.300):

As specified in TS 23.247 [45], the gNB may receive from the 5GC MBS Assistance Information for a UE, which assists the gNB in configuring the UE properly. The MBS Assistance Information indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception.

-	RAN3 would also like to remind SA2 that RRC state handling in CM-CONNECTED is in principle transparent to 5GC. The level of abstraction/generalisation that is necessary for 5GC to ensure system functions performing properly requires the 5GC only to know whether (and, if applicable, when) UEs are reachable for reacting sufficiently prompt on requests from the 5GC. RAN3 therefore kindly asks SA2 to remove all related NG-RAN-internal details from specification text introduced by CR0149r8, CR0171r5 and CR0179r8.

-	On the Editor’s Note regarding MBS Assistance Information transfer during mobility in CR0179r8: RAN3 foresees MBS Assistance Information to be provided at mobility to the target gNB (Xn handover) or to the new gNB (RRC connection resume) by means of Xn signalling – irrespective the MBS session state. At NG/N2 based handover MBS Assistance Information is expected to be provided by the SMF to the target gNB (in the same way as at PDU Session Context setup).

-	Further, RAN3 does not see the necessity of explicitly informing the 5GC about the NG-RAN node’s support of MBS reception in RRC_INACTIVE in addition to the already existing “general” MBS support indication.

2. Actions:

To SA2 and RAN2 group:

ACTION: 	RAN3 would like to ask SA2 to respect the work-split among TSG SA and TSG RAN and update their specifications accordingly

To TSG SA and TSG RAN:

ACTION: 	RAN3 would like to ask TSG SA and TSG RAN to coordinate Work Items with obvious and large overlapping objectives in a way that co-operation among the relevant WGs is possible – in many cases this has involved SA2, RAN2 and RAN3. It is requested that the timelines of Work Items are overlapping sufficiently.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting#120	22nd – 26th May 2023, Incheon, Korea

TSG-RAN WG3 Meeting#121	21st – 25th August 2023, Toulouse, Europe

TSG-RAN WG3 Meeting#121bis	9th – 18th October 2023, online

TSG-RAN WG3 Meeting#122	13th – 17th November 2023, Chicago, US

# Annex B	Text Proposal TS 38.000

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 16.10.5.2	Configuration

A UE can receive data of MBS multicast session in RRC_CONNECTED or RRC_INACTIVE state. If the UE which joined a multicast session is in RRC_CONNECTED state and when the multicast session is activated, the gNB may send RRCReconfiguration message with relevant MBS configuration for the multicast session to the UE.

As specified in TS 23.247 [45], the gNB may receive from the 5GC MBS Assistance Information for a UE, which assists the gNB in configuring the UE properly. The MBS Assistance Information indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 16.10.5.4	Reception of MBS Multicast data

##### 16.10.5.4.1	Reception of MBS Multicast data in RRC_CONNECTED

For multicast service, gNB may deliver Multicast MBS data packets to UEs in RRC_CONNECTED using the following methods:

-	PTP Transmission: gNB individually delivers separate copies of MBS data packets to each UEs independently, i.e., gNB uses UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

-	PTM Transmission: gNB delivers a single copy of MBS data packets to a set of UEs, e.g., gNB uses group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI.

If a UE is configured with both PTM and PTP transmissions, a gNB dynamically decides whether to deliver multicast data by PTM leg and/or PTP leg for a given UE based on the protocol stack defined in clause 16.10.3, based on information such as MBS Session QoS requirements, number of joined UEs, UE individual feedback on reception quality, and other criteria. The same QoS requirements apply regardless of the decision.

##### 16.10.5.4.2	Reception of MBS Multicast data in RRC_INACTIVE

Editor’s Note:	content to be added

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex C	Text Proposal TS 38.401

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 7.7	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

### 7.7.1	Support of dynamic PTP and PTM switching

For UEs in RRC_CONNECTED, NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for a MRB with common PDCP involving both PTP (RLC leg) and PTM (RLC leg), upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg) or both.

For UEs in RRC_INACTIVE only PTM is upported as specified in TS 38.300 [2].

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex D	Text Proposal TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	MBS Assistance Information

The MBS Assistance Information IE indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception, as specified in TS 38.300 [8].

NOTE:	One possible realization may be to keep the UE for which this parameter is received in RRC_CONNECTED when receiving MBS session data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Assistance Information | M |  | ENUMERATED (preferential treatment, ...) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.211	MBS Session Setup Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup Request List |  | 1 |  |  | - |  |
| >MBS Session Setup Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS Assistance Information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

#### 9.3.1.212	MBS Session Setup or Modify Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup or Modify Request List |  | 1 |  |  | - |  |
| >MBS Session Setup or Modify Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup or Modify Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup or Modify Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS QoS Flow To Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows to be released. | - |  |
| >>MBS Assistance Information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-AssistanceInformation ::= ENUMERATED {preferential-treatment, ...}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBSSessionSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetupRequestItem

MBSSessionSetupRequestItem ::= SEQUENCE {

mBS-SessionID									MBS-SessionID,

mBS-AreaSessionID								MBS-AreaSessionID																	OPTIONAL,

associatedMBSQosFlowSetupRequestList			AssociatedMBSQosFlowSetupRequestList												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { MBSSessionSetupRequestItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation			CRITICALITY ignore	TYPE MBS-AssistanceInformation		PRESENCE	optional		},

...

}

MBSSessionSetuporModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetuporModifyRequestItem

MBSSessionSetuporModifyRequestItem ::= SEQUENCE {

mBS-SessionID										MBS-SessionID,

mBS-AreaSessionID									MBS-AreaSessionID									OPTIONAL,

associatedMBSQosFlowSetuporModifyRequestList		AssociatedMBSQosFlowSetuporModifyRequestList		OPTIONAL,

mBS-QosFlowToReleaseList 							QosFlowListWithCause								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{MBSSessionSetuporModifyRequestItem-ExtIEs}}			OPTIONAL,

...

}

MBSSessionSetuporModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation			CRITICALITY ignore	TYPE MBS-AssistanceInformation		PRESENCE	optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-MBS-AssistanceInformation							ProtocolIE-ID ::= 999 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex E	Text Proposal TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	MBS Assistance Information

The MBS Assistance Information IE indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception, as specified in TS 38.300 [9].

NOTE:	One possible realization may be to keep the UE for which this parameter is received in RRC_CONNECTED when receiving MBS session data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Assistance Information | M |  | ENUMERATED (preferential treatment, ...) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.36	MBS Session Information List

This IE contains NG-RAN MBS session resource context related information used at UE context transfer between NG-RAN nodes.

| IE/Group Name | Presence | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MBS Session Information Item |  | 1..<maxnoofMBSSessions> |  |  |  |  |  | – |  |  |  |
| >MBS Session ID | M |  |  | 9.2.3.146 |  |  |  | – |  |  |  |
| >MBS Area Session ID | O |  |  | 9.2.3.148 |  | MBS Area Session ID of the UE at the NG-RAN node from which the UE context is transferred |  | – |  |  |  |
| >Active MBS Session Information | O |  |  |  |  |  |  | – |  |  |  |
| >>MBS QoS Flows to Add List |  | 1..<maxnoofMBSQoSFlows> |  |  |  |  |  | – |  |  |  |
| >>>MBS QoS Flow Identifier | M |  |  | QoS Flow Identifier 9.2.3.10 |  |  |  | – |  |  |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  |  | QoS Flow Level QoS Parameters 9.2.3.5 |  |  |  | – |  |  |  |
| >>MBS Service Area | O |  |  | 9.2.3.150 |  |  |  | – |  |  |  |
| >>MBS Mapping and Data Forwarding Request Info from source NG-RAN node | O |  |  | 9.2.1.39 |  |  |  | – |  |  |  |
| >MBS Assistance Information | O |  | 9.2.3.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions. Value is 256. |
| maxnoofMBSQoSFlows | Maximum no. of QoS flows allowed within one MBS session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-MBS-AssistanceInformation,

maxEARFCN,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-AssistanceInformation ::= ENUMERATED {preferential-treatment, ...}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-SessionInformation-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformation-Item

MBS-SessionInformation-Item ::= SEQUENCE {

mBS-Session-ID			MBS-Session-ID,

mBS-Area-Session-ID		MBS-Area-Session-ID														OPTIONAL,

active-MBS-SessioInformation			Active-MBS-SessionInformation							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-SessionInformation-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-SessionInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation			CRITICALITY ignore	TYPE MBS-AssistanceInformation		PRESENCE	optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-MBS-AssistanceInformation																		ProtocolIE-ID ::= 999 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>