3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231366

Electronic meeting, 17 Apr – 26 Apr, 2023

Title: 	Correction of SIType List

Source: 	Huawei, CATT, Nokia, Nokia Shanghai Bell, Samsung

Agenda item:	9.2.4

Document Type:	Discussion

# 1. Introduction

The CR on the SI Type List for on-demand other SI proposed in [1,2] was discussed during the previous RAN3-119 meeting but without conclusion. In this document we elaborate the details, and propose to agree the corresponding CRs.

# 2. Background

On the on-demand request of other SI broadcast, the following is described in TS 38.300, and excerpted for reference.

### 7.3.1 Overview <skip the irrelevant> - Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED (i.e., upon request, if configured by the network, from UEs in RRC_CONNECTED or when the UE has an active BWP with no common search space configured or when the UE configured with inter cell beam management is receiving DL-SCH from a TRP with PCI different from serving cell's PCI). Other SI consists of: <skip the irrelevant> ### 7.3.2 Scheduling <skip the irrelevant> For UEs in RRC_IDLE and RRC_INACTIVE while SDT procedure is not ongoing (see clause 18), a request for Other SI triggers a random access procedure (see clause 9.2.6) where MSG3 includes the SI request message unless the requested SI is associated to a subset of the PRACH resources, in which case MSG1 is used for indication of the requested Other SI. When MSG1 is used, the minimum granularity of the request is one SI message (i.e. a set of SIBs), one RACH preamble and/or PRACH resource can be used to request multiple SI messages and the gNB acknowledges the request in MSG2. When MSG 3 is used, the gNB acknowledges the request in MSG4. For UEs in RRC_CONNECTED, a request for Other SI may be sent to the network, if configured by the network, in a dedicated manner (i.e., via UL-DCCH) and the granularity of the request is one SIB. The gNB may respond with an RRCReconfiguration including the requested SIB(s). It is a network choice to decide which requested SIBs are delivered in a dedicated or broadcasted manner. The Other SI may be broadcast at a configurable periodicity and for a certain duration. The Other SI may also be broadcast when it is requested by UE in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED.

From the above descriptions, it can be observed that:

- the other SI will not include the MIB and SIB1;

- all UEs (in idle, inactive, connected state) can request the other SI, and the network can broadcast the requested other SI.

- for UEs in RRC_IDLE and RRC_INACTIVE, Msg1 based, or Msg3 based other SI request is supported. The minimum granularity of the request is one SI message.

- For UEs in Reconnected, the dedicated request is supported, and the granularity of the request is one SIB.

Currently over the F1 interface, the SYSTEM INFORMATION DELIVERY COMMAND message was introduced to support the on-demand other SI broadcast. This procedure can be used for Msg3 based other SI request for RRC_IDLE, RRC_INACTIVE UE, and dedicated request for RRC-CONNECTED UE.

As excerpted follows, the semantics descriptions indicate that the SI type value starts from 2 from the CU to the DU.

#### 9.3.1.62	SIType List

This IE is used by gNB-CU to provide SI list of other SI for gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SI type item IEs |  | 1.. <maxnoofSITypes> |  |  |
| >SI Type | M |  | INTEGER (1..32, ...) | Indicates a certain SI type required to be broadcasted by the gNB-DU. The SI Type value of other SI starts from 2 |

# 3. Discussion

On-demand other SI request for idle/inactive UE

According to TS 38.331 below, for the msg3 based other SI request, the idle or inactive UE can request 32-bit bit string in the RRCSystemInfoRequest message, where each bit corresponds to one item in the schedulingInfoList IE or the schedulingInfoList2 IE (for Rel-17) in SIB1 as follows.

RRCSystemInfoRequest-IEs ::=    SEQUENCE {

requested-SI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                               BIT STRING (SIZE (12))

}

requested-SI-List Contains a list of requested SI messages which are configured by schedulingInfoList in si-SchedulingInfo and schedulingInfoList2 in si-SchedulingInfo-v1700 (if present) in SIB1. If si-SchedulingInfo-v1700 is not present: - According to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on. If si-SchedulingInfo-v1700 is present: - The UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. - According to the order of entry in the list of concatenated SI messages, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on.

It can be observed for the other SI on-demand broadcast, when the UE requests the first SI, the network should broadcast the first SI configured by schedulingInfoList IE. When the UE requests the second SI, the network should broadcast the second SI configured in the schedulingInfoList IE and the schedulingInfoList2 IE (for Rel-17) if exist.

- Observation: for the other SI on-demand broadcast for idle/inactive UE, when the UE requests the first SI, the network should broadcast the first SI configured by schedulingInfoList IE. When the UE requests the second SI, the network should broadcast the second SI configured in the schedulingInfoList IE (or si-SchedulingInfo-v1700), and so on.

But with the above semantics for SI type IE in the SYSTEM INFORMATION DELIVERY COMMAND message,

- the CU cannot indicate the first SI to the DU, such that the DU cannot broadcast the first SI when the UE requests the first SI in the other SI.

- in case the UE requests overall 32 SIs, the existing SIType List can only indicate 31 SIs to the gNB-DU.

In addition, it can be noticed that for the PosSI Type IE included in the PosSIType List in the POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message, there is no such issue, since it is clearly described in the semantic descriptions that “The SI Type value of positioning SI starts from 1”.

For idle/inactive other SI on-demand request, remove the “The SI Type value of other SI starts from 2” from the semantic descriptions for the SI Type IE in the SYSTEM INFORMATION DELIVERY COMMAND message.

On-demand Other SI request for connected UE

Also, as indicated above, the connected UE can send the dedicated RRC message for other SI broadcast. According to TS 38.331 below, the connected UE can request the SIB via RRC DedicatedSIBRequest message in the SIB granularity, e.g., SIB12, SIB13 etc.

DedicatedSIBRequest-r16-IEs ::=  SEQUENCE {

onDemandSIB-RequestList-r16       SEQUENCE {

requestedSIB-List-r16            SEQUENCE (SIZE (1..maxOnDemandSIB-r16)) OF SIB-ReqInfo-r16           	    OPTIONAL,

requestedPosSIB-List-r16         SEQUENCE (SIZE (1..maxOnDemandPosSIB-r16)) OF PosSIB-ReqInfo-r16          OPTIONAL

} OPTIONAL,

lateNonCriticalExtension         OCTET STRING             OPTIONAL,

nonCriticalExtension             SEQUENCE {}              OPTIONAL

}

SIB-ReqInfo-r16 ::=                   ENUMERATED { sib12, sib13, sib14, sib20-v1700, sib21-v1700, spare3, spare2, spare1 }

When the CU receives the DedicatedSIBRequest message containing the requested SIB list, there are two options for the CU to deliver the requested information to the DU for broadcast.

- Option 1: the SI type list

- Option 2: the SIB list

For option 1, it requires the CU to translate the requested SIB list into the SI list based on the si-SchedulingInfo IE or the si-SchedulingInfo-v1700 (if present) in the SIB1 signalled on the F1AP interface messages, without specification change. While for other option 2, the CU can simply deliver the requested SIB list, then the DU decides which SI(s) should be broadcast. We consider both options could work, and RAN3 can decide which option can be selected.

For connected UE other SI on-demand request, RAN3 to discuss whether the SI type list or the SIB list is signalled to the DU.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

- Observation: for the other SI on-demand broadcast for idle/inactive UE, when the UE requests the first SI, the network should broadcast the first SI configured by schedulingInfoList IE. When the UE requests the second SI, the network should broadcast the second SI configured in the schedulingInfoList IE (or si-SchedulingInfo-v1700), and so on.

- For idle/inactive other SI on-demand request, remove the “The SI Type value of other SI starts from 2” from the semantic descriptions for the SI Type IE in the SYSTEM INFORMATION DELIVERY COMMAND message.

For connected UE other SI on-demand request, RAN3 to discuss whether the SI type list or the SIB list is signalled to the DU.

- The revised CRs can be found in [3] and [4].

# 5. Reference

- R3-230666 Correction of SIType List Huawei

- R3-230667 Correction of SIType List Huawei

- R3-231367	Correction of SIType List	Huawei, CATT, Nokia, Nokia Shanghai Bell

- R3-231368	Correction of SIType List	Huawei, CATT, Nokia, Nokia Shanghai Bell