3GPP TSG-RAN WG3#119bis-e	R3-231380

online, 17th – 26th April, 2023

Agenda item:		9.2.4 Others

Title:		Format of Routing ID in NGAP: NRPPa Transport Messages

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The Routing ID IE related was discussed in;

| R3-230738 | Clarification on the maximum length of the Routing ID | NTT DOCOMO, INC. | Tianyang Min | discussion |
| --- | --- | --- | --- | --- |
| R3-230745 | Clarification on maximum length of Routing ID | NTT DOCOMO, INC. | Tianyang Min | CR |
| R3-230748 | Clarification on maximum length of Routing ID | NTT DOCOMO, INC. | Tianyang Min | CR |
| R3-230751 | Clarification on the maximum length of Routing ID | NTT DOCOMO, INC. | Tianyang Min | LS out |

And agreed CRs in RAN3#119 (Feb 2023):

| R3-230948 | Clarification on maximum length of Routing ID | Rel-16 | 38.413 | NR_pos-Core |
| --- | --- | --- | --- | --- |
| R3-230949 | Clarification on maximum length of Routing ID | Rel-17 | 38.413 | NR_pos-Core |

But then were postponed in RANP#99 (Mar 2023).

We discuss and reconsider the necessity of the CR that intend to clarify.

# 2. Discussion

The Routing ID IE is specified in the NRPPa Transport Messages:

- DOWNLINK UE ASSOCIATED NRPPA TRANSPORT

- UPLINK UE ASSOCIATED NRPPA TRANSPORT

- DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT

- UPLINK NON UE ASSOCIATED NRPPA TRANSPORT

The RAN3#119 agreed CR in R3-230948 and R3-230949 were postponed in later RANP#99.

Check again if the limitation that was added in R3-230948 will be consistency with 29.571, where the CR tried to clarify that

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Routing ID | M |  | OCTET STRING | The maximum length is 16 octets, corresponding to the binary form of NfInstanceId defined in TS 29.571 [35] |

while the 29.571 the NFinstanceId, it is specified as

| NfInstanceId | string | String uniquely identifying a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [15]. (NOTE 3) |
| --- | --- | --- |

Where the NOTE 3 said:

NOTE 1: The encoding of 3GPP defined identifiers (e.g. IMSI, NAI, IMEI, GCI, GLI) shall be prefixed with its corresponding prefix (e.g. 'imsi-','nai-', 'imei-', 'gci-', 'gli-'). NOTE 2: Whether the Dnn data type contains just the DNN Network Identifier, or the Network Identifier plus the Operator Identifier, shall be documented in each API where this data type is used. NOTE 3: NFs shall be able to receive a NF Instance Id in any UUID format.

Observation: It is clear that the intended clarification in CR (R3-230948) for the Routing ID IE (OCTET STRING) to limit to the binary form, is not consistant with 29.571.

This Routing ID IE is defined in NGAP as OCTET STRING, it is transparent to the RAN i.e. the RAN does not need to dig into the detail how it would be encoded, but just transparently send back to the AMF in the OCTET STRING form. The coding form and its length is already specified in 29.571, we reconsider that there is no need to specify in NGAP what its form and its length as it does not mean anything to RAN.

Proposal: it is proposed not to give any more clarification the Routing ID IE, the current semantic description that refer to 29.571 is enough.

# 3. Conclusion

Observation: It is clear that the intended clarification in CR (R3-230948) for the Routing ID IE (OCTET STRING) to limit to the binary form, is not consistant with 29.571.

Proposal: it is proposed not to give any more clarification the Routing ID IE, the current semantic description that refer to 29.571 is enough.

# References

[1] R3-230738 Clarification on maximum length of Routing ID (discussion paper)

[2] R3-230948 Clarification on maximum length of Routing ID (CR)

[2] TS23.571 v17.8.0