3GPP TSG-RAN WG3 #119bis-e	R3-231469

Online, 17th – 26th April 2023

Source:	CATT

Title:	(TP for 38.423) Updates on non-UE associated messages to support AI/ML

Agenda Item:	12.2.2.1

Document for:	Discussion and decision

- Introduction

This is a TP based on the discussion paper R3-231465, R3-231466, and R3-231467.

- TP for TS 38.423

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

**************** skip unchanged part *******************

| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| --- | --- | --- | --- |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI/ML Information Reporting Initiation (FFS on the name) | AI/ML INFORMATION REQUEST (FFS on the name) | AI/ML INFORMATION RESPONSE (FFS on the name) | AI/ML INFORMATION FAILURE (FFS on the name) |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| AI/ML Information Reporting (FFS on the name) | AI/ML INFORMATION UPDATE (FFS on the name) |

**************** skip unchanged part *******************

8.4.AA AI/ML Information Reporting Initiation (FFS on the name)

8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to "start"; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	FFS

If the Registration Request IE is set to "start" in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide any of requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message. For each type of requested information NG-RAN node2 incapable to provide, NG-RAN node2 may indicate the cause of its failure in the Failed Reporting Characteristics List IE.

If the Time Window Length IE in the AI/ML INFORMATION REQUEST is present, this indicates the time window length for the reporting of AI/ML related information.

8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If none of the requested AI/ML related information reporting can be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value. For each type of requested information NG-RAN node2 incapable to provide, NG-RAN node2 may indicate the cause of its failure in the Failed Reporting Characteristics List IE.

8.4.AA.4	Abnormal Conditions

FFS

8.4.BB	AI/ML Information Reporting (FFS on the name)

8.4.BB.1	General

This procedure is initiated by an NG-RAN node to report AI/ML related information accepted by the NG-RAN node following a successful AI/ML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be reported using this procedure.

Editor’s Note: FFS content of AL/ML related information.

8.4.BB.2	Successful Operation

Figure 8.4.11.2-1: AI/ML Information Reporting, successful operation

NG-RAN node2 shall report the accepted AI/ML related information in AI/ML INFORMATION UPDATE message. The accepted AI/ML related information is the information that was successfully initiated during the preceding AI/ML Information Reporting Initiation procedure.

8.4.BB.3	Unsuccessful Operation

Not applicable.

8.4.BB.4	Abnormal Conditions

Void

9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Forth Bit = Average Packet Delay, Fifth Bit = Average UE Throughput DL, Sixth Bit = Average UE Throughput UL, Seventh Bit = Average Packet Error Rate Eighth Bit = Estimated Actual Delta Energy Consumption FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Time Window Length | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Also used as the reporting periodicity for all periodical reporting. | YES | ignore |
| Number of Predicted Durations per Report | O |  | [FFS] ENUMERATED(one, two, five, ...) |  | YES | ignore |
| Time Offset Between Windows | O |  | [FFS] ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Present if the bit for Estimated Actual Delta Energy Consumption in the Report Characteristics IE is set to “1”. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.3.DD	AI/ML INFORMATION RESPONSE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or part of (exact details of if and how to support partial reporting are FFS)  the objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Reporting Characteristics (FFS) | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is able to report. FFS on the coding | YES | reject |
| Failed Reporting Characteristics List |  | 0..1 |  |  | YES | reject |
| >Failed Reporting Characteristics Item |  | 1 .. <maxnoofFailedReportingCharacteristics> |  |  | - |  |
| >>Failed Reporting Characteristics | M |  | BITSTRING (SIZE(32)) | Each “1” in the bitmap indicates the object the NG-RAN node2 is unable to report. | - |  |
| >>Cause | M |  | 9.2.3.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofFailedReportingCharacteristics | Maximum no. of failed reporting characteristics in a response message. Number is FFS. |

9.1.3.EE	AI/ML INFORMATION FAILURE (FFS on the name)

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of (exact details of if and how to support partial reporting are FFS) the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Failed Reporting Characteristics List |  | 0..1 |  |  | YES | reject |
| >Failed Reporting Characteristics Item |  | 1 .. <maxnoofFailedReportingCharacteristics> |  |  | - |  |
| >>Failed Reporting Characteristics | M |  | BITSTRING (SIZE(32)) | Each “1” in the bitmap indicates the object the NG-RAN node2 is unable to report. | - |  |
| >>Cause | M |  | 9.2.3.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofFailedReportingCharacteristics | Maximum no. of failed reporting characteristics in a response message. Number is FFS. |

9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result (FFS on the name) |  | 0..1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item (FFS on the name) |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Duration List |  | 0..1 |  |  | – |  |
| >>>Predicted Duration Item |  | 1 .. <maxnoofPredictedDurations> |  | Let T be the time window length while t₀ be the present time. The first item corresponds to the duration t₀ to t₀+T, the second item corresponds to the duration t₀+T to t₀+2T, and so on. | – |  |
| >>>>Predicted Radio Resource Status | O |  |  |  | – |  |
| >>>>>Value | M |  | 9.2.2.50 |  | – |  |
| >>>>>Standard Uncertainty | O |  | 9.2.2.50 |  | – |  |
| >>>>Predicted Number of Active UEs per Cell | O |  |  |  | – |  |
| >>>>>Value | M |  | 9.2.2.62 |  | – |  |
| >>>>>Standard Uncertainty | O |  | 9.2.2.62 |  | – |  |
| >>>>Predicted RRC Connections per Cell | O |  |  |  | – |  |
| >>>>>Value | M |  | 9.2.2.56 |  | – |  |
| >>>>>Standard Uncertainty | O |  | 9.2.2.56 |  | – |  |
| >>FFS |  |  |  |  |  |  |
| Estimated Actual Delta Energy Consumption | O |  | Energy 9.2.3.Z |  | YES | ignore |
| UE Feedback | O |  | 9.2.3.Y |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofPredictedDurations | Maximum no. of predicted durations in a report. Value is 5 [FFS]. |

<Unchanged Text Omitted>

9.2	Information Element definitions

<Unchanged Text Omitted>

9.2.3.Y	UE Feedback

This IE indicates the UE performance measurements metrics and ground truth after leaving the source node for prior predictions or model training.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Source UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  |
| Average UE Throughput DL | O |  | 9.2.3.4 |  |
| Average UE Throughput UL | O |  | 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.w31 |  |
| Average Packet Loss | O |  | 9.2.3.w32 |  |
| UE Trajectory Feedback | O |  | 9.2.3.w21 |  |

9.2.3.Z	Energy

This IE indicates the energy.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Energy | M |  | INTEGER | Unit: joule. |