3GPP TSG-RAN WG3 #119bis                                                 R3-231560

Online, 17th – 26th April, 2023

Agenda Item:	8.3

Source:	CATT

Title:	Discussion on RAN information exposure for XRM

Document for:	Discussion

Introduction

In last RAN3 meeting, RAN3 received LS [1] from SA2 about RAN information exposure for XRM.

SA2 has started the normative work based on the Rel-18 FS_XRM study conclusions documented in chapter 8 of TR 23.700-60.

SA2 is discussing the following new functionalities exposing NG-RAN information in the specifications under SA2 control:

- For QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4 of TS 23.501, upon SMF request, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF

- Based on SMF request over NGAP, data rate information on a per QoS Flow basis may be measured and exposed via NGAP to SMF or via GTP-U to UPF.

SA2 asks RAN3 to provide feedbacks on the above functionalities.

- Discussion

- For QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4 of TS 23.501, upon SMF request, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF

This is a UP based solution to support QoS Notification Control for GBR QoS Flow. It is considered that this function already has been support in CP based as below [2]. We are not sure the benefit of supporting UP based solution. It will introduce additional function for UPF but no additional benefit is obtained.

8.2.4	PDU Session Resource Notify

The purpose of the PDU Session Resource Notify procedure is to notify that the already established QoS flow(s) or PDU session(s) for a given UE are released or not fulfilled anymore or fulfilled again by the NG-RAN node for which notification control is requested. It is also used to notify that the updated QoS parameters during the Path Switch Request procedure are not successfully accepted by the NG-RAN node. The procedure uses UE-associated signalling.

Observation 1: QoS Notification Control for GBR QoS Flow has been support in CP based. Not sure the additional benefit to support UP based solution.

Proposal 1: RAN3 does not introduce the mechanism of QoS Notification Control for GBR QoS Flow in UP.

- Based on SMF request over NGAP, data rate information on a per QoS Flow basis may be measured and exposed via NGAP to SMF or via GTP-U to UPF.

In the current specification, the UL/DL data rate i.e. Guaranteed Flow Bit Rate Downlink/Uplink in Alternative QoS Parameters Set List can be indicated to gNB. If gNB cannot fulfil the QoS parameters, it will send "GFBR can no longer be guaranteed" indication to SMF and, if matched, an alternative bit rate can be guaranteed by NG-RAN also send to SMF. After that, the SMF may indicate the QoS parameters (i.e. 5QI, GFBR, MFBR) that the NG-RAN could be fulfilled for the QoS Flow to PCF.

9.3.1.151	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.152 |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.4 |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.4 |  |
| >Packet Delay Budget | O |  | 9.3.1.80 |  |
| >Packet Error Rate | O |  | 9.3.1.81 |  |

9.3.1.153	Alternative QoS Parameters Set Notify Index

This IE indicates the QoS parameters set which can currently be fulfilled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Notify Index | M |  | INTEGER (0..8, ...) | Indicates the index of the item within the the Alternative QoS Parameters Set List IE corresponding to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameters set. |

Observation 2: The NG-RAN can inform the CN about the currently fulfilled QoS parameters and the AF may do some changes accordingly. But there is no such mechanism for non-GBR flow.

As can be found in 23.700-60 Solution #6 [2], there is no explicit bit rate for the non-GBR Flow but it can be defined as the available mean bit rate for the non-GBR flow. The available mean bit rate is the bitrate with a time windows, e.g. averaging windows, i.e. the (default) averaging windows is also applicable to the Non-GBR 5QI. The available mean bitrate can be treated as the near-future bitrate for the non-GBR QoS Flow.

Observation 3: There is no explicit bit rate for the non-GBR Flow but the available mean bitrate can be treated as the near-future bitrate for non-GBR QoS flow.

For XR service, the QoS parameter should be strictly guaranteed and performing dynamic adjustment. The AF should know the data rate which can be supported by NG-RAN. For example, if the NG-RAN detects available mean bit rate over or less than a threshold (which is configured by SMF), the NG-RAN can send Non-GBR QoS Notification Control Report message to SMF, and then AF will determine to change the codec with indicated bit rate. This is helpful for XR service with large data volume and high transmission requirements. The CP based solution for non-GBR flow would be more align with QNC for GBR flow.

Observation 3: Dynamic adjust the data rate by AF based on available mean bit rate reported by NG-RAN would be useful for XR service.

Proposal 2: RAN3 sees the benefit of supporting NG-RAN to report data rate information on a per QoS Flow to SMF.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Observation 1: QoS Notification Control for GBR QoS Flow has been support in CP based. Not sure the additional benefit to support UP based solution.

Proposal 1: RAN3 does not introduce the mechanism of QoS Notification Control for GBR QoS Flow in UP.

Observation 2: The NG-RAN can inform the CN about the currently fulfilled QoS parameters and the AF may do some changes accordingly. But there is no such mechanism for non-GBR flow.

Observation 3: There is no explicit bit rate for the non-GBR Flow but the available mean bitrate can be treated as the near-future bitrate for non-GBR QoS flow.

Observation 3: Dynamic adjust the data rate by AF based on available mean bit rate reported by NG-RAN according to threshold) would be useful for XR service.

Proposal 2: RAN3 sees the benefit of supporting NG-RAN to report data rate information on a per QoS Flow to SMF.

# 4. Reference

[1] R3-230791, LS on RAN information exposure for XRM, SA2.

[2] 3GPP TR 23.700-60, Study on XR (Extended Reality) and media services.