3GPP TSG-RAN WG3 Meeting #119-bis	R3-231584

# Electronic meeting, April 16th – 25th 2023

Agenda Item:	10.2.1

Source:	Ericsson

Title:	Inter-RAT SHR and SPR discussion

Document for:	Discussion and Decision

# 1	Introduction

SPR for NR-DC was discussed during previous RAN3 meetings. In this contribution we continue discussing some of the topics which are not yet concluded such as the SPR configuration and root cause analysis, fetching and forwarding mechanisms and the content of the report.

Inter-RAT SHR was also discussed. In this contribution, we continue discussing some topics such as new information which may be useful to the inter-RAT SHR, and fetching and forwarding the inter-RAT SHR.

# 2	SPR for NR-DC

## 2.1	SPR configuration and root cause analysis

During RAN3#118, the following agreement was made, agreeing to this principle:

- For SN-initiated classic PScell change the source SN node decides the T310/T312 triggers (e.g timer threshold) and the target SN node decides the T304 triggers (e.g timer threshold).

During RAN3#119, the following agreement was made:

- For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis.

This leaves us with T310 and T312 for MN-initiated PSCell change. Also during RAN3#119, the following was agreed:

- If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility

- If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations

During RAN3#119, the following item for study was defined:

- FFS Which Node decides the triggers and which node performs root cause? - Option 1: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis - Option 2：For MN-initiated classic PScell change /CPC, the source SN node decides the T310/T312 triggers and performs root cause analysis - Option 3：For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis

As a SPR is generated due to a successful, but almost failed, MN-initiated PSCell change, the MN needs to consider what was the reason for this almost-failure. One typical reason could be too early PSCell change. The background to this decision is typically incorrect timing when configuring measurements in the UE and triggering PSCell change. The root cause for this is what the SPRs should help optimising.

As the MN has the best knowledge of the history of its initiated PSCell changes, the MN has also the best possibility to know when to configure source SN related SPR triggers, that is, triggers related to T310 and T312.

Both option 1 and option 3 allows for this, where the difference between them is that in option 3, the MN considers the source SN's T310/T312 timer values.

The responsibility for configuring the actual timer values in this case lies with the source SN, and not the MN. The MN is not able to change these values and need to be satisfied with what the source SN has decided. Therefore, the MN is not helped by knowing the values of these timers when it wants to decide on triggers for SPR.

For optimizing MN initiated PSCell change, the MN is not helped by knowing the values of the source SN T310/T312 timers

As stated above, the difference between options 1 and 3 is that in option 3, the MN considers the source SN's T310/T312 timer values. As we have observed above, the MN is not helped with this knowledge. Therefore, this would mean added complexity with no advantage. For these reasons, we propose that RAN3 agrees on option 1.

For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis

The MN configures triggers for SPR generation to the UE, much like a source node configures triggers for SHR. In the case of SHR, the triggers consist of a percentage of the total timer value. It would be easy to copy this behaviour to SPR without thought. However, in the case of SPR and option 1, there is a difference in that the MN does not know the values of the timers. Configuring a percentage here is therefore less relevant, as the MN does not know what this percentage is based on.

For configuring T31/T312 triggers, the MN does not know the timer values it will base its triggers on

A more relevant solution would be to define the T310/T312 SPR triggers as boolean value(s). Here, the MN decides only whether it wants a SPR for an almost-failed PSCell change or not. It does not need to guess on a percentage, for a value it does not know and cannot influence.

Define the T310/T312 triggers as a boolean values

## 2.2	Fetching and forwarding of the SPR

RAN2 agreed during RAN2#120 that only the MN can fetch the SPR.

Only MN can retrieve the SPR from the UE.

Furthermore, in RAN2#121, it was agreed that:

UE stores SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE if not fetched.

As the UE may well go to idle during this time, it is most probable that it will connect to a new node, different from the original MN node which triggered the SPR. For this reason, there is a need for the new node to be able to retrieve the SPR from the UE and send the SPR to the original MN.

RAN3 needs to define a way for a new node to forward the SPR to the original MN.

The original MN or the new node can fetch the SPR. When the original MN fetches the SPR, the UE context is still present in both MN and target SN. And may be present in source MN. In that case, an initial analysis will be performed by the MN, which will decide if the SPR is needed in source and/or target SN. This initial analysis function could also be reused if a new node fetches the SPR. Therefore, it is proposed that the new node forwards the SPR to the original MN.

In case the SPR is fetched by a new node (i.e. not initial MN, source/target SN), the new node forwards it to the initial MN

During RAN3#119, the following agreement was made:

- For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis.

For this case, the SPR was generated due to the target SN timer. As the original MN retrieves the SPR from the UE, or receives the SPR from the new MN, it is the responsibility of the original MN to forward the SPR to the target SN for root cause analysis.

For MN-initiated PSCell change, when the SPR is triggered by T304, the timers are decided by the target SN, and the original MN shall send the SPR to the target SN for root cause analysis.

For the intra-SN classic PSCell change/CPC use case, the source and target are the same node and all timers (T310, T312 and T304) are decided by that node. The original MN shall send the SPR to this SN.

For the intra-SN classic PSCell change/CPC use case, the original MN shall send the SPR to the source and target SN

## 2.3	Content of the SPR

The following RAN3 agreement was taken during previous meetings:

The following information can be included as part of SPCR (parallel discussion happening in RAN2 as well, no need to LS RAN2 if already agreed in RAN2)

a.	Source PSCell information, in case of PSCell change/CPC

b.	Target PSCell information

c.	SPCR cause

d.	Latest measurement results

e.	Location information of the UE

f.	Time elapsed between the CPAC execution and reception of CPAC configuration, in case of CPAC

However, these were not agreed in RAN2 yet. Therefore, and as the previous agreement is stating, and LS to RAN2 is now needed.

Send an LS to inform RAN2 on the RAN3 decision about the information to be included in the SPR

# 3	Inter-RAT SHR

## 3.1	NR to LTE HO - Addition of RACH related information

The RAN2#120 agreement cited in introduction says that the T304 trigger for inter-RAT SHR from NR to LTE is not supported. Therefore, for NR to LTE HO, inter-RAT SHR cannot be used for improving the handover performance in the target eNB.

Inter-RAT SHR cannot be used for improving the handover performance of the target (LTE) cell for NR to LTE HO

For this reason, we may consider other methods for improving the performance for the handover from NR to LTE. The performance of the handover execution (in terms of handover interruption time) towards a target cell depends partly on the RA performance, in terms of number of RA attempts or, in general, the latency caused by the RA procedure as the core procedure of the handover execution phase. Currently, the source gNB cannot select a suitable future target cell based on historical handover interruption time knowledge. Hence the handover decisions may lead to sub-optimal performance for inter-RAT handovers.

To help the source gNB improve its mobility parameters to an LTE target node, we propose to augment the SHR for NR-to-LTE handovers with additional information. We suggest supplementing the SHR for NR-to-LTE handovers with:

- A counter for the number of RACH attempts made for the successful handover.

- A flag on whether contention was present during the RA procedure or not.

Although this new information cannot help the target (LTE) node itself, it can improve the performance of the inter-system HO itself, by helping the source NG-RAN node to better judge which target node it should hand over UEs to.

To enable logging of these information in the SHR, we propose to enhance the SHR configuration with a triggering condition for the number of RA attempts e.g. when the number of RA attempts at the HO execution toward an LTE cell is above a certain number the UE logs inter-RAT SHR.

Note that the NR RA-Report is not created for an NR-to-LTE handover, therefore the NR RA-Report cannot be used for this purpose.

Hence, we propose the following:

For Inter-RAT handover from NR to LTE, augment the SHR with a counter for the number of RACH attempts made for the successful handover

For Inter-RAT handover from NR to LTE, augment the SHR with a flag on whether contention was observed for the successful handover

For Inter-RAT handover from NR to LTE, enhance the inter-RAT SHR configuration with a triggering condition associated to the number of random accesses attempts toward the LTE cell

## 3.2	SHR retrieval - UE logs the time since SHR generation

Some companies argued that the inter-RAT SHR should be stored in the UE until the network fetches it. We assume that it follows the same principle than RLF report and intra-RAT SHR i.e. that the UE may need to store it up to 48 hours. As the SHR is generated during a handover from NR to LTE, the target (LTE) node is not knowledgeable of SHR and cannot fetch it, under the current RAN2 agreement that LTE shall not be impacted. From this follows that the SHR can only be fetched when the UE hands over from an LTE node back to a NR node.

For NG-RAN to E-UTRAN HO, SHR is fetched when UE returns to NG-RAN

For a SHR generated for a NR-to-LTE handover, the SHR can be reported to the source node only when the UE returns to the NR network. This can take up to 48 hours

Thus, similar to CEF and RLF report, it is important that the UE logs the time between SHR report generation and fetching by the network. This will help the source node to retrieve the UE Context, if stored.

Furthermore, logging the time between SHR generation and fetching should be extended to cover not only the Inter-RAT SHR case, but also the Intra-NR SHR case. When doing this, the time is logged in the CEF report, the RLF report, the SPR, the Intra-NR SHR and the Inter-RAT SHR. This way, the possibility of correlating these reports will be improved, for example when a RLF occurs after a SHR, or a SHR occurs after RLF.

Intra-NR SHR and Inter-RAT SHR to include the time between report generation and report fetching.

## 3.3	Forwarding mechanism

The node retrieving the SHR from the UE can use information about the source node to determine where to send the SHR, the same way as for the Intra-NR SHR. For Intra-NR SHR, the retrieving node uses the Xn ACCESS AND MOBILITY INDICATION to send the SHR. We propose to reuse the same mechanism for Inter-RAT SHR.

Reuse Xn ACCESS AND MOBILITY INDICATION for forwarding the Inter-RAT SHR from the retrieving NR node to the source NR node

# 4	Conclusion

SHR for intra-system inter-RAT and SPR have been discussed further and the following observations and agreements have been made:

Successful PSCell Report (SPR):

- Observation 1	For optimizing MN initiated PSCell change, the MN is not helped by knowing the values of the source SN T310/T312 timers

- Proposal 1	For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis

- Observation 2	For configuring T31/T312 triggers, the MN does not know the timer values it will base its triggers on

- Proposal 2	Define the T310/T312 triggers as a boolean values

- Observation 3	RAN3 needs to define a way for a new node to forward the SPR to the original MN

- Proposal 3	In case the SPR is fetched by a new node (i.e. not initial MN, source/target SN), the new node forwards it to the initial MN

- Proposal 4	For MN-initiated PSCell change, when the SPR is triggered by T304, the timers are decided by the target SN, and the original MN shall send the SPR to the target SN for root cause analysis

- Proposal 5	For the intra-SN classic PSCell change/CPC use case, the original MN shall send the SPR to the source and target SN.

- Proposal 6	Send an LS to inform RAN2 on the RAN3 decision about the information to be included in the SPR

SHR for intra-system inter-RAT:

Observation 4	Inter-RAT SHR cannot be used for improving the handover performance of the target (LTE) cell for NR to LTE HO

- Proposal 7	For Inter-RAT handover from NR to LTE, augment the SHR with a counter for the number of RACH attempts made for the successful handover

- Proposal 8	For Inter-RAT handover from NR to LTE, augment the SHR with a flag on whether contention was observed for the successful handover

- Proposal 9	For Inter-RAT handover from NR to LTE, enhance the inter-RAT SHR configuration with a triggering condition associated to the number of random accesses attempts toward the LTE cell

- Proposal 10	For NG-RAN to E-UTRAN HO, SHR is fetched when UE returns to NG-RAN

Observation 5	For a SHR generated for a NR-to-LTE handover, the SHR can be reported to the source node only when the UE returns to the NR network. This can take up to 48 hours

Proposal 11	Proposal 11	Intra-NR SHR and Inter-RAT SHR to include the time between report generation and report fetching

Proposal 12	Reuse Xn ACCESS AND MOBILITY INDICATION for forwarding the Inter-RAT SHR from the retrieving NR node to the source NR node