3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231629

E-meeting, 17 – 26 April, 2023

Agenda item:	10.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for MDT BL CR to TS 38.413) Support of NPN in MDT area scope

Document for:	Text Proposal

# 1	Introduction

In the present paper we address the following agreement taken at RAN3#118:

RAN3 to focus on the following use case for SNPN and to continue discussions on how to address MDT Area Scope for specific cells or TAs of an SNPN:

Use Case 3: Enable collection of MDT measurements in the SNPN where the UE is registered.

We also perform checking as per editor’s notes introduced in the BL CR at RAN3#119.

# 2	Discussion

Area scope of specific cells of an SNPN:

A cell within an SNPN is identified by its NR CGI, composed of its 36-bit NR Cell Identity and the PLMN ID. In order to take into account MDT scenarios involving planned Rel-18 support for inter-SNPN mobility, we propose to introduce a list which additionally indicates which SNPN the cell belongs to.

Proposal 1: For cell-based MDT area scope for SNPN, indicate which SNPN the cell belongs to.

Area scope of specific TAs of an SNPN:

A Tracking Area (TA) is identified by a Tracking Area Code (TAC) within the PLMN. The globally unique identifier of the TA is the TAI (Tracking Area Identity), which is composed of the PLMN ID and the TAC. The legacy MDT Area Scope includes a TAC-based option and a TAI-based option. In the TAC-based option the TAI is derived using the current serving PLMN and is therefore not explicitly given. This option is beneficial in specific network configurations for PLMNs. The option could also be used in Rel-17 where there is no support of inter-SNPN mobility. However an area scope defined based on TAC only could become ambiguous in networks supporting inter-SNPN mobility, and we therefore believe that SNPN operators might prefer to use the TAI-based option for MDT area scope. We propose to enhance the TAI-based option to additionally indicate which SNPN the TA belongs to.

Proposal 2: For TAI-based MDT area scope for SNPN, indicate which SNPN the TA belongs to.

Area scope covering the SNPN where the UE is registered:

The legacy MDT area scope contains a PLMN-wide option, enabling MDT within the full PLMN serving the UE. We believe that a similar SNPN-wide option would be beneficial.

Proposal 3: Add an SNPN-wide choice to the MDT area scope.

Checking of BL CR:

We here address the following editor’s notes:

Editor´s note 1: The text in the Semantics Descriptions may be revisited if needed

Editor´s note 2: Further checks are needed on whether the newly added information is compatible with configuration of MDT measurements and MDT Area Scope over RRC

The updates of the NGAP MDT Area Scope introduced for PNI-NPN at RAN3#119 didn’t take into account that the Area Scope IE is mandatorily present in NGAP but optionally present in XnAP and RRC. Furthermore, the PLMN Wide choice alternative is not available in the XnAP Area Scope nor in the the RRC AreaConfiguration CHOICE structures. Over these interfaces the functionality corresponding to the PLMN Wide choice alternative, i.e. absence of area restrictions, is achieved by not including area scope information. In order to preserve the legacy functionality we therefore propose use the following semantics for the PLMN Wide choice alternative in NGAP: “This alternative corresponds to non-CAG and CAG cells.” However, a new choice alternative seems needed on all three interfaces (NG, Xn, RRC) corresponding to functionality filtering out CAG cells, hence area scope including non-CAG cells only.

Proposal 4: Keep legacy functionality for existing NGAP PLMN Wide choice alternative.

Proposal 5: Add a new choice alternative for area scope including non-CAG cells only (intended for NGAP, XnAP, RRC).

# 3	Conclusion

We have made the following proposals:

Proposal 1: For cell-based MDT area scope for SNPN, indicate which SNPN the cell belongs to.

Proposal 2: For TAI-based MDT area scope for SNPN, indicate which SNPN the TA belongs to.

Proposal 3: Add an SNPN-wide choice to the MDT area scope.

Proposal 4: Keep legacy functionality for existing NGAP PLMN Wide choice alternative.

Proposal 5: Add a new choice alternative for area scope including non-CAG cells only (intended for NGAP, XnAP, RRC).

These proposals are reflected in TP for SON BL CR to TS 38.413 included in annex of this paper.

# References

[1]		R3-231044, (BLCR to 38.413) for SON

# Annex	- TP for MDT BL CR to TS 38.413

This TP is based on [1].

<<< start of changes >>>

#### 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this alternative corresponds to non-CAG cells only. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this alternative corresponds to non-CAG cells only. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL | This alternative corresponds to non-CAG and CAG cells. |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this alternative corresponds to non-CAG cells only. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAI | M |  |  |  | - |  |
| >PLMN wide non-CAG cells |  |  |  | This alternative corresponds to non-CAG cells only. |  |  |
| >PNI-NPN based |  |  |  |  |  |  |
| >>CAG List for MDT |  | 1..<maxnoofCAGforMDT> |  |  |  |  |
| >>>PLMN ID |  |  | 9.3.3.5 |  |  |  |
| >>>CAG ID | M |  | 9.3.3.43 |  |  |  |
| >Cell based for SNPN |  |  |  |  |  |  |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - | - |
| >>>NPN Support | M |  | 9.3.3.44 | Identifies an SNPN together with the PLMN Identity IE. | - | - |
| > SNPN wide |  |  |  |  |  |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | This IE is defined in TS 38.331 [18]. | - |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | This IE is defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT | O |  | 9.3.3.x | This IE is ignored if the Area Scope of MDT IE is set to "PLMN wide" or “PNI-NPN based” | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGforMDT | Maximum no. of PNI-NPN CAG IDs for MDT scope. Value is FFS. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

Editor´s note 1: The text in the Semantics Descriptions may be revisited if needed

Editor´s note 2: Further checks are needed on whether the newly added information is compatible with configuration of MDT measurements and MDT Area Scope over RRC

#### 9.3.3.x	PNI-NPN Area Scope of MDT

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CAG List for MDT |  | 1..< maxnoofCAGforMDT > |  |  |
| >PLMN Identity | M |  | 9.3.3.5 |  |
| >CAG ID | M |  | 9.3.3.43 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCAGforMDT | Maximum no. of PNI-NPN CAG IDs for MDT area scope. Value is 8. |

<<< end of changes >>>