3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231727

Electronic meeting, 17 Apr – 26 Apr, 2023

Title: 	Discussion on the presence of area scope in MDT configuration

Source: 	Huawei, CMCC,  China Unicom

Agenda item:	9.2.1

Document Type:	Discussion

1. Introduction

Two issues were detected in MDT Configuration IE in Rel-16 XnAP in the context of preparing the Rel-18 XnAP TPs for support of MDT in PNI-NPN.

- Issue 1: PLMN wide branch is missing in the area scope IE.

- Issue 2: The presence of the area scope IE is optional which is not aligned to NGAP and to S1/X2 in LTE MDT.

In this contribution, we discuss how to correct these two issues for MDT before Rel-18.

2. Background

Status quo in LTE

In section 9.2.1.81 TS 36.413, it shows that the Area Scope IE is mandatory in the MDT Configuration IE. And the area scope IE supports the option of PLMN wide branch to enable the MDT data is corrected in the whole serving PLMN.

In section 9.2.56, TS 36.423, it shows that the area scope IE is mandatory when forwarding the MDT Configuration IE over X2 interface. And the PLMN wide option is also supported on X2.

#### TS 36.413 #### 9.2.1.81 MDT Configuration The IE defines the MDT configuration parameters. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. MDT Activation; M; ENUMERATED(Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…, Logged MBSFN MDT); -; -. CHOICE Area Scope of MDT; M; -; -. >Cell based; -. >>Cell ID List for MDT; 1 .. <maxnoofCellIDforMDT>; -. >>>E-CGI; M; 9.2.1.38; -; -. >TA based; -. >>TA List for MDT; 1 .. <maxnoofTAforMDT>; -. >>>TAC; M; 9.2.3.7; The TAI is derived using the current serving PLMN.; -; -. >PLMN Wide; NULL; -; -. >TAI based; -; -. >>TAI List for MDT; 1 .. <maxnoofTAforMDT>; -; -. >>>TAI; M; 9.2.3.16; -; -. ### TS 36.423 ### 9.2.56 MDT Configuration The IE defines the MDT configuration parameters. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. MDT Activation; M; ENUMERATED(Immediate MDT only, Immediate MDT and Trace, …); –. CHOICE Area Scope of MDT; M; –. >Cell Based; –. >>Cell ID List for MDT; 1..<maxnoofCellIDforMDT>; –. >>>ECGI; M; 9.2.14; –. >TA Based; –. >>TA List for MDT; 1..<maxnoofTAforMDT>; –. >>>TAC; M; OCTET STRING (2); Tracking Area Code. The TAI is derived using the current serving PLMN.; –. >PLMN Wide; NULL; –. >TAI based. >>TAI List for MDT; 1..<maxnoofTAforMDT>. >>>TAC; M; OCTET STRING (2); Tracking Area Code. >>>PLMN Identity; M; 9.2.4.

Observation 1: The Area Scope IE is mandatory in MDT Configuration IE in S1 and X2.

Observation 2: The PLMN wide option is supported for area scope in MDT Configuration IE on X2.

Status quo in NG-RAN

TS 38.413 #### 9.3.1.169 MDT Configuration-NR This IE defines the MDT configuration parameters of NR. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. MDT Activation; M; ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …); -. CHOICE Area Scope of MDT; M; -. >Cell based. >>Cell ID List for MDT; 1..<maxnoofCellIDforMDT>. >>>NR CGI; M; 9.3.1.7; -. >TA based. >>TA List for MDT; 1..<maxnoofTAforMDT>. >>>TAC; M; 9.3.3.10; The TAI is derived using the current serving PLMN.; -. >PLMN wide; NULL. >TAI based. >>TAI List for MDT; 1..<maxnoofTAforMDT>. >>>TAI; M; -. TS 38.423 #### 9.2.3.126 MDT Configuration-NR The IE defines the MDT configuration parameters of NR. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MDT Activation; M; ENUMERATED (Immediate MDT only, Immediate MDT and Trace, Logged MDT only, ...). CHOICE Area Scope of MDT-NR; O. >Cell based. >>Cell ID List for MDT-NR; 1 .. <maxnoofCellIDforMDT>. >>>NR CGI; M; 9.2.2.7. >TA based. >>TA List for MDT; 1 .. <maxnoofTAforMDT>. >>>TAC; M; OCTET STRING (SIZE (3)); The TAI is derived using the current serving PLMN.. >TAI based. >>TAI List for MDT; 1. >>>TAI List for MDT Item; 1 .. <maxnoofTAforMDT>. >>>>PLMN Identity; M; 9.2.2.4. >>>>TAC; M; 9.2.2.5.

In section 9.3.1.169, TS 38.413, it shows that the Area Scope IE is mandatory in the MDT Configuration-NR IE. And the area scope IE supports the option of PLMN wide branch. The same status is found in the MDT Configuration-EUTRA IE.

In 9.2.3.126, TS 38.423, it shows that the Area Scope IE is  implemented as an optional IE by mistake, and the PLMN wide option is missing in the Area Scope IE. The same issues are found in the MDT Configuration-EUTRA IE.

Observation 3: The Area Scope IE is mandatory in MDT Configuration-NR IE and MDT Configuration-EUTRA IE in NG.

Observation 4: The following two issues exist in MDT Configuration-NR IE and MDT Configuration-EUTRA IE in X2:

- Issue 1: PLMN wide branch is missing in the area scope IE.

- Issue 2: The presence of the area scope IE is optional which is not aligned to NGAP and to S1/X2 in LTE MDT.

Status quo in SA5 spec

TS 28.622 #### 4.3.30.3 Attribute constraints Name; Definition. listOfInterfaces (support qualifier); This attribute shall be present when jobType includes Trace.. listOfNETypes (support qualifier); This attribute shall be present only for Trace with Signalling Based Activation. pLMNTarget (support qualifier); This attribute shall be present for management based activation when several PLMNs are supported in the RAN.. traceReportingConsumerUri (support qualifier); This attribute shall be present if streaming trace data reporting is supported and traceReportingFormat set to "streaming".. traceCollectionEntityIPAddress (support qualifier); This attribute shall be present if file based trace data reporting is supported and traceReportingFormat set to "file based" or when jobType is set to Logged MDT or Logged MBSFN MDT.. traceDepth (support qualifier); This attribute shall be present when jobType includes Trace.. triggeringEvents (support qualifier); This attribute shall be present when jobType includes Trace.. anonymizationOfMDTData (support qualifier); This attribute shall be present only if MDT is supported and the areaScope attribute is present. This attribute is only applicable for management based activation.. areaConfigurationForNeighCell (support qualifier); This attribute shall be present only if NR MDT is supported and the jobType attribute is set to Logged MDT.. areaScope (support qualifier); This attribute shall be present if MDT is supported.. collectionPeriodRRMLTE (support qualifier); This attribute shall be present only if MDT is supported and the jobType attribute is set to Immediate MDT or combined Trace and Immediate MDT and the listOfMeasurements attribute has any of M2, M3 measurement set in case of LTE.. TS 32.422 5.10.2 Area Scope The Area Scope optional parameter defines the area in terms of Cells or Tracking Area/Routing Area/Location Area where the MDT data collection shall take place. The area scope specified in an MDT session shall support the PLMNs of the MDT PLMN list (defined in clause 5.10.24). If the parameter is not present the MDT data collection shall be done throughout the PLMNs of the MDT PLMN list. For further details see also TS 37.320 [30].

TS 28.622 is the staged 3 specification of the Northbound interface between NMS and EMS maintaining by SA5. And the NMS is the original network element where a MDT task is initiated. In section 4.3.30.3, it specifies that the area scope attribute shall be present on Northbound interface for MDT.

However, in TS 32.422, the stage 2 spec in SA5 for both LTE MDT and NG-RAN MDT, in section 5.10.2, it describers that the area scope is optional, and if the parameter is not present the MDT data collection shall be done throughout the PLMNs of the MDT PLMN list.

Observation 5: The Area Scope attribute is mandatory on Northbound interface for MDT.

Observation 6:  SA5’s stage 2 specification for MDT describers that the area scope is an optional parameter. And if not present the MDT data collection shall be done throughout the PLMNs of the MDT PLMN list.

3. Discussion

According to the observations in section 2, the following issuers needs to be clarified and solved in RAN3.

- Issue 1: the presence of the area scope IE in MDT configuration on Xn

- Issue 2: Does PLMN wide option need to support on Xn?

For Issue 1, there are two options:

Option 1: The IE is mandatory on Xn to align with LTE and NGAP from function perspective. So, changing the presence from optional to mandatory on Xn in either backwards compatible way or NBC way is needed.

Option 2: The IE could be optional according to the stage 2 text in TS 32.422.  Procedural text is need in Xn.

We see the following problems for option 2:

- Function misalignment for MDT between LTE and NR, and between NGAP and Xn interface.

- The stage 2 text in TS 32.422 saying the area scope is optional is ambiguity.  It is not sure whether it is applicable for MDT initiation in OAM, S1/NG or X2/Xn.

- If  stage 2 text in TS 32.422 is correct, it means that changing the area scope to optional in S1/X2 and NG is needed since Rel-10. And as per past experience, when there is conflict between stage 2 spec and stage 3 spec, stage 3 will take the precedence.

- If OAM does not include  the area scope in a MDT task, it is not clear how the AMF/MME will create a area scope for such MDT task on NGAP because it’s mandatory on NGAP. And the impact on AMF/MME is not preferable since usually, the AMF/MME will not interpret and modify the MDT configuration receiving from HSS/UDM.

- If this IE is optional on Xn, it enables the NG-RAN node  not to forward it out over Xn when the IE is received from NGAP.

And for issue 2,  the answer should be yes.  Otherwise, if a PLMN wide area scope is received on NGAP, there is no way for the NG-RAN node to forward it to target on Xn correctly.

In summary, we propose to adopt option 1 which is to make the area scope mandatory for MDT on Xn in either backwards compatible way or NBC way.

Proposal 1: To change the Area Scope IE in Xn mandatory in either backwards compatible way or NBC way.

Proposal 2: To introduce the PLMN wide option in the Area Scope IE.

4. Conclusion

- In this contribution, we discussed the issues related to the area scope in MDT in Xn and made the following observations and proposals.

Observation 1: The Area Scope IE is mandatory in MDT Configuration IE in S1 and X2.

Observation 2: The PLMN wide option is supported for area scope in MDT Configuration IE on X2.

Observation 3: The Area Scope IE is mandatory in MDT Configuration-NR IE and MDT Configuration-EUTRA IE in NG.

Observation 4: The following two issues exist in MDT Configuration-NR IE and MDT Configuration-EUTRA IE in X2:

Observation 5: The Area Scope attribute is mandatory on Northbound interface for MDT.

Observation 6:  SA5’s stage 2 specification for MDT describers that the area scope is an optional parameter. And if not present the MDT data collection shall be done throughout the PLMNs of the MDT PLMN list.

Proposal 1: To change the Area Scope IE in Xn mandatory in either backwards compatible way or NBC way.

Proposal 2: To introduce the PLMN wide option in the Area Scope IE.

The CRs are provided in [1] and [2].

- And if there is no consensus in RAN3, it is proposed to send a LS to SA5 for clarification.

5. Reference

- R3-231728	Correction on the Area Scope IE in MDT Configuration

- R3-231729	Correction on the Area Scope IE in MDT Configuration

Annex – TP

<optional – here you can add your TP or backgrounds of simulations>