# 3GPP RAN WG3 Meeting #120							R3-233320

# Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	9.2.1

Source:	Ericsson (Moderator)

Title:	Summary of Offline Discussion on CB # 8_RedcapLSout

Document for:	Approval

# Introduction

CB: # 8_RedcapLSout

- Work on the issues to be clarified in other WGs

- Check whether solution in R3-233085 is agreeable

- LS to RAN2, RAN4

(moderator – E///)

# For the Chairman’s Notes

Agree the LS to RAN2, cc RAN4 in R3-233346

Treat the issue in R3-233085 together with NCD-SSB measurement discussion after RAN2 reply

# Discussion

In RAN3#119bis-e meeting, companies have questioned the potential mixed usage of preconfigured measurement gap and concurrent measurement gap over F1, which was raised after discussing the CR in [1] proposing the usage of “gapToAddModList” within the MeasGapConfig to ask DU to generate pre-configured measurement gap (Pre-MG) to be sent by CU to the RedCap UE.

Below are the findings from the offline e-mail discussion after checking TS 38.331 and TS 38.133:

- The “gapToAddModList” IE defined in 38.331 is not only being used for concurrent measurement gap, but also for Pre-MG:

gapToAddModList A list of measurement gap configuration to be added or modified. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8), the maximum number of configured measurement gap is limited by the gap combinations defined in Table 9.1.8-1 in TS 38.133 [14]. The network configures at most one NCSG or pre-configured measurement gap for a given gap type. In this version of the specification, the network configures this field only in NR standalone. This field is used only for a UE that supports pre-configured measurement gap, concurrent measurement gap, or NCSG. In this version of the specification, the network does not configure concurrent measurement gap together with MUSIM gap or preconfigured measurement gap for positioning.

We can see that Pre-MG is included in the definition, and that the network configures at most one NCSG (network-controlled small gap) or Pre-MG for a given gap type. There is also no limitation for RedCap UE in TS 38.331.

- The (RedCap) UE can report to network its support of Pre-MG (or concurrent gap) function via RRC UE Capability report. From TS 38.331:

MeasAndMobParametersCommon ::= SEQUENCE { supportedGapPattern BIT STRING (SIZE (22)) OPTIONAL, ... [[ -- R4 19-2 Concurrent measurement gaps concurrentMeasGap-r17 CHOICE { concurrentPerUE-OnlyMeasGap-r17 ENUMERATED {supported}, concurrentPerUE-PerFRCombMeasGap-r17 ENUMERATED {supported} } OPTIONAL, ... -- R4 19-3-2 pre-configured measurement gap preconfiguredUE-AutonomousMeasGap-r17 ENUMERATED {supported} OPTIONAL, -- R4 19-3-1 pre-configured measurement gap preconfiguredNW-ControlledMeasGap-r17 ENUMERATED {supported} OPTIONAL, ... -- R4 19-2-1 Concurrent measurement gaps for EUTRA concurrentMeasGapEUTRA-r17 ENUMERATED {supported} OPTIONAL, ... }

- If UE supports pre-configured measurement gap or concurrent measurement gap, the NG-RAN can configure it according to the network behaviour specified in TS 38.133 (RedCap UE is not mentioned):

##### 9.1.7.3.2 Requirements for network-controlled activation/deactivation mechanism The requirements in this section apply when network-controlled mechanism [1] is used for activation/deactivation of Pre-MG pattern. For per-UE Pre-configured MG, - the UE determines that the Pre-configured MG is activated for the active DL BWP if the corresponding gap ID is not present in deactivatedMeasGapList-r17 [2] for the active DL BWP of any of the activated CCs, or if the corresponding gap ID is not present in deactivatedMeasGapList-r17 [2] for any of the deactivated SCCs, - otherwise, the UE determines that the Pre-configured MG is deactivated For per-FR Pre-configured MG, - the UE determines that the Pre-configured MG is activated for the active DL BWP if the corresponding gap ID is not present in deactivatedMeasGapList-r17 [2] for the active DL BWP of any of the activated CCs in the corresponding FR, or if the corresponding gap ID is not present in deactivatedMeasGapList-r17 [2] for any of the deactivated SCCs in the corresponding FR,

The deactivatedMeasGapList-r17 comes from BWP config and its definition is preConfGapStatus-r17 in TS 38.331:

(note: deactivatedMeasGapList-r17 was the old name of preConfGapStatus-r17 in RRC spec. See RAN2 CR http://portal.3gpp.org/ngppapp/DownloadTDoc.aspx?contributionUid=R2-2206455)

TS 38.331:

BWP-DownlinkDedicated IE -> preConfGapStatus-r17 : preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated upon the switch to this BWP. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

Based on the above, the RedCap UE expects the network to configure all possible Pre-MGs that will be activated by the UE in case of active BWP switching. However, if the UE only supports single Pre-MG (based on “gapToAddModList” definition), then it's impossible to activate different Pre-MGs for different BWPs, as all BWPs will use the same Pre-MG.

Also, the RAN4 specifications do not have clear description if the RedCap UE does support Pre-MG.

Observation 1: Based on the RAN2 specifications, in network-controlled mode and based on UE capability, the RedCap UE expects the network to configure all possible Pre-MGs that will be activated by the UE in case of active BWP switching via the presence of the preConfGapStatus IE.

Observation 2: based on “gapToAddModList” definition, only one Pre-MG can at most be configured by the network.

Observation 3: The RAN4 specifications do not have clear description if the RedCap UE does support Pre-MG(s)

- Q1) Do companies agree with these three Observations:

| Company | O1, O2, O3 | Comment |
| --- | --- | --- |
| Ericsson | Yes to all | Our understanding is that signalling-wise from RAN2 spec it’s allowed to support Pre-MG in RedCap, but RedCap UE behaviour for Pre-MG is unclear in RAN4 spec. Also it is not clear what “at most one Pre-MG per gap type” means in the “gapToAddModList” definition. |
| Huawei | Yes to all | This is exactly something we are not clear and need to ask RAN2/RAN4 for clairfication |
| Qualcomm | Yes | We can check with RAN2/RAN4 for clarification |
| Nokia |  | Main point is to confirm these points with RAN2/RAN4 |
| ZTE | Yes to all | All the points should be checked by RAN2/RAN4 |
| Moderator’s conclusion: - Clarification is needed on applicability of different Pre-MG for RedCap UE |  |  |

Another question is how to handle the solution in [2] by ZTE for the scenario of UE performing Handover to a NCD-SSB BWP in target cell directly. To moderator’s understanding, the issue can be also related to the topic of neighbour cell measurements, since Pre-MG are needed when the UE performs neighbour cell measurements and the target CU should be able to realize if UE’s 1st active DL BWP in the target cell upon handover is a BWP with NCD-SSB.

- Q2) Should the solution proposed in [2] by ZTE be seen as:

- a solution for a issue to be handled separately from the NCD-SSB measurement discussion

- or included as part of the “package” for Pre-MG discussion?

- Other solutions (e.g. implementation)

| Company | 1, 2 or 3 | Comment |
| --- | --- | --- |
| Ericsson | 2) | The solution by [2] can be addressed by either adding a “used servingCellMO IE” and active BWP and location IE in the existing ServingCellMO-encoded-in-CGC List IE, or as part of the solution proposed in [1]. In both cases, the CU can know which SSB/servingCellMO is the current used one for the 1st active BWP, and thus know if UE can read the SI message (if NCD-SSB) |
| Huawei | 3 with comments | Our view is implementation way can solve the issue. If companies share a different view, then the issue raised in [2] may has a correlation to the CR we are discussing, so we can postpone the discussion. |
| Qualcomm | 2 | Same view as Ericsson |
| Nokia | 2 | Prefer to treat this issue together with NCD-SSB measurement discussion. |
| ZTE | 1 or 2 | We prefer 1 since the SI delivery issue has been acknowledged as a Rel-17 issue, and whether the Pre-MGs for RedCap UE can be supported in Rel-17 is still FFS. As a compromise, we can wait for the rely from RAN2/RAN4, if the Pre-MGs for RedCap UE can be supported in Rel-17, we can go for 2. Otherwise, an individual CR in [2] is needed to solve the SI delivery issue. |
| Moderator’s conclusion: - Treat the issue with NCD-SSB measurement discussion after RAN2 reply |  |  |

During the online discussion, majority of the companies have expressed wish to ask RAN2 and RAN4 via LS from RAN3 for clarification whether multiple Pre-MG scenario as defined in TS 38.331 via preConfGapStatus IE can be supported for RedCap in NCD-SSB measurement Rel-17.

A draft LS to RAN2 and RAN4 is proposed below for review to be sent and proposed to be taken by Qualcomm as contact company:

1. Overall Description: During Release 17 RedCap maintenance discussion, RAN3 has discussed the topic of RedCap UE configured with BWP specific Serving Cell Measurement Object (ServingCellMO) support over F1. To support the scenario of DCI based BWP switching scenario for a RedCap UE, based on the introduction of preConfGapStatus-r17 IE in BWP-DownlinkDedicated IE in TS 38.331, RAN3 is discussing how the gNB-CU can request the gNB-DU to provide configuration of all Pre-Configured Measurement GAP (Pre-MG) that can be used after the active BWP associated with NCD-SSB is switched via DCI for the RedCap UE that supports this capability. However, based on “gapToAddModList IE” definition in MeasGapConfig, RAN3 would like to ask RAN2 whether it is possible that this IE can be used to activate different Pre-MGs for different BWPs, since the RedCap UE will require a MG whenever it performs a measurement outside its active BWP. Moreover, RAN3 would like to ask RAN4 whether Rel-17 Pre-MG can be used to handle RedCap NCD-SSB measurement. 2. Actions: To RAN2/RAN4: ACTION: RAN3 kindly asks RAN2 and RAN4 to feedback on the questions above. 3. Date of Next TSG-RAN3 Meetings: TSG RAN3 Meeting #120 21 - 25 August 2023 Republic of France, Toulouse TBD

- Q3) Companies are invited to review the LS to RAN2/RAN4 proposed above

| Company | Comment |
| --- | --- |
| Huawei | Looks ok. Could QC also put a draft LS out in the folder, so companies can modify it directly. |
| Nokia | Generally OK, but final LS draft should be reviewed separately from SoD as suggested by Huawei |
| Based on offline discussion: - The RAN4 question can lead to confusion and vague interpretation., It is proposed to first get clarification from RAN2 on the signalling No need to question RAN4, put in cc |  |

# Conclusion, Recommendations

Agree the LS to RAN2, cc RAN4 in R3-233346

# References

- R3-233058, Support of preconfigured Measurement GAPs for RedCap UE (Ericsson, Qualcomm Incorporated, CATT)

- R3-233085, Correction on SI delivery to RedCap UE during handover (ZTE, China Telecom, CMCC)