3GPP TSG-RAN WG3 Meeting #120	R3-233353

# Incheon, Republic of Korea, May 22nd – 26th 2023

Agenda Item:	11.3

Source:	Ericsson (moderator)

Title:	Summary of offline discussion on QoE measurements for NR-DC

Document for:	Approval

# Introduction

These are the proposals resulting from the offline discussion on QoE measurement support for NR-DC, and the subsequent discussion over the reflector.

# For the Chairman notes

QoE measurement configuration

Proposal 1-1: When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it is going to receive the QoE/RVQoE reports via the SRB5.

FFS whether:

- For management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE via SRB3.

- For management-based QoE, the SN can explicitly indicate to the MN whether it wants to configure the UE transparently for the MN via SRB1.

QoE measurement reporting

Proposal 2: RAN3 understanding is that the UE is explicitly instructed how to send the QoE and RVQoE reports.

RVQoE coordination

Proposal 3-1: When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit.

Proposal 3-2: In the response, the non-RVQoE-configuring node:

- Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports.

- Can indicate whether it prefers to receive the RVQoE reports directly from the UE.

- Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration.

Proposal 3-3: When the non-RVQoE-configuring node receives an RVQoE report and determines that the RVQoE-configuring node provides the bearer(s) for the application session, the non-RVQoE-configuring node indicates that to the RVQoE-configuring node.

Proposal 3-4: The non-RVQoE-configuring node can update its preferred RVQoE configuration parameters and reporting leg preference, and it can request from the RVQoE-configuring node to update these parameters at the UE.

Mobility

Proposal 4-1: In line with the Rel-17 principles, if the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RVQoE measurements.

PSCell change

Proposal 4-2: At PSCell change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

SN release

Proposal 5-1: If the SN configured a UE with QoE measurements, at SN release, the QoE measurement configuration can be released.

Proposal 5-2: If the SN configured a UE with RVQoE measurements, at SN release, the RVQoE measurement configuration can be released.

FFS whether the SN-configured QoE/RVQoE measurements can be passed to the MN in case of SN release.