3GPP TSG-RAN WG3 Meeting #120	R3-233355

Incheon, KR, 22 May – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0103 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SRS Resource correction on Comb 8, Number of Symbols and Repetition Factor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, China Telecom, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.331, there are some Rel-17 parameters for the SRS-Resource IE which are not aligned with the TS 38.455, as highlighted follows: SRS-Resource ::= SEQUENCE { srs-ResourceId SRS-ResourceId, nrofSRS-Ports ENUMERATED {port1, ports2, ports4}, ptrs-PortIndex ENUMERATED {n0, n1 } OPTIONAL, -- Need R transmissionComb CHOICE { n2 SEQUENCE { combOffset-n2 INTEGER (0..1), cyclicShift-n2 INTEGER (0..7) }, n4 SEQUENCE { combOffset-n4 INTEGER (0..3), cyclicShift-n4 INTEGER (0..11) } }, resourceMapping SEQUENCE { startPosition INTEGER (0..5), nrofSymbols ENUMERATED {n1, n2, n4}, repetitionFactor ENUMERATED {n1, n2, n4} }, freqDomainPosition INTEGER (0..67), freqDomainShift INTEGER (0..268), freqHopping SEQUENCE { c-SRS INTEGER (0..63), b-SRS INTEGER (0..3), b-hop INTEGER (0..3) }, groupOrSequenceHopping ENUMERATED { neither, groupHopping, sequenceHopping }, resourceType CHOICE { aperiodic SEQUENCE { ... }, semi-persistent SEQUENCE { periodicityAndOffset-sp SRS-PeriodicityAndOffset, ... }, periodic SEQUENCE { periodicityAndOffset-p SRS-PeriodicityAndOffset, ... } }, sequenceId INTEGER (0..1023), spatialRelationInfo SRS-SpatialRelationInfo OPTIONAL, -- Need R ..., [[ resourceMapping-r16 SEQUENCE { startPosition-r16 INTEGER (0..13), nrofSymbols-r16 ENUMERATED {n1, n2, n4}, repetitionFactor-r16 ENUMERATED {n1, n2, n4} } OPTIONAL -- Need R ]], [[ spatialRelationInfo-PDC-r17 SetupRelease { SpatialRelationInfo-PDC-r17 } OPTIONAL, -- Need M resourceMapping-r17 SEQUENCE { startPosition-r17 INTEGER (0..13), nrofSymbols-r17 ENUMERATED {n1, n2, n4, n8, n10, n12, n14}, repetitionFactor-r17 ENUMERATED {n1, n2, n4, n5, n6, n7, n8, n10, n12, n14} } OPTIONAL, -- Need R partialFreqSounding-r17 SEQUENCE { startRBIndexFScaling-r17 CHOICE{ startRBIndexAndFreqScalingFactor2-r17 INTEGER (0..1), startRBIndexAndFreqScalingFactor4-r17 INTEGER (0..3) }, enableStartRBHopping-r17 ENUMERATED {enable} OPTIONAL -- Need R } OPTIONAL, -- Need R transmissionComb-n8-r17 SEQUENCE { combOffset-n8-r17 INTEGER (0..7), cyclicShift-n8-r17 INTEGER (0..5) } OPTIONAL, -- Need R srs-TCI-State-r17 CHOICE { srs-UL-TCI-State TCI-UL-StateId-r17, srs-DLorJointTCI-State TCI-StateId } OPTIONAL -- Need R ]], [[ repetitionFactor-v1730 ENUMERATED {n3} OPTIONAL, -- Need R srs-DLorJointTCI-State-v1730 SEQUENCE { cellAndBWP-r17 ServingCellAndBWP-Id-r17 } OPTIONAL -- Cond DLorJointTCI-SRS ]] } |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1. Add comb 8 to SRS Resource IE. Change 2. Extend the values for Number of Symbols and Repetition Factor. Change 3. Add the start RB location hopping for partial frequency sounding. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification, it is only affected the SRS Resource configuration for Combo node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NRPPa is not aligned with RRC specification, UE cannot be configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.29, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.473 CR 1177 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: correction of the work Item Code, tabular order correction, add co-source and ASN.1 corrections |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.2.29	SRS Resource

This information element contains the SRS resource.

| IE/Group Name |  | Presence |  | Range |  | IE Type and Reference |  | Semantics Description |  |  |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SRS Resource ID |  | M |  |  |  | INTEGER(0..63) |  |  |  |  |  | - |  |
| Number of Ports |  | M |  |  |  | ENUMERATED(port1, ports2, ports4) |  |  |  |  |  | - |  |
| CHOICE Transmission Comb |  | M |  |  |  |  |  |  |  |  |  | - |  |
| >Comb Two |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Comb Offset |  | M |  |  |  | INTEGER(0..1) |  |  |  |  |  | - |  |
| >>Cyclic Shift |  | M |  |  |  | INTEGER(0..7) |  |  |  |  |  | - |  |
| >Comb Four |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Comb Offset |  | M |  |  |  | INTEGER(0..3) |  |  |  |  |  | - |  |
| >>Cyclic Shift |  | M |  |  |  | INTEGER(0..11) |  |  |  |  |  | - |  |
| >Comb Eight |  | M |  |  |  |  |  |  |  |  |  | YES | reject |
| >>Comb Offset |  | M |  |  |  | INTEGER(0..7) |  |  |  |  |  | - | - |
| >>Cyclic Shift |  | M |  |  |  | INTEGER(0..5) |  |  |  |  |  | - | - |
| Start Position |  | M |  |  |  | INTEGER(0..13) |  |  |  |  |  | - |  |
| Number of Symbols |  | M |  |  |  | ENUMERATED(n1,n2,n4) |  | This IE is ignored if the Number of Symbols Extended IE is included |  |  |  | - |  |
| Repetition Factor |  | M |  |  |  | ENUMERATED(r1,r2,r4) |  | This IE is ignored if the Repetition Factor Extended IE is included |  |  |  | - |  |
| Frequency Domain Position |  | M |  |  |  | INTEGER(0..67) |  |  |  |  |  | - |  |
| Frequency Domain Shift |  | M |  |  |  | INTEGER(0..268) |  |  |  |  |  | - |  |
| C-SRS |  | M |  |  |  | INTEGER(0..63) |  |  |  |  |  | - |  |
| B-SRS |  | M |  |  |  | INTEGER(0..3) |  |  |  |  |  | - |  |
| B-Hop |  | M |  |  |  | INTEGER(0..3) |  |  |  |  |  | - |  |
| Group or Sequence Hopping |  | M |  |  |  | ENUMERATED(neither, groupHopping, sequenceHopping) |  |  |  |  |  | - |  |
| CHOICE Resource Type |  | M |  |  |  |  |  |  |  |  |  | - |  |
| >Periodic |  |  |  |  |  |  |  |  |  |  |  | - |  |
| >>Periodicity |  | M |  |  |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  |  |  |  |  | - |  |
| >>Offset |  | M |  |  |  | INTEGER(0..2559, …) |  |  |  |  |  | - |  |
| >Semi-persistent |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Periodicity |  | M |  |  |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  |  |  |  |  | - |  |
| >>Offset |  | M |  |  |  | INTEGER(0..2559, …) |  |  |  |  |  | - |  |
| >Aperiodic |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Aperiodic Resource Type |  | M |  |  |  | ENUMERATED(true,…) |  |  |  |  |  | - |  |
| Sequence ID |  | M |  |  |  | INTEGER(0..1023) |  |  |  |  |  | - |  |
| Number of Symbols Extended | O |  | ENUMERATED(n8,n10,n12, n14, …) |  | YES | ignore |
| Repetition Factor Extended | O |  | ENUMERATED(r3, r5, r6, r7, r8, r10, r12, r14, …) |  | YES | ignore |
| Start RB Hopping | O |  | ENUMERATED(enable) |  | YES | ignore |
| CHOICE Start RB Index | O |  |  |  | YES | ignore |
| >FreqScalingFactor2 | M |  | INTEGER (0..1) |  | - | - |
| >FreqScalingFactor4 | M |  | INTEGER (0..3) |  | - | - |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRP-Rx-TEGInformation,

id-TRPBeamAntennaInformation,

id-NR-TADV,

id-pathPower,

id-SRSPortIndex,

id-UETxTimingErrorMargin,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-StartRBHopping,

id-StartRBIndex,

id-transmissionCombn8

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

NRCellIdentifier ::= BIT STRING (SIZE (36))

nrofSymbolsExtended ::=  ENUMERATED {n8, n10, n12, n14, ...}

NR-PCI ::= INTEGER (0..1007)

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

repetitionFactorExtended ::=  ENUMERATED {n3, n5, n6, n7, n8, n10, n12, n14, ...}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

SRSResource::= SEQUENCE {

sRSResourceID					SRSResourceID,

nrofSRS-Ports					ENUMERATED {port1, ports2, ports4},

transmissionComb				TransmissionComb,

startPosition					INTEGER (0..13),

nrofSymbols						ENUMERATED {n1, n2, n4},

repetitionFactor				ENUMERATED {n1, n2, n4},

freqDomainPosition				INTEGER (0..67),

freqDomainShift					INTEGER (0..268),

c-SRS							INTEGER (0..63),

b-SRS							INTEGER (0..3),

b-hop							INTEGER (0..3),

groupOrSequenceHopping			ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType					ResourceType,

sequenceId						INTEGER (0..1023),

iE-Extensions					ProtocolExtensionContainer { { SRSResource-ExtIEs } } OPTIONAL,

...

}

SRSResource-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-nrofSymbolsExtended			CRITICALITY ignore TYPE nrofSymbolsExtended 			PRESENCE optional}|

{ ID id-repetitionFactorExtended		CRITICALITY ignore TYPE repetitionFactorExtended 	PRESENCE optional}|

{ ID id-StartRBHopping			CRITICALITY ignore TYPE StartRBHopping 			PRESENCE optional}|

{ ID id-StartRBIndex			CRITICALITY ignore TYPE StartRBIndex 			PRESENCE optional},

...

}

SRSResourceID ::= INTEGER (0..63)

SRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-Resources)) OF SRSResource

SRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-ResourceSets)) OF SRSResourceSet

SRSResourceID-List::= SEQUENCE (SIZE (1..maxnoSRS-ResourcePerSet)) OF SRSResourceID

SRSResourceSet::= SEQUENCE {

sRSResourceSetID                INTEGER(0..15),

sRSResourceID-List				SRSResourceID-List,

resourceSetType					ResourceSetType,

iE-Extensions					ProtocolExtensionContainer { { SRSResourceSet-ExtIEs } } OPTIONAL,

...

}

SRSResourceSet-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceSetID ::= INTEGER (0..15, ...)

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

SSB-Index ::= INTEGER(0..63)

SSID ::= OCTET STRING (SIZE(1..32))

StartRBIndex  ::= CHOICE{

FreqScalingFactor2  INTEGER(0..1)

FreqScalingFactor4  INTEGER(0..3)

choice-extension	ProtocolIE-Single-Container { { StartRBIndex-ExtIEs} }

}

StartRBIndex-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

StartRBHopping  ::= ENUMERATED {enable}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

TransmissionComb ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

choice-extension				ProtocolIE-Single-Container { { TransmissionComb-ExtIEs} }

}

TransmissionComb-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TransmissionCombn8	CRITICALITY reject TYPE TransmissionCombn8 PRESENCE mandatory},

...

}

TransmissionCombn8 ::= SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5),

iE-Extensions					ProtocolExtensionContainer { { TransmissionCombn8-ExtIEs } } OPTIONAL

}

TransmissionCombn8-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted>>>>>>>>>>>>>>>>>>>>

id-SRSPortIndex													ProtocolIE-ID ::= 100

id-procedure-code-101-not-to-be-used							ProtocolIE-ID ::= 101

id-procedure-code-102-not-to-be-used							ProtocolIE-ID ::= 102

id-procedure-code-103-not-to-be-used							ProtocolIE-ID ::= 103

id-UETxTimingErrorMargin										ProtocolIE-ID ::= 104

id-MeasurementPeriodicityNR-AoA									ProtocolIE-ID ::= 105

id-SRSTransmissionStatus										ProtocolIE-ID ::= 106

id-nrofSymbolsExtended												ProtocolIE-ID ::= xx1

id-repetitionFactorExtended											ProtocolIE-ID ::= xx2

id-StartRBHopping												ProtocolIE-ID ::= xx3

id-StartRBIndex													ProtocolIE-ID ::= xx4

id-transmissionCombn8											ProtocolIE-ID ::= xx5

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>