3GPP TSG-RAN WG3 Meeting #120	R3-233404

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TP to MT-SDT TS 38.401) consideration on MT-SDT

Source: 	Huawei, Qualcomm Incorporated, Ericsson, China Telecom, Nokia, Nokia Shanghai Bell, Samsung, ZTE, LG Electronics, CATT, Google, Lenovo

Agenda item:	20.2

Document Type:	Other

# Introduction

This paper provides the TP to TS 38.401 BL CR of MT-SDT WI, to capture the agreements made during RAN3#120 meeting.

# TP to TS 38.401

-----------Start of the Change-----------------

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1. During the setup or modification of the bearer context as specified in 8.9.2, the gNB-CU-CP requests the gNB-CU-UP to provide MT-SDT information.

2a-0.	The gNB-CU-UP receives  DL  data for the UE in RRC Inactive on NG-U interface.

2a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If available, the gNB-CU-UP includes the MT-SDT information in the DL DATA NOTIFICATION message based on determining that DL data packets are only mapped to SDT bearers, as requested in step 1.

2b. The gNB-CU-CP receives DL NAS signalling over NGAP.

3.	After 2a or 2b, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT information may be included in the PAGING message.

4.	The gNB-DU sends the Paging message to the UE. In case the MT-SDT information is received in step 3, the gNB-DU includes the MT-SDT indicator in the Paging message.

5. If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

In case SDT procedure is initiated, the UE may indicate the RRC Resume Request is for MT-SDT, which may be without UL data.

------------End of the Change-----------------