3GPP TSG-RAN WG3 #120																		R3-233454

22nd – 26th May 2023

Incheon, KR

Agenda item:	17.2

Source:	Nokia, Nokia Shanghai Bell, CATT, Huawei, ZTE

Title:	(TP for NR NTN BL CR TS38.300) Early Data Forwarding in Xn/NG-HO with time-based trigger condition

Document for:	Approval

# 1 Introduction

This is TP to capture the RAN3 agreement in Stage-2 TS38.300:

When time-based trigger condition is used, the source NG-RAN node should consider the time indicated to the UE to decide when start the early data forwarding to the target NG-RAN node.

## Text Proposal for BL CR 38.300

##### 16.14.3.2.2	Conditional Handover

The same principle as described in 9.2.3.4 applies to NTN unless hereunder specified.

NTN supports the following additional trigger conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]:

-	The RRM measurement-based event A4;

-	A time-based trigger condition;

-	A location-based trigger condition.

A time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12].

It is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event.

When time-based trigger condition is used, the source NG-RAN node should consider the time indicated to the UE to decide when start the early data forwarding to the target NG-RAN node.