3GPP TSG-RAN WG3 Meeting #120	R3-233464

Incheon, Republic of Korea, May 22nd – 26th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0101 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for mobile TRP Location Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Xiaomi, Qualcomm Inc., CATT, Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-core, NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introducing the necessary codepoints and IEs to support signalling the mobile TRP capability, Mobile IAB-MT UE ID and location information in the TRP Information Exchange procedure and Measurement procedure to support Mobile TRP positioning options as described in TS 23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of a new 'mobile trp' codepoint in the TRP Type IE in the TRP Information IE to indicate mobile TRP capability to LMF and to trigger TS 23.273 actions. - Addition of new Mobile IAB-MT UE ID IE in the TRP Information IE, that is conditonally present if the TRP Type IE is set to 'mobile trp'. The Mobile IAB-MT UE ID IE indicates the GPSI as defined in TS 29.571. - Addition of new 'mobile trp location info' in the TRP Information Type Item IE in the TRP INFORMATION REQUEST message that RAN uses to trigger TS 23.273 actions. - Addition of Mobile TRP Location Information IE in the TRP Information IE in the TRP INFORMATION RESPONSE message that includes Mobile TRP updated location. The IE contains the Location Information, Velocity Information and the Location Time stamp IEs as defined in LPP TS 37.355. - Addition of new bit in the Measurement Characteristics Request Indicator IE to request reporting of mobile TRP location information during the measurement reporting. - Addition of the Mobile TRP Location Information IE in the TRP Measurement Result IE. - Addition of necessary references to other specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support of positioning Mobile TRP as specified in TS 23.273. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.3, 8.2.8.2, 9.1.1.4, 9.2.25, 9.2.37, 9.2.81, 9.2.X (new), 9.3.4, 9.3.5, 9.3.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR1176 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: resubmitted to RAN3#120. Rev2: Revision after CB # IAB2_Others discussion |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:"NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[4]	Void.

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[8]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[9]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[10]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.455: " Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 37.355: " Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[15]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[16]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[17]	3GPP TS 36:214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer (PHY); Measurements".

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[19]	3GPP TS 38.215: "NR; Physical layer (PHY); Measurements".

[Xx]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[Yy]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces".

-------------------------------------------Next change-------------------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

gNB: as defined in TS 38.300 [3].

NG-RAN node: as defined in TS 38.300 [3].

ng-eNB: as defined in TS 38.300 [3].

IAB-MT: as defined in TS 38.300 [3].

-------------------------------------------Next change-------------------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

CG-SDT	Configured Grant Small Data Transmission

CID	Cell-ID (positioning method)

DL-PRS	Downlink Positioning Reference Signal

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GPSI	Generic Public Subscription Identifier

LMF	Location Management Function

LPP	LTE Positioning Protocol

MSAS	Multi-functional Satellite Augmentation System

NavIC	NAVigation with Indian Constellation

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference of Arrival

posSIB	Positioning SIB

PRS	Positioning Reference Signal (for E-UTRA)

QZSS	Quasi-Zenith Satellite System

RSRP	Reference Signal Received Power

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SBAS	Space Based Augmentation System

SRS	Sounding Reference Signal

TEG	Timing Error group

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL SRS-RSRPP	UL SRS reference signal received path power

WAAS	Wide Area Augmentation System

Z-AoA	Zenith Angles of Arrival

-------------------------------------------Next change-------------------------------------------

### 8.2.8	TRP Information Exchange

#### 8.2.8.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.8.2	Successful Operation

Figure 8.2.8.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the muting information for the given PRS resource set.

If the  QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account for the given PRS resource list.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the DL PRS Resource Coordinates relative to the TRP coordinate.

If the Mobile IAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [Xx].

If the TRP Information Type Item IE is set to 'mobile trp location info', the NG-RAN node shall, if supported, derive the location of the Mobile TRP as specified in TS 23.273 [Xx] and include the Mobile TRP Location Information in the TRP INFORMATION RESPONSE message.

-------------------------------------------Next change-------------------------------------------

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type List |  | 1 |  |  |  |  |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info, mobile trp location info) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

-------------------------------------------Next change-------------------------------------------

### 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>NR CGI | M |  | 9.2.9 |  | - |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>PRS Configuration | M |  | 9.2.44 |  | - |  |
| >>SSB Information | M |  | 9.2.54 |  | - |  |
| >>SFN Initialisation Time | M |  | Relative Time 1900 9.2.36 |  | - |  |
| >>Spatial Direction Information | M |  | 9.2.45 |  | - |  |
| >>Geographical Coordinates | M |  | 9.2.46 |  | - |  |
| >>TRP type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mobile trp) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information | M |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association | M |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information | M |  | 9.2.82 |  | YES | reject |
| >>Mobile TRP Location Information | M |  | 9.2.X |  | YES | reject |
| Mobile IAB-MT UE ID | C-ifMobileTRP |  | OCTET STRING | The UE ID of the IAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [Yy] | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifMobileTRP | This IE shall be present if the TRP type IE is set to the value 'mobile trp' |

-------------------------------------------Next change-------------------------------------------

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Z-AoA | M |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA | M |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP | M |  | 9.2.72 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

-------------------------------------------Next change-------------------------------------------

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: Additional Path Power Fourth Bit: Multiple UL AoA of Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG-ID information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

-------------------------------------------Next change-------------------------------------------

### 9.2.X	Mobile TRP Location Information

This IE contains location information for one mobile TRP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Location Information | O |  | OCTET STRING | Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14] |
| Velocity Infomration | O |  | OCTET STRING | Velocity of the mobile TRP, Includes the velocityEstimate IE as defined in TS 37.355 [14] |
| Location Time stamp | O |  | Time Stamp 9.2.42 | Indicates the time when the mobile TRP location information is generated. |

-------------------------------------------Next change-------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

id-ResultEUTRA,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoPosMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxPRS-ResourceSets,

maxPRS-ResourcesPerSet,

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource,

maxnoofULAoAs,

maxNoPathExtended,

maxnoARPs,

maxnoTRPTEGs,

maxnoUETEGs,

maxFreqLayers,

maxnoPRSTRPs,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

id-Cell-ID,

id-TRPInformationTypeItem,

id-SrsFrequency,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-PRS-Resource-ID,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoA,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-ARPLocationInfo,

id-ARP-ID,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRP-Rx-TEGInformation,

id-TRPBeamAntennaInformation,

id-NR-TADV,

id-pathPower,

id-SRSPortIndex,

id-UETxTimingErrorMargin,

id-Mobile-TRP-LocationInformation,

id-Mobile-IAB-MT-UE-ID

-------------------------------------------Next change-------------------------------------------

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival-EUTRA				INTEGER (0..719),

valueTimingAdvanceType1-EUTRA			INTEGER (0..7690),

valueTimingAdvanceType2-EUTRA			INTEGER (0..7690),

resultRSRP-EUTRA						ResultRSRP-EUTRA,

resultRSRQ-EUTRA						ResultRSRQ-EUTRA,

choice-Extension			ProtocolIE-Single-Container {{ MeasuredResultsValue-ExtensionIE }}

}

MeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultSS-RSRP		CRITICALITY ignore	TYPE ResultSS-RSRP		PRESENCE mandatory	}|

{ ID id-ResultSS-RSRQ		CRITICALITY ignore	TYPE ResultSS-RSRQ		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRP		CRITICALITY ignore	TYPE ResultCSI-RSRP		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRQ		CRITICALITY ignore	TYPE ResultCSI-RSRQ		PRESENCE mandatory	}|

{ ID id-AngleOfArrivalNR	CRITICALITY ignore	TYPE UL-AoA				PRESENCE mandatory	}|

{ ID id-NR-TADV				CRITICALITY ignore	TYPE NR-TADV			PRESENCE mandatory	},

...

}

Mobile-TRP-LocationInformation ::= SEQUENCE {

location-Information			OCTET STRING	OPTIONAL,

velocity-Information			OCTET STRING	OPTIONAL,

location-time-stamp				TimeStamp	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Mobile-TRP-LocationInformation-ExtIEs} } OPTIONAL,

...

}

Mobile-TRP-LocationInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Mobile-IAB-MT-UE-ID ::= OCTET STRING

-------------------------------------------Next change-------------------------------------------

TRPInformation ::= SEQUENCE {

tRP-ID							TRP-ID,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL,

...

}

TRPInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-Mobile-IAB-MT-UE-ID			CRITICALITY reject EXTENSION Mobile-IAB-MT-UE-ID		PRESENCE optional},

--This IE shall be present if the TRP type IE is set to the value "mobile trp"

...

}

TRPInformationTypeResponseList ::= SEQUENCE (SIZE (1..maxnoTRPInfoTypes)) OF TRPInformationTypeResponseItem

TRPInformationTypeResponseItem ::= CHOICE {

pCI-NR										INTEGER  (0..1007),

cGI-NR									CGI-NR,

aRFCN										INTEGER (0..3279165),

pRSConfiguration							PRSConfiguration,

sSBinformation								SSBInfo,

sFNInitialisationTime						RelativeTime1900,

spatialDirectionInformation					SpatialDirectionInformation,

geographicalCoordinates						GeographicalCoordinates,

choice-extension							ProtocolIE-Single-Container { { TRPInformationTypeResponseItem-ExtIEs } }

}

TRPInformationTypeResponseItem-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPType						CRITICALITY reject TYPE TRPType					PRESENCE mandatory }|

{ ID id-OnDemandPRS				CRITICALITY reject TYPE OnDemandPRS-Info		PRESENCE mandatory}|

{ ID id-TRPTxTEGAssociation			CRITICALITY reject TYPE TRPTxTEGAssociation		PRESENCE mandatory}|

{ ID id-TRPBeamAntennaInformation	CRITICALITY reject TYPE TRPBeamAntennaInformation	PRESENCE mandatory }|

{ ID id-Mobile-TRP-LocationInformation	CRITICALITY reject TYPE Mobile-TRP-LocationInformation	PRESENCE mandatory },

...

}

TRPInformationTypeListTRPReq ::= SEQUENCE (SIZE(1.. maxnoTRPInfoTypes)) OF ProtocolIE-Single-Container { {TRPInformationTypeItemTRPReq} }

TRPInformationTypeItemTRPReq NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPInformationTypeItem	 CRITICALITY reject		TYPE TRPInformationTypeItem  	PRESENCE mandatory },

...

}

TRPInformationTypeItem ::= ENUMERATED {

nrPCI,

nG-RAN-CGI,

arfcn,

pRSConfig,

sSBInfo,

sFNInitTime,

spatialDirectInfo,

geoCoord,

...,

trp-type,

ondemandPRSInfo,

trpTxTeg,

beam-antenna-info,

mobile-trp-location-info

}

-------------------------------------------Next change-------------------------------------------

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSResourcetype	CRITICALITY ignore EXTENSION SRSResourcetype PRESENCE optional}|

{ ID id-ARP-ID		CRITICALITY ignore EXTENSION ARP-ID 	PRESENCE optional}|

{ ID id-LoS-NLoSInformation		CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional }|

{ ID id-Mobile-TRP-LocationInformation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE ignore },

...

}

-------------------------------------------Next change-------------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

-------------------------------------------Next change-------------------------------------------

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-OtherRATMeasurementQuantities								ProtocolIE-ID ::= 15

id-OtherRATMeasurementQuantities-Item							ProtocolIE-ID ::= 16

id-OtherRATMeasurementResult									ProtocolIE-ID ::= 17

id-WLANMeasurementQuantities									ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item								ProtocolIE-ID ::= 20

id-WLANMeasurementResult										ProtocolIE-ID ::= 21

id-TDD-Config-EUTRA-Item										ProtocolIE-ID ::= 22

id-Assistance-Information										ProtocolIE-ID ::= 23

id-Broadcast													ProtocolIE-ID ::= 24

id-AssistanceInformationFailureList								ProtocolIE-ID ::= 25

id-SRSConfiguration												ProtocolIE-ID ::= 26

id-MeasurementResult											ProtocolIE-ID ::= 27

id-TRP-ID														ProtocolIE-ID ::= 28

id-TRPInformationTypeListTRPReq									ProtocolIE-ID ::= 29

id-TRPInformationListTRPResp									ProtocolIE-ID ::= 30

id-MeasurementBeamInfoRequest									ProtocolIE-ID ::= 31

id-ResultSS-RSRP												ProtocolIE-ID ::= 32

id-ResultSS-RSRQ												ProtocolIE-ID ::= 33

id-ResultCSI-RSRP												ProtocolIE-ID ::= 34

id-ResultCSI-RSRQ												ProtocolIE-ID ::= 35

id-AngleOfArrivalNR												ProtocolIE-ID ::= 36

id-GeographicalCoordinates										ProtocolIE-ID ::= 37

id-PositioningBroadcastCells									ProtocolIE-ID ::= 38

id-LMF-Measurement-ID											ProtocolIE-ID ::= 39

id-RAN-Measurement-ID											ProtocolIE-ID ::= 40

id-TRP-MeasurementRequestList									ProtocolIE-ID ::= 41

id-TRP-MeasurementResponseList									ProtocolIE-ID ::= 42

id-TRP-MeasurementReportList									ProtocolIE-ID ::= 43

id-SRSType														ProtocolIE-ID ::= 44

id-ActivationTime												ProtocolIE-ID ::= 45

id-SRSResourceSetID												ProtocolIE-ID ::= 46

id-TRPList														ProtocolIE-ID ::= 47

id-SRSSpatialRelation											ProtocolIE-ID ::= 48

id-SystemFrameNumber											ProtocolIE-ID ::= 49

id-SlotNumber													ProtocolIE-ID ::= 50

id-SRSResourceTrigger											ProtocolIE-ID ::= 51

id-TRPMeasurementQuantities										ProtocolIE-ID ::= 52

id-AbortTransmission											ProtocolIE-ID ::= 53

id-SFNInitialisationTime										ProtocolIE-ID ::= 54

id-ResultNR														ProtocolIE-ID ::= 55

id-ResultEUTRA													ProtocolIE-ID ::= 56

id-TRPInformationTypeItem										ProtocolIE-ID ::= 57

id-CGI-NR														ProtocolIE-ID ::= 58

id-SFNInitialisationTime-NR										ProtocolIE-ID ::= 59

id-Cell-ID														ProtocolIE-ID ::= 60

id-SrsFrequency													ProtocolIE-ID ::= 61

id-TRPType														ProtocolIE-ID ::= 62

id-SRSSpatialRelationPerSRSResource								ProtocolIE-ID ::= 63

id-MeasurementPeriodicityExtended								ProtocolIE-ID ::= 64

id-PRS-Resource-ID												ProtocolIE-ID ::= 65

id-PRSTRPList													ProtocolIE-ID ::= 66

id-PRSTransmissionTRPList										ProtocolIE-ID ::= 67

id-OnDemandPRS												ProtocolIE-ID ::= 68

id-AoA-SearchWindow												ProtocolIE-ID ::= 69

id-TRP-MeasurementUpdateList									ProtocolIE-ID ::= 70

id-ZoA															ProtocolIE-ID ::= 71

id-ResponseTime													ProtocolIE-ID ::= 72

id-UEReportingInformation										ProtocolIE-ID ::= 73

id-MultipleULAoA												ProtocolIE-ID ::= 74

id-UL-SRS-RSRPP													ProtocolIE-ID ::= 75

id-SRSResourcetype												ProtocolIE-ID ::= 76

id-ExtendedAdditionalPathList									ProtocolIE-ID ::= 77

id-ARPLocationInfo												ProtocolIE-ID ::= 78

id-ARP-ID														ProtocolIE-ID ::= 79

id-LoS-NLoSInformation											ProtocolIE-ID ::= 80

id-UETxTEGAssociationList										ProtocolIE-ID ::= 81

id-NumberOfTRPRxTEG												ProtocolIE-ID ::= 82

id-NumberOfTRPRxTxTEG											ProtocolIE-ID ::= 83

id-TRPTxTEGAssociation											ProtocolIE-ID ::= 84

id-TRPTEGInformation											ProtocolIE-ID ::= 85

id-TRP-Rx-TEGInformation													ProtocolIE-ID ::= 86

id-TRP-PRS-Information-List										ProtocolIE-ID ::= 87

id-PRS-Measurements-Info-List									ProtocolIE-ID ::= 88

id-PRSConfigRequestType											ProtocolIE-ID ::= 89

id-UE-TEG-Info-Request											ProtocolIE-ID ::= 90

id-MeasurementTimeOccasion										ProtocolIE-ID ::= 91

id-MeasurementCharacteristicsRequestIndicator					ProtocolIE-ID ::= 92

id-TRPBeamAntennaInformation									ProtocolIE-ID ::= 93

id-NR-TADV														ProtocolIE-ID ::= 94

id-MeasurementAmount											ProtocolIE-ID ::= 95

id-pathPower													ProtocolIE-ID ::= 96

id-PreconfigurationResult										ProtocolIE-ID ::= 97

id-RequestType													ProtocolIE-ID ::= 98

id-UE-TEG-ReportingPeriodicity									ProtocolIE-ID ::= 99

id-SRSPortIndex													ProtocolIE-ID ::= 100

id-procedure-code-101-not-to-be-used							ProtocolIE-ID ::= 101

id-procedure-code-102-not-to-be-used							ProtocolIE-ID ::= 102

id-procedure-code-103-not-to-be-used							ProtocolIE-ID ::= 103

id-UETxTimingErrorMargin										ProtocolIE-ID ::= 104

id-MeasurementPeriodicityNR-AoA									ProtocolIE-ID ::= 105

id-SRSTransmissionStatus										ProtocolIE-ID ::= 106

id-Mobile-TRP-LocationInformation								ProtocolIE-ID ::= xx1

id-Mobile-IAB-MT-UE-ID											ProtocolIE-ID ::= xx2

END

-- ASN1STOP

-------------------------------------------End of changes-------------------------------------------