3GPP TSG-RAN WG3 Meeting #120	R3-233405

Incheon, Korea, 22 – 26 May 2023

Agenda Item:	20.2

Source:	Ericsson, China Telecom, Nokia, Nokia Shanghai Bell, Samsung, LG Electronics, CATT, Huawei

Title:	[TP for BL CR 38.423 on MT-SDT]

Document for:	Other

# 1	Introduction

This TP follows discussions in R3-233330.

# 2	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.xxx	MT-SDT Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, …) |  |
| MT-SDT Data Size | M |  | INTEGER (1…96000, …) | Indicates the total data size for DL NAS signalling and user plane data for all SDT bearers. Unit: byte. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

MT-SDT-Information ::= SEQUENCE {

mT-SDT-Indicator			MT-SDT-Indicator,

mT-SDT-DataSize 			MT-SDT-DataSize	,

iE-Extensions				ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-DataSize	::= INTEGER (1..96000, ...)

MT-SDT-Indicator ::= ENUMERATED {true, ...}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>