3GPP TSG-RAN WG3 Meeting #120	R3-232610

# Incheon, Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0931 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Burst Arrival Time semantics description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Burst Arrival Time IE reuses the same encoding as the RRC ReferenceTime IE, which is a sequence of four INTEGER values having decreasing time granularity as shown below: ReferenceTime-r16 ::= SEQUENCE { refDays-r16 INTEGER (0..72999), refSeconds-r16 INTEGER (0..86399), refMilliSeconds-r16 INTEGER (0..999), refTenNanoSeconds-r16 INTEGER (0..99999) } It is unclear what “truncated to 1 us granularity” means in the context of the above encoding structure (since e.g. the value of the IE cannot be shortened, nor can the 10ns granularity of the IE be changed). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The semantics description is reworded to: “rounded to 1us accuracy”. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR does not have impact from protocol point of view. This CR may have an impact from functional point of view because, along the current semantics, there are various interpretations (and implementations) of the current IE possible. The impact depends on which interpretation a potential implementation has chosen. The impact can be considered isolated because the change affects only the Burst Arrival Time IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Burst Arrival Time remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.133 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CR0057 TS 38.423 CR0980 TS 38.473 CR1119 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Resubmission to RAN3#119bis-e, with some cover page updates. Rev2: Resubmission of endorsed CR to RAN3#120 for agreement |  |  |  |  |  |  |  |  |

Start of change

#### 9.3.1.133	Burst Arrival Time

This IE indicates the Burst Arrival Time of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time | M |  | OCTET STRING | Encoded in the same format as the ReferenceTime IE as defined in TS 38.331 [18]. The value is provided with 1 us accuracy. |

End of change