3GPP TSG-RAN WG3 Meeting #120	R3-232773

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	14.4

Source:	Qualcomm Incorporated

Title:	SCG Selective Activation in NR-DC

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and Decision

# Introduction

In the last RAN3 meeting (RAN3 #119-bis-e [1]), there was continued progress on the most basic signaling aspects. The agreements from the last RAN3 meeting are as follows.

RAN3 #119-bis-e agreements:

RAN3 assumes that a UE can be configured to keep a conditional configuration for CPA after CPA execution. The kept CPA conditional configuration is used for subsequent CPC (but with different triggering conditions). This can be revisited based on RAN2 progress.

RAN3 should further analyze the impacts if RAN2 decides to support activation/deactivation of candidate PSCell evaluation after the first time SCG selective activation configuration.

WA: Add a new indication as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message to indicate that the request is for SCG Selective Activation. It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container).

Note: This WA may be revisited based on RAN2 progress.

For inter-SN SCG selective activation, after CPC execution, the MN needs to notify the source SN and the selected SN of the cell change. FFS how to notify the source SN and the selected SN.

Reuse the Xn-U Address Indication message and the Early Status Transfer message to support early data forwarding for SCG Selective Activation. FFS on enhancement and FFS when to use these two messages.

Solution details need to be further discussed.

In this contribution, we continue discussion on the procedures for SCG selective activation and provide some proposals for RAN3 discussion.

# Procedures and signalling aspects

We assume that the UE is in MR-DC operation when the preparation procedure is initiated. The discussion would proceed along similar lines in case the UE is in standalone mode when the preparation procedure is initiated and is thus omitted.

Assumption. We assume that the UE is in MR-DC operation when the configuration preparation procedure is initiated.

Source MN initiated inter-SN SCG selective activation

Figure 1 shows an example signalling flow for the procedure.

Steps 1-6: Source MN initiates the preparation procedure based on UE measurement reports and transmits SN Addition Request to a candidate SN including a list of all candidate PSCells, i.e., candidate PSCells belonging to all candidate SNs, and associated UE measurements.

RAN2 #121 agreement:

For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration).

It was agreed in RAN2 #121 [2] that source MN should provide the reference SCG configuration to all candidate SNs. The candidate SNs may generate delta target SCG configurations with respect to the reference SCG configuration for the candidate target PSCells.

Proposal 1. The source MN provides the reference SCG configuration to all candidate SNs in SN Addition Req messages. The reference SCG configuration is included in an inter-node RRC container.

Proposal 2. The candidate SNs respond by including in SN Addition Req Ack the target SCG configurations for the prepared candidate target PSCells that are delta configurations with respect to the reference SCG configuration. The target SCG configurations are included in an inter-node RRC container.

MN prepares the initial RRC reconfiguration message containing the configuration for SCG selective activation.

Proposal 3. Upon receiving the RRCReconfigurationComplete from the UE, using Xn-U Address Indication message MN provides at least the data forwarding addresses for the initial CPC to the source SN.

Proposal 4. Upon receiving the Xn-U Address Indication message, source SN may initiate Early Data Forwarding corresponding to the initial CPC. The Early Status Transfer message can be reused for this purpose.

Steps 7-12: Upon CPC triggering for a candidate target PSCell, UE may keep the source configuration as well as the conditional configurations. This seems to be a basic mechanism to achieve the goal of this objective in the WI and was agreed as a Working Assumption in a previous RAN3 meeting (RAN3 #117-e [3]) and as an agreement in a previous RAN2 meeting (RAN2 #119-bis-e [4]).

RAN2 #119-bis-e agreements:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

a. Step 1: When the execution condition of a CPC candidate PSCell is met, a UE performs the execution of CPC towards this candidate PSCell.

b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PSCells.

RAN3 #117-e agreement:

WA: A primary focus of the objective is to enable subsequent cell changes by keeping conditional reconfigurations after a cell change. RAN3 to pursue study of the Xn/F1 signaling changes required to support this objective.

Upon receiving SN Release Request from the MN:

- Source SN stops transmitting user data to the UE.

- Source SN keeps the data forwarding addresses provided earlier by the source MN, since the UE may come back to it later. The Source SN may also keep the data forwarding addresses based on a received network indication.

After a CPC is triggered, the source MN notifies the selected SN and the serving SN of the cell change.

Proposal 5. After a CPC is triggered, source MN notifies the selected SN of the cell change using the SN Reconfiguration Complete message (similar as in Rel-17 inter-SN CPC).

RAN2 #119-bis-e agreement:

Baseline procedure to support subsequent secondary cell group change:

b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PSCells.

Step 14: After accessing the candidate target PSCell, UE initiates evaluation for the subsequent CPC. This was also agreed in a previous RAN2 meeting (RAN2 #119-bis-e [4]).

Figure 1: MN initiated inter-SN SCG selective activation

Source SN initiated inter-SN SCG selective activation

For the source SN initiated inter-SN procedure, we think that a new SCG Selective Activation indicator should be included in the initial message SN Change Required.

Proposal 6. Introduce a new SCG Selective Activation indicator in SN Change Required.

# 3 Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following observations and proposals.

Source MN initiated inter-SN SCG selective activation

Proposal 1. The source MN provides the reference SCG configuration to all candidate SNs in SN Addition Req messages. The reference SCG configuration is included in an inter-node RRC container.

Proposal 2. The candidate SNs respond by including in SN Addition Req Ack the target SCG configurations for the prepared candidate target PSCells that are delta configurations with respect to the reference SCG configuration. The target SCG configurations are included in an inter-node RRC container.

Proposal 3. Upon receiving the RRCReconfigurationComplete from the UE, using Xn-U Address Indication message MN provides at least the data forwarding addresses for the initial CPC to the source SN.

Proposal 4. Upon receiving the Xn-U Address Indication message, source SN may initiate Early Data Forwarding corresponding to the initial CPC. The Early Status Transfer message can be reused for this purpose.

Proposal 5. After a CPC is triggered, source MN notifies the selected SN of the cell change using the SN Reconfiguration Complete message (similar as in Rel-17 inter-SN CPC).

Source SN initiated inter-SN SCG selective activation

Proposal 6. Introduce a new SCG Selective Activation indicator in SN Change Required.

# References

[1] Chair notes, RAN3 #119-bis-e meeting.

[2] Chair notes, RAN2 #121 meeting.

[3] Chair notes, RAN3 #117-e meeting.

[4] Chair notes, RAN2 #119-bis-e meeting.