3GPP TSG-RAN WG3 Meeting #120	R3-232822

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TP for MDT BLCR for TS 37.320): MDT support in NPN

Source: 	Huawei, Orange, China Telecom, CMCC

Agenda item:	10.2.4

Document Type:	Other

1. Introduction

This document contains a stage 2 TP for TS 37.320 to introduce the MDT in NPN function.

Annex – TP For MDT BLCR for TS 37.320

5.4.x	Support of NPN

MDT is supported in PNI-NPN and SNPN.

For signalling based MDT in PNI-NPN, the AMF indicates the PNI-NPN Area Scope both inside and outside the current Area Scope in MDT Configuration.

For signalling based MDT in SNPN, the AMF indicates the SNPN Area Scope inside the current Area Scope in MDT Configuration.