3GPP TSG-RAN WG3 Meeting #120	R3-232823

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TPs for SON BLCR for TS 38.423 and TS 38.473): SON for NR-U

Source: 	Huawei

Agenda item:	10.2.5

Document Type:	Discussion

1. Introduction

In the last RAN3 #119b meeting [1], the MRO and MLB for NR-U are further discussed, while there are still the following remaining issues:

FFS on whether the EDT UL sent over Xn can also consider the EDT UL reported by UEs. Continue to discuss on the addition to RLF report of information determining the outcome of the channel access procedure, with focus on EDT UL. FFS on whether to enable a UE-based solution or a NW-based solution to inform the source gNB of a handover about DL LBT issues occurring at the target gNB, during a handover execution. Continue to discuss on the need for reporting EDT UL over F1. Continue to discuss on additional load metrics for NR-U.

In this paper, we provide our opinions about the remaining issues in NR-U.

2. Discussion

RLF report enhancement:

According to clause 4.2.3 of TS 37.213, the EDT used by UE, is a self-determined value not larger than the value configured by network, or a default value related to the single channel bandwidth. Considering the consistent LBT failure is counted and recorded per BWP, we think using average EDT value corresponding to each BWP, is reasonable. However, since the EDT will not be frequently reconfigured, an overall average EDT value during the UL transmission is also acceptable.

The UL average EDT should be included in the RLF report.

As discussed above, it is obviously that the network side configures the UE with LBT configurations and thus is aware of the detail configurations. However, the network side cannot predict whether an RLF occurred in the UE. Before receiving an RLF report, if the network has changed the LBT configurations, the network may not be able to associate the RLF report and the corresponding LBT configurations.

The RLF report should include the LBT configurations.

The waiting time in UL due to LBT is not clear enough. For example, the UE may perform RA procedures among different BWPs. In this case, a single waiting time cannot reflect the detail LBT situations for each BWP. We thus prefer the average sensing time. The average sensing time represents the average time from when the UE starts to sense the channel for transmission purpose, to the time the UE successfully occupies the channel or consistent LBT failures are triggered. A longer average sensing time indicates the higher load of the specific transmission channel.

The RLF report should include the average sensing time.

MLB:

For the EDT UL sent over XN interface, it has been agreed that the EDT UL value in resource status should at least reflect the values configured for the UEs. As specified in TS 37.213, the actually applied EDT UL value, is based on implementation and could be less than the NW configured value.

A UE accessing a channel on which UL transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold XThresh_max.

Therefore, for more accurate resource report, the EDT UL value should be based on the reported value from UE. To keep alignment with the achieved agreements, we propose that an extra EDT UL from UE should be included in the resource status report, as in the attached annex.

For the CU-DU split structure, the DU should be able to report EDT UL over F1, and thus the CU can further exchange the value with neighbour gNBs. In similar to the Xn interface, the EDT UL sent over F1 can also consider the EDT UL reported by UEs and an extra EDT UL from UE should be included in the resource status report.

The EDT UL sent over Xn and F1 can also consider the EDT UL reported by UEs, and an extra EDT UL from UE should be included in the resource status report.

Another remaining issue is to evaluate whether more additional load metrics can be introduced for MLB in NR-U. In our opinion, the radio resource status and CAC per NR-U channel should be introduced, which can further reflect the load in each utilized NR-U channel and help to make rational MLB strategies. The related specification modifications are attached in the annex, including Xn and F1 interfaces.

RAN3 to introduce per NR-U channel radio resource status and CAC as load metrics for MLB in NR-U.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- The UL average EDT should be included in the RLF report.

- The RLF report should include the LBT configurations.

- The RLF report should include the average sensing time.

- The EDT UL sent over Xn and F1 can also consider the EDT UL reported by UEs, and an extra EDT UL from UE should be included in the resource status report.

- RAN3 to introduce per NR-U channel radio resource status and CAC as load metrics for MLB in NR-U.

5. Reference

- RAN3_119bis-e_agenda_20230426_EOM1.

Annex – TP of TS 38.423

9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | – |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Average ED Threshold used for UL channel sensing as available at the gNB. Value is in dBm. | YES | ignore |
| >>>>Applied Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Average actually applied ED Threshold used for UL channel sensing as available at the gNB. Value is in dBm. | YES | ignore |
| >>>>Radio Resource Status of NR-U | O |  | 9.2.2.50 | Radio Resource Status per NR-U channel. |  |  |
| >>>>Composite Available Capacity Group of NR-U | O |  | 9.2.2.51 | Composite Available Capacity Group per NR-U channel. |  |  |

End of Changes

Annex – TP of TS 38.473

9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 corresponds to the duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Average ED Threshold used for UL channel sensing. Value is in dBm. |  |  |
| >>>>Applied Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | Average actually applied ED Threshold used for UL channel sensing as available at the gNB. Value is in dBm. | YES | ignore |
| >>>>Radio Resource Status of NR-U | M |  | 9.3.1.129 | Radio Resource Status per NR-U channel. |  |  |
| >>>>Composite Available Capacity Group of NR-U | O |  | 9.3.1.130 | Composite Available Capacity Group per NR-U channel. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

End of Changes