3GPP TSG-RAN WG3 Meeting #120	R3-232837

Incheon, KR, 22 May – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0718 | rev | 1 | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Paging Priority Indicator in QoS Flow Level QoS Parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom, Orange, BT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Paging Priority Indicator (PPI) IE is included in the 9.3.1.26 QoS Flow Level QoS Parameters IE (see the agreed CR in R3-193005), and in the DL DATA NOTIFICATION message respectively. Its IE type is given as INTEGER (0.. 7, …). However, in ASN.1, the paging-Policy-Indicator is defined as INTERGET (1..8) in the QoSFlowLevelQoSParameters. While it is correctly defined for the DLDataNotification message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To make ASN.1 backwards compatible, - update the IE type of the Paging Priority Indicator (PPI) IE to be the same as the definition in ASN.1; - add a new PPI IE referring to the Paging Priority Indicator (PPI) Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the Paging Priority Indicator in the QoS Flow Level QoS Parameters IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For Paging Priority Indicator (PPI) IE in QoS Flow Level QoS Parameters, the value 0 cannot be delivered, and value 8 may be erroneously used. Not aligned between the Tabular and the ASN.1 for the Paging Priority Indicator (PPI) IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.26, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-231748 Rev1: R3-232837 Minor update the coverage page. |  |  |  |  |  |  |  |  |

Change Begins

< Unchanged Text Omitted >

#### 9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name |  |  | Presence |  | Range |  | IE type and reference | Semantics description |  |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics |  |  | M |  |  |  |  |  |  |  | - |  |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor |  |  | M |  |  |  | 9.3.1.27 |  |  |  | - |  |  |
| >Dynamic 5QI |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor |  |  | M |  |  |  | 9.3.1.28 |  |  |  | - |  |  |
| NG-RAN Allocation and Retention Priority |  |  | M |  |  |  | 9.3.1.29 |  |  |  | - |  |  |
| GBR QoS Flow Information |  |  | O |  |  |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows and is ignored otherwise. |  |  | - |  |  |
| Reflective QoS Attribute |  |  | O |  |  |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and is ignored otherwise. |  |  | - |  |  |
| Additional QoS Flow Information |  |  | O |  |  |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. |  |  | - |  |  |
| Paging Priority Indicator (PPI) |  |  | O |  |  |  | INTEGER (1.. 8, …) | This IE indicates the paging policy differentiation in TS 23.501 [20]. This IE is ignored if the New Paging Priority Indicator (PPI) IE is present. |  |  | - |  |  |
| RDI |  |  | O |  |  |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. |  |  | - |  |  |
| New Paging Priority Indicator (PPI) | O |  |  | 9.3.1.55 |  |  |  |  | YES | ignore |  |

<For Information Only>

#### 9.3.1.55	Paging Priority Indicator (PPI)

The Paging Policy Indicator is used for paging policy differentiation (see details in TS 23.501 [20]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PPI | M |  | INTEGER (0.. 7, …) |  |

<Unchanged Text Omitted>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-PDCP-StatusReportIndication,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-SecurityIndicationModify,

id-new-paging-Policy-Indicator,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

<Unchanged Text Omitted>

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-new-paging-Policy-Indicator	CRITICALITY ignore	EXTENSION PPI 	PRESENCE optional},

...

}

<Unchanged Text Omitted>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-PDCP-StatusReportIndication									ProtocolIE-ID ::= 81

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= 136

id-SecurityIndicationModify										ProtocolIE-ID ::= 143

id-new-paging-Policy-Indicator									ProtocolIE-ID ::= aaa

Change Ends