3GPP TSG-RAN WG3 Meeting #120                                                      	R3-232855

Incheon, May 22nd – May 26th, 2023

Agenda item:	26.3

Source:	Qualcomm Incorporated

Title:	Supporting Partially Allowed NSSAI and Alternative NSSAI

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss how to support the following objectives for eNS_Ph3-NR-Core:

Evaluate the impact, if any, on RAN to support Network Slice Service continuity scenario. [RAN3]

Evaluate the impact, and specify the solution, if needed, to support Partially Allowed NSSAI in RRC_Connected Mode. [RAN3]

# Discussion

  - Partially Allowed NSSAI

SA2 has sent a reply LS to RAN3 on Partially Allowed NSSAI in S2-2306254.

| RAN3 question | SA2 reply and observations |
| --- | --- |
| - Whether the Target NSSAI and Partially Allowed NSSAI could be signalled simultaneously to the NG-RAN node? And in such case, what is the expected behaviour at the NG-RAN node (also considering the Allowed NSSAI)? | SA2 reply: The Target NSSAI could be signalled simultaneously with the Partially Allowed NSSAI to the NG-RAN node. If received the NG-RAN would use the Target NSSAI to steer the UE to a cell supporting the S-NSSAIs in the Target NSSAI also taking any associated RFSP into account. Observation 1: Based on SA2 reply LS, the Target NSSAI could be signalled simultaneously with the Partially Allowed NSSAI to the NG-RAN node. Hence there are no issues in supporting both Partially Allowed NSSAI and Target NSSAI Further there were some proposals in a previous meeting to extend the existing IE for Allowed NSSAI for the purpose of Partially Allowed NSSAI as well. But in our view, that is not possible because Allowed NSSAI should only include those NSSAIs that are allowed in the whole RA and can’t be dynamically updated. Also, this is the current definition of Allowed NSSAI: Allowed NSSAI: Indicating the S-NSSAIs values the UE could use in the Serving PLMN in the current Registration Area. Observation 2: Allowed NSSAI can’t be repurposed for supporting Partially Allowed NSSAI because Allowed NSSAI should only include those NSSAIs that are allowed in the whole RA and can’t be dynamically updated It is therefore most suitable to introduce a new IE for supporting Partially Allowed NSSAI and we therefore have the following proposal: Proposal 1: Introduce a new IE for supporting Partially Allowed NSSAI in NGAP Proposal 2: AMF sends the “Partially Allowed S-NSSAI” to NG-RAN in the same messages as the Allowed NSSAI is sent i.e., in the following messages: - INITIAL CONTEXT SETUP REQUEST - CONNECTION ESTABLISHMENT INDICATION - AMF CP RELOCATION INDICATION - UE INFORMATION TRANSFER - HANDOVER REQUEST - PATH SWITCH REQUEST ACKNOWLEDGE - INITIAL UE MESSAGE - DOWNLINK NAS TRANSPORT - REROUTE NAS REQUEST |
| - Whether and how the Partially Allowed NSSAI would work together with the slice-based cell reselection? | SA2 Reply: It is expected that the Partially Allowed NSSAI is indeed given as input into the cell reselection algorithm. The current text in TS 23.501 reads (the red text is the new text added by SA#99 approved CRs): “The UE NAS provides to the UE AS the NSAG Information as received from the AMF and the S-NSSAIs in the Allowed NSSAI and any Partially Allowed NSSAI as input to cell reselection, except when the UE intends to register with a new (including any S-NSSAIs rejected partially in the RA) set of S-NSSAIs with a Requested NSSAI different from the current Allowed NSSAI, and any Partially Allowed NSSAI, in which case the UE NAS provides to the UE AS layer the NSAG Information as received from the AMF and the S-NSSAIs in the Requested NSSAI, and this may trigger a cell reselection, before sending the Registration Request including the new Requested NSSAI.” Observation 3: As per SA2 reply LS, Partially Allowed NSSAI needs to be considered as an input into the cell reselection algorithms. This means that there might be impacts to RAN2 specifications (e.g., TS 38.304) and hence this has to be considered as a new objective in the WID. But there are no RAN3 impacts for this objective. Proposal 3: There are no RAN3 impacts for taking Partially Allowed NSSAI into account during cell reselection. There might be RAN2 specification impacts (e.g., TS 38.304). |
| - Whether the Partially Allowed NSSAI would be associated with its own RFSP and if yes, how would this relate to existing information signalled today? How would the RFSP(s) related to other information, e.g., Allowed NSSAI signaled today, relate to the Partially Allowed NSSAI? | SA2 Reply: It is expected that the AMF provides the RAN with a single RFSP index, taking the Allowed and the Partially Allowed NSSAI into account. The RFSP is handled like any RFSP signalled to the NG-RAN from AMF. It logically relates to both the Allowed NSSAI and Partially allowed NSSAI as the current text in TS 23.501 reads as follows (the red text is the new text added by SA#99 approved CRs): “In order to enable UE idle mode mobility control and priority-based reselection mechanism considering availability of Network Slices at the network and the Network Slices allowed for a UE, an RFSP is derived as described in clause 5.3.4.3, considering also the Allowed NSSAI, and any Partially Allowed NSSAI for the UE”. Observation 4: Based on SA2 reply LS, AMF will only provide a single RFSP index associated to each S-NSSAI and it doesn’t matter where this S-NSSAI is signalled (whether in Allowed NSSAI or Partially Allowed NSSAI or Target NSSAI). And therefore, there is no “RFSP conflict”. Proposal 4: RAN3 should discuss whether the currently signaled IE for RFSP index in NGAP can be reused to signal the RFSP for Partially Allowed NSSAI or whether a new IE is needed. |
| - How does the Partially Allowed NSSAI work with respect to UE-Slice-MBR? | SA2 Reply: SA2 expects to have UE-Slice-MBR information stored in the UE context in the RAN also for the S-NSSAIs of the Partially Allowed NSSAI. UE-Slice-MBR is applied for applicable S-NSSAIs when there are active PDU sessions for the S-NSSAI and it works in the same way as for S-NSSAIs in the Allowed NSSAI. |

  - Alternative NSSAI

SA2 considered different scenarios where network slice service continuity might need to be ensured e.g.,

- Network slice is overloaded in NG-RAN

- Network slice is overloaded in the target CN

- Network slice is undergoing planned maintenance in CN

- Network performance of the network slice cannot meet the SLA

- Network slice is not supported in the target RAN node

- Network slice is not supported in the target CN

In some of the above scenarios, to ensure network slice service continuity, SA2 determined that it might be useful to have an “Alternative S-NSSAI” to act as backup and provide network slice service continuity without the need of PDU session re-establishment.

Observation 5: In order to ensure network slice service continuity in overload scenarios, SA2 concluded that it might be useful to have an “Alternative S-NSSAI” to act as backup without the need of PDU session re-establishment.

We therefore make the following proposals:

Proposal 5: In order to support S-NSSAI replacement for an existing PDU session without PDU session re-establishment, AMF should indicate an “Alternative S-NSSAI” to NG-RAN node during PDU session setup/modification and initial context setup

Proposal 6: During handover or Retrieve UE context, source NG-RAN node should send the “Alternative S-NSSAI” to target NG-RAN node

# Conclusion

Partially Allowed NSSAI

Observation 1: Based on SA2 reply LS, the Target NSSAI could be signalled simultaneously with the Partially Allowed NSSAI to the NG-RAN node. Hence there are no issues in supporting both Partially Allowed NSSAI and Target NSSAI

Observation 2: Allowed NSSAI can’t be repurposed for supporting Partially Allowed NSSAI because Allowed NSSAI should only include those NSSAIs that are allowed in the whole RA and can’t be dynamically updated

Proposal 1: Introduce a new IE for supporting Partially Allowed NSSAI in NGAP

Proposal 2: AMF sends the “Partially Allowed S-NSSAI” to NG-RAN in the same messages as the Allowed NSSAI is sent i.e., in the following messages:

- INITIAL CONTEXT SETUP REQUEST

- CONNECTION ESTABLISHMENT INDICATION

- AMF CP RELOCATION INDICATION

- UE INFORMATION TRANSFER

- HANDOVER REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

- INITIAL UE MESSAGE

- DOWNLINK NAS TRANSPORT

- REROUTE NAS REQUEST

Observation 3: As per SA2 reply LS, Partially Allowed NSSAI needs to be considered as an input into the cell reselection algorithms.

Proposal 3: There are no RAN3 impacts for taking Partially Allowed NSSAI into account during cell reselection. There might be RAN2 specification impacts (e.g., TS 38.304).

Observation 4: Based on SA2 reply LS, AMF will only provide a single RFSP index associated to each S-NSSAI and it doesn’t matter where this S-NSSAI is signalled (whether in Allowed NSSAI or Partially Allowed NSSAI or Target NSSAI). And therefore, there is no “RFSP conflict”.

Proposal 4: RAN3 should discuss whether the currently signalled IE for RFSP index in NGAP can be reused to signal the RFSP for Partially Allowed NSSAI or whether a new IE is needed.

Alternative NSSAI

Observation 5: In order to ensure network slice service continuity in overload scenarios, SA2 concluded that it might be useful to have an “Alternative S-NSSAI” to act as backup without the need of PDU session re-establishment.

Proposal 5: In order to support S-NSSAI replacement for an existing PDU session without PDU session re-establishment, AMF should indicate an “Alternative S-NSSAI” to NG-RAN node during PDU session setup/modification and initial context setup

Proposal 6: During handover or Retrieve UE context, source NG-RAN node should send the “Alternative S-NSSAI” to target NG-RAN node

# 4. References