3GPP TSG-RAN WG3 Meeting #120	R3-232996

Incheon, Republic of Korea, 22nd – 26th May 2023

Title: 	(Draft LS out) Discussion on Sidelink & Others topics

Source: 	Huawei

Agenda item:	23.2

Document Type:	Other

# 1. Introduction

According to the discussion during RAN3 #119bis, the work plan for RAN3 #120 is as follows [1]:

Focus RAN3#120 on following topics: - SL-Pos: authorizations - SL-Pos QoS Parameters, if possible - LPHAP: SRS Conf validity area - LPHAP vs SDT Pos - LMF-based Integrity

This paper provides our thinking on the potential enhancements for RAN3 based on the above progress. The document focus on slidelink, Integrity and also include the progress of other group on Carrier Phase Positioning.

# 2. Discussion

## 2.1 Ranging/Sidelink Positioning Service Authorization

A LS is received from SA2 on Ranging/SL Positioning authorization with the following information [2]:

SA2 has developed the solution on Authorisation and policy/parameter provisioning to NG-RAN, in which the "Ranging/Sidelink Positioning authorised" information and the RSPP transport QoS parameters shall be provided to the NG-RAN node for scheduled resource allocation mode resource management. SA2 would further check with RAN3 about the following question: Whether Ranging/SL Positioning QoS parameters as defined in clause 5.7.2 of TS 23.586 also need to be provided to NG-RAN?

### 2.1.1 UE Role and PC5 QoS Parameters

According to TS 23.586 [3], "Ranging/SL positioning services authorised" information may also include the authorized UE roles, i.e. Located UE, Client UE and Server UE. RAN3 should discuss whether and how, if supported, to differentiate UE roles in the authorization:

- If the UE is authorised to use Ranging/SL positioning services, then the AMF shall include in a NGAP message sent to NG-RAN: - "Ranging/SL positioning services authorised" information, including one or more of the following: - whether the UE is authorized to use Ranging/SL Positioning over PC5; - whether the UE is authorized to act as a Located UE; - whether the UE is authorized to act as a SL Positioning Client UE; - whether the UE is authorized to act as a SL Positioning Server UE. Editor's note: "Ranging/SL positioning services authorised" information needs the coordination with RAN WGs.

The issue for us is whether the UE roles affect the PC5 radio resource control in NG-RAN.

Following definition is capture from TS 23.586:

Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning.

SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it.

NOTE 2:	The SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result.

SL Positioning Server UE: A UE offering method determination, assistant data distribution and/or location calculation functionalities for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to determine Ranging/SL Position method, distribute assistant data and calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if any of the functionalities is supported.

We do understand that a Located UE is anchor UE with known location, and server UE can be anchor UE or target UE. From RAN perspective, it seems those UE roles do not cause any limitations or impacts on the resource allocation for the UE. The Client UE, according to the definition, is used to send service request and does not have to support Ranging/Sidelink positioning capability. This means the AMF does not need to send Ranging/Sidelink positioning service authorization to NG-RAN for client UE because the authorization is used for scheme1 resource allocation for the transmission of SLPP or SL-PRS. Therefore, it is suggested not to provide the UE roles to NG-RAN.

Observation 1: The listed UE roles do not impact the resource allocation for Ranging/Sidelink positioning.

Proposal 1: It is suggested not to provide the UE roles to NG-RAN in authorization information.

In addition to the “authorized” information, SA2 has agreed that the RSPP transport QoS parameters shall also be provided to the NG-RAN node. Similarly, it is proposed to add RSPP transport QoS parameters over NGAP and XnAP in the following messages:

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

-  HANDOVER REQUEST

-  PATH SWITCH REQUEST ACKNOWLEDGE

XnAP:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

The details should wait for SA2 progress.

Proposal 2: It is proposed to add RSPP transport QoS parameters over NGAP and XnAP in the following messages. Wait for SA2 progress on the details.

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

-  HANDOVER REQUEST

-  PATH SWITCH REQUEST ACKNOWLEDGE

XnAP:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

### 2.1.2 Ranging/SL Positioning QoS Parameters

RAN3 is asked whether the Ranging/SL Positioning QoS parameters also need to be provided to NG-RAN. As following description captured from TS 23.586, the Ranging/SL Positioning QoS parameters may include: attributes defined for LCS QoS, accuracy of horizontal/vertical distance and direction for ranging, and the range of the applicability:

### 5.7.2 Handling of Ranging/SL Positioning QoS Ranging/SL Positioning QoS requirement may be provided in the Ranging/SL Positioning service request generated at the application layer, and is provided from the application layer to the Ranging/SL Positioning layer. Ranging/SL Positioning QoS information contains attributes defined in clause 4.1b of TS 23.273 [8] with the following additions: - The accuracy attribute also includes the accuracy of horizontal/vertical distance and the accuracy of horizontal/vertical direction; - Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5. Editor's note: Other QoS attributes are FFS, and will be aligned with RAN WGs.

From our understanding, the Ranging/SL Positioning QoS parameters should be different with LCS QoS in Uu-based positioning, which is not provided to NG-RAN. These parameters are stored in the UDM and the motivation is to avoid excessive request for SL-PRS resources from the UE. However, it should be noticed that the gNB may have no idea how to interpret the QoS information. For example, the accuracy is dependent on the positioning method, which is transparent to the gNB. With different method, the requirements for SL-PRS resource can be different. Therefore, it is proposed that the Ranging/SL Positioning QoS parameters are not provided to NG-RAN node with authorization.

Observation 2: The gNB may have no idea how to interpret the QoS information because the accuracy is dependent on the positioning method.

Propose 3: The Ranging/SL Positioning QoS parameters are not provided to NG-RAN node with authorization.

### 2.1.3 Reply LS to SA2

The reply LS should be sent to SA2 in response of [2]. Based on the above discussion, the following is proposed:

Propose 4: Reply SA2 that the UE role and Ranging/SL Positioning QoS parameters are not provided to NG-RAN node with authorization from RAN3 perspective.

## 2.2 Integrity

During last meeting, RAN2 made the following working assumption. According to the assumption and agreement, the

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Agreement:

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

Last meeting several proposals indicates possible report to the LMF, however other clarified that no specification impact would be expected. We in line with the RAN2 proposal and do not see specification impact related to the LMF based Integrity topic.

Proposal 5:  RAN3 confirms the RAN2 WA, there should be no specification impact.

## 2.3 Carrier Phase Positioning

The following agreements have been made by RAN1 during the previous meetings:

Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting.

RAN has agreed to introduce new measurements UL RSCP for UL CPP positioning. According to the agreements, the UL RSCP can be reported by the TRP together with RTOA measurements and/or gNB Rx-Tx time difference measurements to the LMF. It is proposed to enhance the TRP Measurement Result in TS 38.455 and TS 48.473 to capture the carrier phase measurements to support UL CPP. Since RAN1 is still discussing the details, RAN3 can wait for more RAN progress.

Proposal 6: It is proposed to enhance the TRP Measurement Result in TS 38.455 and TS 38.473 to capture the carrier phase measurements to support UL CPP. Wait for RAN1 progress.

# 4. Conclusion

Observation 1: The listed UE roles do not impact the resource allocation for Ranging/Sidelink positioning.

Proposal 1: It is suggested not to provide the UE roles to NG-RAN in authorization information.

Proposal 2: It is proposed to add RSPP transport QoS parameters over NGAP and XnAP in the following messages. Wait for SA2 progress on the details.

Observation 2: The gNB may have no idea how to interpret the QoS information because the accuracy is dependent on the positioning method.

Propose 3: The Ranging/SL Positioning QoS parameters are not provided to NG-RAN node with authorization.

Propose 4: Reply SA2 that the UE role and Ranging/SL Positioning QoS parameters are not provided to NG-RAN node with authorization from RAN3 perspective.

Proposal 5:  RAN3 confirms the RAN2 WA, there should be no specification impact.

Proposal 6: It is proposed to enhance the TRP Measurement Result in TS 38.455 and TS 38.473 to capture the carrier phase measurements to support UL CPP. Wait for RAN1 progress.

# 5. Reference

- RAN3-119bis Chair’s notes

- S2-2305734 LS on Authorization and Provisioning for Ranging/SL positioning service

- TS 23.586 V0.3.0 Architectural Enhancements to support Ranging based services and Sidelink Positioning