3GPP TSG-RAN WG3 Meeting #120	R3-233040

Incheon, South Korea, 22th – 26th May, 2023

Agenda Item:	13.2

Source: 	Lenovo

Title: 	Discussion on mobile IAB-node inter-donor topology adaptation

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, we continue to discuss the general principles of mobile IAB-node inter-donor topology adaptation, including consecutive IAB-MT handover, IAB-DU migration and how to avoid the scenario without IP connectivity.

- Discussion

Consecutive IAB-MT handover

RAN3 118: The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed. The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID. RAN3 119bis-e: For consecutive partial migration, after the mIAB-MT HO is completed, the mIAB-DU’s CU sends the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the mIAB-MT’s target CU, including the UE XnAP ID assigned to the mIAB-MT by the mIAB-MT’s target CU as the “Non-F1-Terminating IAB-donor UE XnAP ID”.

As in Figure 1, to realize consecutive partial migration, source IAB-donor A of IAB-MT HO needs to inform F1-terminating IAB-donor C that gNB ID of target IAB-donor B for the IAB-MT HO. Then the target IAB-donor B and F1-ternating IAB-donor C can perform XnAP IAB TMM procedure for the mobile IAB-node4 to transport the F1 traffic via the topology of target IAB-donor B. In addition, a UE XnAP ID also needs to be informed from the source IAB-donor A to the F1-terminating IAB-donor C to identify the mobile IAB-node4. As the F1-terminating IAB-donor C maintain the UE XnAP ID allocated by both the source IAB-donor A and F1-terminating IAB-donor C, therefore, the UE XnAP ID allocated by either source IAB-donor A or F1-terminating IAB-donor C is feasible to identify the mobile IAB-MT.

Proposal 1: In case of consecutive partial migration, UE XnAP ID allocated by source IAB-donor or F1-terminating IAB-donor for mobile IAB-node is informed from the source IAB-donor to the F1-terminating IAB-donor to identify the mobile IAB-MT’s Handover.

Figure 1. Example of consecutive IAB-MT handover

As in the IAB TMM REQUEST message, both F1-Terminating IAB-donor UE XnAP ID and Non-F1-Terminating IAB-donor UE XnAP ID are mandatory. If F1-terminating IAB-donor C initiates the IAB TMM procedure to target IAB-donor B, the F1-terminating IAB-donor C should have the UE XnAP ID assigned by the target IAB-donor B in advance. Then, we have two following options for the UE XnAP ID assigned by the target IAB-donor B.

- Option A: Reuse the UE XnAP ID allocated by target IAB-donor B over the Xn interface between the source IAB-donor A and the target IAB-donor B

- Option A: A new UE XnAP ID allocated by target IAB-donor B over the Xn interface between the F1-terminating IAB-donor C and the target IAB-donor B

For Option A, the UE XnAP ID over the Xn interface between the IAB-donor A and the IAB-donor B may be not feasible to be reused because the UE XnAP ID may be already allocated to another UE over the Xn interface between IAB-donor B and IAB-donor C, which may lead to unexpected inconsistent problem when the IAB-donor B receives the IAB TMM REQUEST message including a different peer UE XnAP ID.

Therefore, F1-terminating IAB-donor C needs to be indicated from the source IAB-donor A with a new UE XnAP ID allocated by target IAB-donor B over the Xn interface between the F1-terminating IAB-donor C and the target IAB-donor B, and the source IAB-donor A may need to request the UE XnAP ID from the target IAB-donor B.

Proposal 2: In case of consecutive partial migration, F1-terminating IAB-donor needs to be indicated from source IAB-donor with a UE XnAP ID allocated by target IAB-donor over the Xn interface between the F1-terminating IAB-donor and the target IAB-donor.

Proposal 3: In case of consecutive partial migration, source IAB-donor to request from target IAB-donor a UE XnAP ID allocated by the target IAB-donor over the Xn interface between the F1-terminating IAB-donor and the target IAB-donor.

IAB-DU migration

As shown in Figure 2, before IAB-DU migration, IAB-MT of IAB-node connects to IAB-donor C and IAB-DU of the IAB-node connects to IAB-donor A. And IAB-DU performs migration from source IAB-donor A to target IAB-donor B.

Figure 2. Example of IAB-DU migration

RAN3 119: In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU. RAN3 119bis-e: Down select between the following two options for providing the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU to mIAB-DU’s target CU: - Option A: XnAP signalling from the mIAB-DU’s source CU. - Option B: F1AP signalling from the target logical mIAB-DU. For Option B, discuss whether and how the mIAB-DU can obtain the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU. The “Non-F1-Terminating IAB-donor UE XnAP ID” in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message sent from mIAB-DU’s target CU to the mIAB-MT’s CU is generated by the mIAB-MT’s CU. To be continued: Discuss whether the mIAB-DU’s CU is allowed to generate an XnAP UE ID for an mIAB-MT even if it has never terminated the RRC connection of the mIAB-MT.

Different to the IAB-MT handover, source F1-terminating IAB-donor doesn’t need to initiate a procedure to the target F1-terminating IAB-donor for the IAB-DU migration. In addition, the IAB-DU migration may be triggered by the IAB-node itself based on OAM or pre-configuration, and the source F1-terminating IAB-donor may not have the information for the target F1-terminating IAB-donor in this case. Therefore, both gNB ID and UE XnAP ID need to be informed by IAB-node and these IDs can be included in F1 setup request message to target F1-terminating IAB-donor-CU. And the IAB-node can obtain these IDs from the RRC-terminating IAB-donor via the enhancement RRC messages.

As for the UE XnAP ID, similar to principle analyzed in above consecutive IAB-MT handover, it’s a UE XnAP ID allocated by RRC-terminating IAB-donor over the Xn interface between the target F1-terminating IAB-donor and RRC-terminating IAB-donor.

Proposal 4: In case target logical mIAB-DU’s CU is different from mIAB-MT’s CU, both gNB ID of mIAB-MT’s CU and UE XnAP ID need to be informed via F1 setup request message from target logical mIAB-DU to target logical mIAB-DU’s CU. And the UE XnAP ID is allocated by mIAB-MT’s CU over Xn interface between mIAB-MT’s CU and target logical mIAB-DU’s CU.

Proposal 5: mIAB-node obtains gNB ID of mIAB-MT’s CU and UE XnAP ID from the mIAB-MT’s CU via RRC messages.

As in the IAB TMM request message, target F1-terminating IAB-donor also needs to include a UE XnAP ID allocated by itself for mIAB-MT as “F1-Terminating IAB-donor UE XnAP ID” IE, and it’s the first message between target F1-terminating IAB-donor and RRC-terminating IAB-donor for mIAB-node. Unless a new procedure is introduced before the IAB TMM procedure to coordinate the peer UE XnAP IDs between these two donors which cause additional impacts, the target F1-terminating IAB-donor generates a UE XnAP ID for mIAB-MT and include the UE XnAP ID in the IAB TMM request message.

Proposal 6: mIAB-DU’s CU generates a UE XnAP ID for mIAB-MT and includes the UE XnAP ID in the IAB TMM request message as “F1-Terminating IAB-donor UE XnAP ID” IE.

Avoidance of migration without IP connectivity

RAN3 119: RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available.

Once there is no IP connection between (target) RRC-terminating IAB-donor and (target) F1-terminating IAB-donor, the F1 traffic cannot be routed between the RRC-terminating IAB-donor-DU and F1-terminating IAB-donor-CU, which may lead to failure of the handover/migration. Therefore, it’s necessary to avoid performing IAB-MT handover or IAB-DU migration in case there is no IP connection between (target) RRC-terminating IAB-donor and (target) F1-terminating IAB-donor after handover/migration.

For IAB-MT handover as in Figure 1, once target IAB-donor B of IAB-MT handover is different from F1-terminating IAB-donor C, then source IAB-donor A doesn’t have the information about whether there is IP connection between target IAB-donor B and F1-terminating IAB-donor C. To avoid unnecessary failure of IAB-MT handover, source IAB-donor A may need to inquiry target IAB-donor B or F1-terminating IAB-donor C whether there is IP connection between these two IAB-donors.

And for the IAB-DU migration as in Figure 2, once target IAB-donor B of the IAB-DU migration is different from RRC-terminating IAB-donor C, then source IAB-donor A or the IAB-node doesn’t have the information about whether there is IP connection between target IAB-donor B and RRC-terminating IAB-donor C. Similarity, to avoid unnecessary failure of IAB-DU migration, source IAB-donor A or IAB-node may need to inquiry target IAB-donor B or F1-terminating IAB-donor C whether there is IP connection between these two IAB-donors.

Proposal 7: For IAB-MT handover, if target IAB-donor is different from F1-terminating IAB-donor, source IAB-donor needs to inquiry target IAB-donor or F1-terminating IAB-donor before the handover whether there is IP connection between these two IAB-donors.

Proposal 8: For IAB-DU migration, if target IAB-donor is different from RRC-terminating IAB-donor, source IAB-donor or IAB-node needs to inquiry target IAB-donor or RRC-terminating IAB-donor before the migration whether there is IP connection between these two IAB-donors.

# Conclusion

This contribution aims to discuss the general principles of mobile IAB-node inter-donor topology adaptation. And following observations and proposals are concluded.

Proposal 1: In case of consecutive partial migration, UE XnAP ID allocated by source IAB-donor or F1-terminating IAB-donor for mobile IAB-node is informed from the source IAB-donor to the F1-terminating IAB-donor to identify the mobile IAB-MT’s Handover.

Proposal 2: In case of consecutive partial migration, F1-terminating IAB-donor needs to be indicated from source IAB-donor with a UE XnAP ID allocated by target IAB-donor over the Xn interface between the F1-terminating IAB-donor and the target IAB-donor.

Proposal 3: In case of consecutive partial migration, source IAB-donor to request from target IAB-donor a UE XnAP ID allocated by the target IAB-donor over the Xn interface between the F1-terminating IAB-donor and the target IAB-donor.

Proposal 4: In case target logical mIAB-DU’s CU is different from mIAB-MT’s CU, both gNB ID of mIAB-MT’s CU and UE XnAP ID need to be informed via F1 setup request message from target logical mIAB-DU to target logical mIAB-DU’s CU. And the UE XnAP ID is allocated by mIAB-MT’s CU over Xn interface between mIAB-MT’s CU and target logical mIAB-DU’s CU.

Proposal 5: mIAB-node obtains gNB ID of mIAB-MT’s CU and UE XnAP ID from the mIAB-MT’s CU via RRC messages.

Proposal 6: mIAB-DU’s CU generates a UE XnAP ID for mIAB-MT and includes the UE XnAP ID in the IAB TMM request message as “F1-Terminating IAB-donor UE XnAP ID” IE.

Proposal 7: For IAB-MT handover, if target IAB-donor is different from F1-terminating IAB-donor, source IAB-donor needs to inquiry target IAB-donor or F1-terminating IAB-donor before the handover whether there is IP connection between these two IAB-donors.

Proposal 8: For IAB-DU migration, if target IAB-donor is different from RRC-terminating IAB-donor, source IAB-donor or IAB-node needs to inquiry target IAB-donor or RRC-terminating IAB-donor before the migration whether there is IP connection between these two IAB-donors.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN3 118 – 119bis-e meetings.