3GPP TSG-RAN WG3 Meeting #120	R3-233147

Incheon, Korea, 22 – 26 May, 2023

Agenda item:	12.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.423) AI/ML Mobility Optimization

Document for:	Text Proposal

# 1	Introduction

Some of the agreed outputs for AI/ML Mobility Enhancements in TR 37.817 are the following:

- Estimated arrival probability in CHO and relevant confidence interval

- Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication

- Priority, handover execution timing, predicted resource reservation time window for CHO.

In this contribution, we discuss our views on the standardization impacts related to those outputs.

# 2	AI/ML Mobility Optimization for CHO

In case of a CHO Handover, according to TS 38.423 a source NG-RAN node can send in Handover Request message a Conditional Handover Information Request IE that contains an Estimated Arrival Probability IE which can be used by the target NG-RAN node to allocate necessary resources. The Estimated Arrival Probability IE indicates a likelihood that the UE will end up accessing the target cell for which a handover request is sent. The algorithm used to calculate this probability is up to node’s implementation and could be based on internal legacy algorithms that could estimate such probability or done through AI/ML procedures. How this estimated arrival probability is calculated does not need to be indicated to a candidate target node.

Observation 1: A source node does not need to indicate to the candidate target the method with which (legacy algorithm, AI/ML, etc.) the Estimated Arrival Probability is calculated.

Proposal 1: The Estimated Arrival Probability IE in CHO can be calculated by legacy or by AI/ML methods.

When it comes to the confidence interval of the estimated arrival probability, once a probability is calculated for a given UE there is no more randomness and therefore the concept of confidence is not relevant. Therefore, indication of a confidence interval of an estimated arrival probability is not meaningful.

Observation 2: There is no need to indicate a confidence interval on the estimated arrival probability since it has no randomness.

A confidence interval would be meaningful to capture the randomness of the arrival process and the actual time when a UE arrives at a target node. If the arrival time at the target NG-RAN node can be predicted with a high confidence, this will allow further optimization of the resources at the target NG-RAN node. Currently there is no time indication as to when a UE is expected to arrive at the target node. It is possible that a UE has a very low confidence to arrive at a certain time interval (e.g., in the very near future), but a much higher confidence to arrive at another (e.g., a later time interval in the future).

Observation 3: If a target node knows information about the time interval during which different UEs are expected to arrive with high confidence, it can optimize its allocated resources.

For example, if the confidence of arrival at a future time interval [T1, T2] is very high, the target node could at time t< T1 accept a CHO handover even if it runs low in resources as long as it can expect that by the time that the UE will actually arrive at the target node it will have better resource availability.

Proposal 2: Introduce in Conditional Handover Request message a predicted time interval and corresponding confidence level during which a UE is expected to arrive.

Note that this proposal effectively provides a predicted resource reservation time window for CHO, which is one of the agreed outputs for AI/ML Mobility Enhancements in TR 37.817.

Handover execution timing information is not very clear in the context of normal handover. This is because handover happens immediately after Handover Conditions are met in which case the Handover timing is already determined.

Observation 4: There is no need to provide Handover Execution timing for normal Handover.

Also, for CHO Handover, indicating a handover priority is not necessary in the sense that it is unclear how different target nodes can be prioritized from the source perspective. Furthermore, it is also unclear how a target node can interpret different priority levels in its actions.

Observation 5: There is no need to provide a priority as part of an output of AI/ML Mobility Enhancements.

CHO is more robust in the sense that the risk that a UE loses coverage of the source cell is very small. However, this comes at a cost of reserving a much higher number of network resources which have to be reserved for a longer period of time than in normal Handover. This is because the CHO configuration is typically very early to achieve robustness. In addition, different target cells may be prepared, but handover will take place only in one of those cells. Therefore, robustness comes at the cost of a higher number of cell preparations which are longer than normal handover. Introduction of AI/ML in a NG-RAN node enables better resource optimization both at the source and target nodes.

Observation 6: AI/ML in the RAN can help to avoid too many and too long CHO cell preparations.

Another AI/ML Model output for AI/ML Mobility Optimization is to predict a handover target node as well as a set of candidate cells in CHO together with the confidence of the prediction. In our view, this will be an internally consumed output by the source node of the Handover. Such AI/ML Model output can further help the source node limit the number of CHO Handover initiations only to the candidate target cells where the handover is expected to happen with high confidence.

Proposal 3: Predicted handover target node and candidate cells in CHO are AI/ML Model Outputs consumed internally by the source node and have no standards impacts.

In the context of CHO, it would be also useful if the source NG-RAN node is able to cut down on the number of unnecessary CHO preparations (i.e., not even originate a Handover Request) to a potential target NG-RAN node if it knows in advance that the target NG-RAN node is not suitable to host the UE at this given point of time from the perspective of provided QoS for services established for the UE or a UE would end up with a potential failure when handing over to a target NG-RAN node. This information cannot be a static determination as the target NG-RAN nodes’ resource utilization/capacity is ever changing (including possible power saving actions etc.). One way to imagine this is to visualize the target NG-RAN nodes’ function as an estimator that can be invoked at the source NG-RAN node. The target NG-RAN nodes’ function can represent a list of values of achievable QoS indicators such as end user throughput, packet loss, rate, packet delay, etc., offered by target to source NG-RAN node or some indicators related to radio quality conditions that may point out on potential failure after handing over the UE to target NG-RAN node. By invoking this estimator and making a check with it, a source NG-RAN node would be able to potentially avert those preparations that are bound for failure. As such this requires transacting an estimator (as part of an AI/ML algorithm or otherwise) from the target NG-RAN node to the source NG-RAN node that can potentially allow the source NG-RAN node to perform this estimation. The estimator will of course go out of tune as the target NG-RAN node’s resource utilization/capacity changes over time, hence some kind of update is required back to the source NG-RAN node when that happens.

Proposal 4: To avoid unnecessary triggering of mobility procedures (e.g., CHO) towards a target NG-RAN node, it is desirable for the source NG-RAN node to be able to predict/foresee the performance (or failures) of a given UE at all possible candidate target NG-RAN nodes.

Using AI/ML, the source node can take into account in its cell preparations information about the impact of those in-vain preparations to different target NG-RAN nodes, which may not be the same across different nodes since different nodes may have different load situation, number of concurrent CHO preparations, proportion of in-vain preparations compared to successful, etc. Without any information measuring the impact of a CHO preparation for a specific target, a source NG-RAN node may only assume that the impact of in-vain preparations is the same across all candidate target NG-RAN nodes. To help source NG-RAN node predict the best candidate target gNBs for a CHO, feedback information is needed also from candidate target nodes that didn’t meet the execution condition.

Proposal 5: A configured candidate target NG-RAN node that didn’t meet the execution condition may send feedback to the source NG-RAN node to indicate the impact of the CHO preparation to the node’s performance e.g., with respect to a load related to CHO preparation, time duration of CHO configuration by this source compared to other sources, etc.

We also provide a TP in the annex of this contribution.

Proposal 6: Agree the TP for BL CR to TS 38.423.

# 3 	Conclusion

In this paper we make the following observations and proposals:

Observation 1: A source node does not need to indicate to the candidate target the method with which (legacy algorithm, AI/ML, etc.) the Estimated Arrival Probability is calculated.

Proposal 1: The Estimated Arrival Probability IE in CHO can be calculated by legacy or by AI/ML methods.

Observation 2: There is no need to indicate a confidence interval on the estimated arrival probability since it has no randomness.

Observation 3: If a target node knows information about the time interval during which different UEs are expected to arrive with high confidence, it can optimize its allocated resources.

Proposal 2: Introduce in Conditional Handover Request message a predicted time interval and corresponding confidence level during which a UE is expected to arrive.

Observation 4: There is no need to provide Handover Execution timing for normal Handover.

Observation 5: There is no need to provide a priority as part of an output of AI/ML Mobility Enhancements.

Observation 6: AI/ML in the RAN can help to avoid too many and too long CHO cell preparations.

Proposal 3: Predicted handover target node and candidate cells in CHO are AI/ML Model Outputs consumed internally by the source node and have no standards impacts.

Proposal 4: To avoid unnecessary triggering of mobility procedures (e.g., CHO) towards a target NG-RAN node, it is desirable for the source NG-RAN node to be able to predict/foresee the performance (or failures) of a given UE at all possible candidate target NG-RAN nodes.

Proposal 5: A configured candidate target NG-RAN node that didn’t meet the execution condition may send feedback to the source NG-RAN node to indicate the impact of the CHO preparation to the node’s performance e.g., with respect to a load related to CHO preparation, time duration of CHO configuration by this source compared to other sources, etc.

Proposal 6: Agree the TP for BL CR to TS 38.423.

# References

[1]		R3-232523, (BLCR to 38.423) for AI/ML for NG-RAN

# Annex - TP for BL CR to TS 38.423

This TP is based on [1].

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall stop the timer TXnRELOCprep and terminate the Handover Preparation procedure. If the procedure was initiated for an immediate handover, the source NG-RAN node shall start the timer TXnRELOCoverall. The source NG-RAN node is then defined to have a Prepared Handover for that Xn UE-associated signalling.

For each E-RAB ID IE included in the QoS Flow To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the IE in the UE context and use it for subsequent inter-system handover.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

At reception of the HANDOVER REQUEST message the target NG-RAN node shall prepare the configuration of the AS security relation between the UE and the target NG-RAN node by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE, as specified in TS 33.501 [28].

Upon reception of the PDU Session Resource Setup List IE, contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as specified in TS 38.413 [5] for the PDU Session Resource Setup procedure. The target NG-RAN node shall report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU session resources. When the target NG-RAN node reports the unsuccessful establishment of a PDU session resource, the cause value should be precise enough to enable the source NG-RAN node to know the reason for the unsuccessful establishment.

For each PDU session if the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store the received PDU Session Aggregate Maximum Bit Rate in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [7].

For each QoS flow for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" for all the QoS flows mapped to a DRB, if it requests a DAPS handover for that DRB.

For each PDU session for which the target NG-RAN node decides to admit the data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session level DL data forwarding UP TNL Information IE within the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resource Admitted Info IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

For each QoS flow for which the source NG-RAN node has not yet received the SDAP end marker packet if QoS flow re-mapping happened before handover, the source NG-RAN node shall include the UL Forwarding Proposal IE within the Data Forwarding and Offloading Info from source NG-RAN node IE in the HANDOVER REQUEST message, and if the target NG-RAN node decides to admit uplink data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted Item IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the uplink data forwarding.

For each PDU session resource successfully setup at the target NG-RAN, the target NG-RAN node may allocate resources for additional Xn-U PDU session resource GTP-U tunnels, indicated in the Secondary Data Forwarding Info from target NG-RAN node List IE.

For each PDU session in the HANDOVER REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resources To Be Setup List IE, the target NG-RAN node may accept the setup of the involved QoS flow when notification control has been enabled if the requested QoS parameters set or at least one of the alternative QoS parameters sets can be fulfilled at the time of handover as specified in TS 23.501 [7]. In case the target NG-RAN node accepts the handover fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it can currently fulfil in the Current QoS Parameters Set Index IE within the PDU Session Resources Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message while setting the QoS parameters towards the UE according to the requested QoS parameters set as specified in TS 23.501 [7].

For each DRB for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DRB ID IE and the mapped QoS Flows List IE within the Source DRB to QoS Flow Mapping List IE contained in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node may include the QoS Flow Mapping Indication IE in the Source DRB to QoS Flow Mapping List IE to indicate that only the uplink or downlink QoS flow is mapped to the DRB. If the target NG-RAN node decides to use the same DRB configuration and to map the same QoS flows as the source NG-RAN node, the target NG-RAN node includes the DL Forwarding GTP Tunnel Endpoint IE within the Data Forwarding Response DRB List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this DRB.

The target NG-RAN node may additionally include the Redundant DL Forwarding UP TNL Information IE if at least one of the QoS flow mapped to the DRB is eligible to the redundant transmission feature as indicated in the Redundant QoS Flow Indicator IE within the PDU Session Resource To Be Setup List IE received in the HANDOVER REQUEST message for the QoS flow.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding UP TNL Information IE for a given DRB in the Data Forwarding Response DRB List IE within Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted List IE and the source NG-RAN node accepts the data forwarding proposed by the target NG-RAN node, the source NG-RAN node shall perform forwarding of uplink data for the DRB.

If the HANDOVER REQUEST includes PDU session resources for PDU sessions associated to S-NSSAIs not supported by target NG-RAN, the target NG-RAN node shall reject such PDU session resources. In this case, and if at least one PDU Session Resource To Be Setup Item IE is admitted, the target NG-RAN node shall send the HANDOVER REQUEST ACKNOWLEDGE message including the PDU Session Resources Not Admitted List IE listing corresponding PDU sessions rejected at the target NG-RAN.

If the Mobility Restriction List IE is

-	contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	store the information received in the Mobility Restriction List IE in the UE context;

-	use this information to determine a target for the UE during subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE, except when one of the PDU sessions has a particular ARP value (TS 23.501 [7]) in which case the information shall not apply;

-	use this information to select a proper SCG during dual connectivity operation.

-	use this information to select proper RNA(s) for the UE when moving the UE to RRC_INACTIVE.

-	not contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	consider that no roaming and no access restriction apply to the UE except for the PNI-NPN mobility as described in TS 23.501 [7].

The target NG-RAN node shall consider that roaming or access to CAG cells is only allowed if the Allowed PNI-NPN ID List IE is contained in the HANDOVER REQUEST message, as described in TS 23.501 [7].

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Index to RAT/Frequency Selection Priority IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the UE Context Reference at the S-NG-RAN IE is contained in the HANDOVER REQUEST message the target NG-RAN node may use it as specified in TS 37.340 [8]. In this case, the source NG-RAN node may expect the target NG-RAN node to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [8].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource To Be Setup List IE and the Common Network Instance IE is not present, the target NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

Redundant transmission:

-	For each PDU session, if the Redundant UL NG-U UP TNL Information at UPF IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for the redundant transmission for the concerned PDU session.

-	For each PDU session, if the Additional Redundant UL NG-U UP TNL Information at UPF List IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use them as the uplink termination points for the user plane data for the redundant transmission for the concerned PDU session.

-	For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [7].

-	For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context and set up the redundant user plane for the concerned PDU session, as specified in TS 23.501 [7]. If the PDU Session Pair ID IE is included in the Redundant PDU Session Information IE, the target NG-RAN node may store and use it to identify the paired PDU sessions.

If the TSC Traffic Characteristics IE is included in the QoS Flows To Be Setup List in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Be Setup List IE or in the Additional UL NG-U UP TNL Information at UPF List IE, or in the Additional Redundant UL NG-U UP TNL Information at UPF List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the concerned NG-U transport bearer as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", the target NG-RAN node shall perform user plane integrity protection or ciphering, respectively. If the NG-RAN node is not able to perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required".

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or the Confidentiality Protection Indication IE is set to "preferred", the target NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively and shall notify the SMF whether it succeeded the user plane integrity protection or ciphering or not for the concerned security policy.

For each PDU session for which the Maximum Integrity Protected Data Rate IE is included in the Security Indication IE in the PDU Session Resources To Be Setup List IE, the NG-RAN node shall store the respective information and, if integrity protection is to be performed for the PDU session, it shall enforce the traffic corresponding to the received Maximum Integrity Protected Data Rate IE, for the concerned PDU session and concerned UE, as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "not needed", the target NG-RAN node shall not perform user plane integrity protection or ciphering, respectively, for the concerned PDU session.

For each PDU session, if the Additional UL NG-U UP TNL Information List IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node may forward the UP transport layer information to the target S-NG-RAN node as the uplink termination point for the user plane data for this PDU session split in different tunnel.

If the Location Reporting Information IE is included in the HANDOVER REQUEST message, then the target NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

Upon reception of UE History Information IE in the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

If the Trace Activation IE is included in the HANDOVER REQUEST message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE and if the target NG-RAN node is a gNB receiving a MDT Configuration-EUTRA IE, or the target NG-RAN node is a ng-eNB receiving a MDT Configuration-NR IE, the target NG-RAN node shall store it as part of the UE context, and use it as described in TS 37.320 [43].

If the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the HANDOVER REQUEST message includes the Management Based MDT PLMN List IE, the target NG-RAN node shall, if supported, store it in the UE context, and take it into account if it includes information regarding the PLMN serving the UE in the target NG-RAN node.

If the Mobility Information IE is provided in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information. The target NG-RAN shall, if supported, store the C-RNTI assigned at the source cell as received in the HANDOVER REQUEST message.

Upon reception of the UE History Information from the UE IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

For each QoS flow which has been successfully established in the target NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting.

If the 5GC Mobility Restriction List Container IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

-	If the NR V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the LTE V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

5G ProSe:

-	If the 5G ProSe Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

- 	If the 5G ProSe PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.304 [48].

If the PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287 [38].

If the DAPS Request Information IE is included for a given DRB in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a DAPS handover for that DRB, as described in TS 38.300 [9]. Accordingly, the target NG-RAN node shall include the DAPS Response Information IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Maximum Number of CHO Preparations IE is included in the Conditional Handover Information Acknowledge IE contained in the HANDOVER REQUEST ACKNOWLEDGE message, then the source NG-RAN node should not prepare more candidate target cells for a CHO for the same UE towards the target NG-RAN node than the number indicated in the IE.

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use the information to allocate necessary resources for the incoming CHO.

If the Estimated Arrival Interval IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use the information to allocate necessary resources for the incoming CHO.

If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node. In addition:

-	If the No PDU Session Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resources To Be Setup List IE, and shall not take any action with respect to PDU session setup. Subsequently, the source NG-RAN node shall, if supported, ignore the PDU Session Resources Admitted To Be Added List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If for a given QoS Flow the Source DL Forwarding IP Address IE is included within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the MBS Session Information List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, establish an NG-RAN MBS session resources context as specified in TS 23.247 [46] and TS 38.300 [9], if applicable.

If the HANDOVER REQUEST message includes the MBS Area Session ID IE, the target NG-RAN, if supported, shall use this information as an indication from which MBS Area Session ID the UE is handed over. For each MBS session for which the Active MBS Session Information IE is included in the MBS Session Information Item List IE, the target NG-RAN shall, if supported, use this information to setup respective MBS session resources. The target NG-RAN node shall, if supported, consider that the MBS sessions for which the Active MBS Session Information IE is not included are inactive.

If the HANDOVER REQUEST ACKNOWLEDGE message contains in the MBS Session Information Response List IE the MBS Data Forwarding Response Info IE that the source NG-RAN node shall use the information for forwarding MBS traffic to the target NG-RAN node.

If the MBS Session Associated Information List IE is included in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the information contained in the Associated QoS Flows Information List IE as specified in TS 23.247 [46].

For each MRB indicated in the MBS Mapping and Data Forwarding Request Info from source NG-RAN node IE, the target NG-RAN node shall use the MRB ID IE and, if included, the MRB Progress Information IE which includes the highest PDCP SN of the packet which has already been delivered to the UE for the MRB, to decide whether to apply data forwarding for that MRB and to establish respective resources.

The source NG-RAN shall, for each MRB in the MBS Data Forwarding Response Info from target NG-RAN node IE in the HANDOVER REQUEST ACKNOWLEDGE message, start data forwarding to the indicated DL Forwarding UP TNL Information. If the MRB Progress Information IE is included the source NG-RAN node may use the information to determine when to stop data forwarding.

If the Time Synchronisation Assistance Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Cell Based UE Trajectory Prediction IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, considers the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. mobility decisions.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

#### 8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message and the target NG-RAN node rejects the handover or a failure occurs during the Handover Preparation, the target NG-RAN node shall include the Requested Target Cell ID IE in the HANDOVER PREPARATION FAILURE message.

Interactions with Handover Cancel procedure:

If there is no response from the target NG-RAN node to the HANDOVER REQUEST message before timer TXnRELOCprep expires in the source NG-RAN node, the source NG-RAN node should cancel the Handover Preparation procedure towards the target NG-RAN node by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source NG-RAN node shall ignore any HANDOVER REQUEST ACKNOWLEDGE or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure and remove any reference and release any resources related to the concerned Xn UE-associated signalling.

Interaction with AI/ML Information Reporting procedure:

If the AI/ML Measurement ID is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after the completion of handover procedure, via the AI/ML Information Reporting procedure, the requested AI/ML information configured via the previous AI/ML Information Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement and the NG-RAN node2 Measurement ID IEs.

#### 8.2.1.4	Abnormal Conditions

If the supported algorithms for encryption defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EEA0 and NEA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the supported algorithms for integrity defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EIA0 and NIA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the CHO trigger IE is set to "CHO-replace" in the HANDOVER REQUEST message, but there is no CHO prepared for the included Target NG-RAN node UE XnAP ID, or the candidate cell in the Target Cell ID IE was not prepared using the same UE-associated signaling connection, the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the HANDOVER REQUEST message includes information for a PLMN not serving the UE in the target NG-RAN node in the Management Based MDT PLMN List IE, the target NG-RAN node shall ignore information for that PLMN within the Management Based MDT PLMN List.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Estimated Arrival Interval | O |  | 9.2.3.y |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x |  | YES | ignore |
| AI/ML Measurement ID (FFS on the name) | O |  | 9.2.3.M |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.y	Estimated Arrival Interval

This IE indicates a time interval when a UE is expected to arrive at the target node and a related confidence level.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Estimated Arrival Interval |  |  |  |  |
| > Estimated Earliest Arrival Time | M |  | INTEGER (0..4095) | Corresponds to the earliest estimated time of UE arrival Unit: [second]. |
| > Estimated Latest Arrival Time | M |  | INTEGER (0..4095) | Corresponds to the latest estimated time of UE arrival Unit: [second]. |
| >Confidence Level | M |  | INTEGER (1..100) | This IE indicates the confidence level of the estimated arrival interval in terms of a probability that the arrival time is between the estimated earliest arrival time and the estimated latest arrival time. |

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>