3GPP TSG-RAN WG3 Meeting # 120	 R3-233196

# Incheon, Korea 22 - 26 May 2023

Agenda item:	14.2

Source:	Google

Title:	(TP for L1L2Mob BLCR for TS 38.401) Discussion on LTM execution

Document for:	Discussion & Decision

# Introduction

This paper discusses steps in LTM execution considering the CU-DU split and provides a TP to the TS 38.401 BLCR for LTM.

# Discussion

In RAN3#119bis-e, RAN3 made some agreements related to DDDS.

1. DDDS: For intra-DU LTM, the gNB-DU sends a DDDS frame about unsuccessfully transmitted downlink data to the gNB-CU after LTM cell switch if RLC reestablishment is configured. For inter-DU LTM, the DDDS should be sent from source gNB-DU to CU-UP when the LTM cell switch command is sent. Then the CU-UP can start forwarding the unsuccessfully transmitted data to target gNB-DU. Second Round: DDDS： For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs for which RLC is not re-established. FFS on whether or how to capture it in spec. For intra-CU inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after target gNB-DU detects the UE access (following legacy). FFS on how to capture in spec. FFS: For intra-DU LTM, the gNB-DU sends initial DDDS using the new UL TEID if assigned by the CU to CU-UP after target gNB-DU detects the UE access.

On the topic of LTM procedure consolidation, RAN2 made some further agreements [1] in RAN2#121bis-e:

R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data. Following behaviors of LTM supervisor timer are agreed: - - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) LTM supervisor timer is RRC layer timer. At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

It was marked as FFS on “How the target gNB-DU detects the UE access” in the TS 38.401 BLCR. Considering the above progresses in RAN3 and RAN2, we propose to update the BLCR to resolve the FFS. As the UE may or may not perform RACH with the (target) gNB-DU, the RACH procedure can be optional in the figure and the corresponding descriptions. The (target) gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received RRCReconfigurationComplete message upon LTM execution.

Proposal 1: RAN3 to agree the TP for LTM execution in TS 38.401 BLCR.

# Conclusion

Proposal 1: RAN3 to agree the TP for LTM execution in TS 38.401 BLCR.

# References

[1] RAN2#121bis-e Chair Notes EOM, MediaTek

[2] R2-2304214	Summary of [AT121bis-e][018][eMob] Procedure Consolidation (Huawei), Huawei, HiSilicon

# TP for TS 38.401 BL CR

#### 8.2.1.X	Intra-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for L1/L2 triggered mobility. Figure 8.2.1.x-1 shows the intra-gNB-DU L1/L2 triggered mobility procedure for intra-NR..

Figure 8.2.1.x-1: Intra-gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the target candidate cells.

- If the gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell(s).

FFS for Step 3 and Step 4: either a single or multiple UE Context Modification procedure(s) should be used.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 triggered mobility configuration.

FFS: whether it is DL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION REQUEST message.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION RESPONSE message.

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2 inter-cell mobility.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU signals the gNB-CU about the initiation of the L1/L2 triggered mobility command to the UE including the Target cell ID.

FFS: introduce new message or reuse legacy message.

12a. The UE may perform random access procedure with the gNB-DU.

Editor’s note: Step 12 can be updated according to RAN2’s discussion.

12b. The UE sends an RRCReconfigurationComplete message to the gNB-DU.

12c. The gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received RRCReconfigurationComplete message.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU may send the UE CONTEXT MODIFICATION message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

/***********************************Next change***********************************/

#### 8.2.1.Y	Inter-gNB-DU L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for L1/L2 Triggered Mobility. Figure 8.2.1.Y-1 shows the inter-gNB-DU L1/L2 Triggered Mobility procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU L1/L2 Triggered Mobility

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2 triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the target candidate cells.

FFS for Step 3 and Step 4: either a single or multiple UE Context Setup procedure(s) should be used.

- If the candidate gNB-DU accepts the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the L1/L2 triggered mobility configuration.

FFS: whether it is DL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION REQUEST message.

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

FFS: whether it is UL RRC MESSAGE TRANSFER message or UE CONTEXT MODIFICATION RESPONSE message.

- The UE sends the lower layer measurement result to the source gNB-DU.

- The source gNB-DU decides to execute L1/L2 triggered mobility to a candidate target cell.

FFS: Notifying the LTM triggering decision to the other nodes as well.

- The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The source gNB-DU signals the gNB-CU about the initiation of the L1/L2 triggered mobility command to the UE including the Target cell ID.

FFS: introduce new message or reuse legacy message.

13a. The UE may perform random access procedure with the target gNB-DU. The target gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU.

Editor’s note: In case of RACH-less LTM, the Downlink Data Delivery Status frame is sent after the step 13b instead of step 13a. Step 13 can be updated according to RAN2’s discussion.

13b. The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

13c. The target gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received RRCReconfigurationComplete message.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

- The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

- The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

- /***********************************End of changes***********************************/