3GPP TSG-RAN WG3 Meeting #120	R3-233292

Incheon, 22 - 26 May, 2023

Title: 	Discussion on QoE measurement for high mobility scenarios

Source: 	China Unicom

Agenda item:	11.2

Document Type:	Discussion

Introduction

The Work Item on QoE reporting has been agreed in RAN#96 (RP-221803) with the following objectives:

- Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains.

This paper discusses to support the QoE measurement for high mobility scenarios.

Discussion

The following was captured in the 118 meeting with respect to high speed scenarios:

RAN3 first focus on supporting the following scenario QMC:

- QoE measurement collection and reporting when the UE is in HSDN cells

For confining the QoE measurements to HSDN cells, RAN3 to choose between the HSDN-wide indication, existing area scope and other possible enhancements if needed.

For supporting QMC in high mobility scenarios, RAN3 to determine the meaning of “high mobility”.

Regarding HSDN cells, OAM is aware of the whole HSDN cells list and it can configure those HSDN cell accordingly. Enlarging the QoE area scope of whole list of HSDN cells seems to be costly for the purpose of the continuity QMC in HSDN cells. Then a HSDN-cell indication can be considered, only if the HSDN cell should performance the HSDN-QoE configuration to its UEs according to this indication sent from the OAM.

Meanwhile considering the handover scenario, the target gNB can check whether it belongs to the HSDN-cell, then to decide whether to release the corresponding HSDN-QoE measurement or not. The non-HSDN cell can release the HSDN-dedicated QoE measurement while the HSDN cell can proceed the corresponding QMC.

Observation 1: OAM can configure the HSDN-dedicated QoE measurement towards the HSDN cells.

Proposal 1: A HSDN-wide indication can be added for HSDN QoE configuration for handover scenarios.

Proposal 2: The non-HSDN cells can release the HSDN-dedicated QoE measurement.

For determining the meaning of “high mobility”, based on the previous agreement, we first focus on the UEs in HSDN cell, generally most of HSDN cell serving UEs would be in high-mobility state, then RAN3 can start with the HSDN cells in high mobility scenarios.

Proposal 3: For supporting QMC in high mobility scenarios, RAN3 decides to support HSDN-dedicated QoE measurement.

Conclusion

Observation 1: OAM can configure the HSDN-dedicated QoE measurement towards the HSDN cells.

Proposal 1: A HSDN-wide indication can be added for HSDN QoE configuration for handover scenarios.

Proposal 2: The non-HSDN cells can release the HSDN-dedicated QoE measurement.

Proposal 3: For supporting QMC in high mobility scenarios, RAN3 decides to support HSDN-dedicated QoE measurement.