3GPP TSG-RAN WG3#121bis	R3-235040

Xiamen, China, October 9-13, 2023

Agenda item:	14.2 Signalling Support for L1/L2 based Inter-Cell Mobility

Source:	Nokia, Nokia Shanghai Bell

Title:	TP (BL CR TS 38.401) L1/2 Triggered Mobility (LTM) Procedures

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#121, L1/L2 centric mobility (RP-221799: Further NR Mobility Enhancements) was discussed, with some agreements reached for both intra-gNB-DU and inter-gNB-DU scenarios. RAN3#121 agreements are listed below:

Agreements at RAN3#121 meeting RAN3#121: Turn the WA to agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch. The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM. After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE. The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU. RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue. CU shall send the TA value to the source DU. For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU. Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3 with FFS. The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations. In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure. Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2. WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU. WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU. How the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU). Source DU is in charge of checking the TA validity. In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority. In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority, WA: In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority. Prioritize the reference configuration on F1 at next meeting. Work on stage2 and stage3 details Other issues unconcluded from last meeting Issue 1: L1 measurement configuration and reporting FFS on the details of Stage 2 and stage3. FFS whether CU needs to request the DU to provide the RS configuration. Issue 2: TCI state configuration FFS on the need of an explicit request from CU Issue 3: Selected beam transfer FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID. Option 1: Two new class 2 procedures with different name. Option 2: same new class 2 procedure Option 3: class 1 procedure. Down selection among those 3 options at next meeting. Issue 4: TA acquisition Issue 6: LTM and L3 handover collision Issue 7: Reference configuration List all options and down selection.

In this contribution, we describe our view on the remaining open points which needs further discussion in RAN3. To align with RAN2 terminology, we stick to the term “Candidate gNB-DU” when referring to a gNB-DU that hosts candidate target cells subject to LTM configuration.

# 2	Baseline Procedure

Taking the agreements from last meetings, the following, which affect the baseline for intra-DU and inter-DU LTM procedures, can be discussed further:

LTM Preparation Phase

- RS Configuration signaling to gNB-CU

In the last RAN3 meeting, the following agreements were reached:

For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU. Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3 with FFS.

Similarly, the following were remaining open items:

FFS on the details of Stage 2 and stage3. FFS whether CU needs to request the DU to provide the RS configuration.

Firstly, we understand that “RS Configuration” in this case is referring to SSB related configuration. Hence, the reason for indicating FFS on “RS configuration” for Stage 2 and Stage 3 BL CRs was due that another alternative (described in R3-233840) is to rely on the non-associated signaling taking place during cell activation over F1, where the SSB related configuration for each cell at a gNB-DU is already informed to the gNB-CU, and which was proposed to be further studied as an alternative in RAN3.

We see this proposal as a reasonable solution that would allow reducing the signaling size from each configured LTM UE context. In this manner, the SSB related configuration would be signaled once during cell action via F1, rather than retrieved per configured LTM UE context (i.e. signaling over UE Context Modification Response message). Similarly, there is no need for an explicit request of SSB related configuration to the Candidate gNB-DU(s).

Proposal 1: Each Candidate gNB-DU signals the SSB related configuration for its cells via legacy cell activation procedures over F1 (F1 Setup, gNB-DU Configuration Update), incurring no specification changes.

Proposal 2: There is no need for gNB-CU to explicitly request a Candidate gNB-DU to provide its SSB related configuration and this takes place implicitly as part of the F1 Setup and gNB-DU Configuration Update procedures.

Secondly, the gNB-CU nevertheless needs to retrieve the CSI Resource Configuration for LTM via a UE-associated procedure, and which is signaled by the (Candidate) gNB-DU via UE Context Modification Response message. Hence, we propose to rephrase and clarify this in both Stage 2 and Stage 3 by clearly indicating what is signaled is “CSI Resource Configuration” rather than “RS Configuration”.

Proposal 3: Replace “RS Configuration” with “CSI Resource Configuration” IE in UE Context Modification Response message.

Proposal 4: Remove “RS configuration” IE from TS 38.473 and TS 38.401 BL CRs in UE Context Setup Response message.

Proposal 5: There is no need for gNB-CU to explicitly request a Candidate gNB-DU to provide its CSI Resource Configuration and this takes place implicitly as part of the UE Context Modification procedure.

- TCI state configuration

RAN3 has agreed on the mechanism to signal the TCI configuration from Candidate gNB-DUs to gNB-CU. However, it was left open whether a explicit indication over F1AP is needed to request this information:

FFS on the need of an explicit request from CU

Given that during the F1AP procedures used during LTM Preparation Phase, the Candidate gNB-DU is aware that LTM is being configured, there is no need for an explicit indication to the Candidate gNB-DU requesting the TCI state. That is, the Candidate gNB-DU can implicitly understand that the TCI state configuration is to be included in the UE Context Setup  Response message.

Proposal 6: There is no need for gNB-CU to explicitly request a Candidate gNB-DU to provide its TCI state configuration.

- Assignment of Contention-Free Random Access (CFRA) Preambles

For TA acquisition, the prepared candidate gNB-DU(s) cell(s) needs to reserve CFRA preambles for each UE and for a long-time duration if dynamic switching is enabled. This reduces the network flexibility in assigning CFRA preambles to different UEs, especially in scenarios where many UEs are configured with LTM.

Observation 1: In scenarios where many UEs are configured with LTM, reserving unique CFRA preamble for each UE will limit the network flexibility in assigning CFRA preambles.

From RAN2 perspective, it is already assumed that the candidate gNB-DU cell can assign a pool of CFRA preambles that can be used by the UEs of any source cell. This is highlighted in bold text below:

RAN2#121bis-e agreement: From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

However, there is no restriction that the CFRA resource cannot be shared among multiple UEs being served by different source gNB-DUs. This results in the problem that different source gNB-DUs can decide to use the same CFRA resource for different UEs which may result in PRACH collision. In other words, CFRA becomes Contention Based Random Access (CBRA) which should be avoided.

Observation 2: In case of shared CFRA resources, different source gNB-DUs may decide to use the same CFRA resource for different UEs which may result in PRACH collision.

To avoid the PRACH collision, it is important that RAN3 restricts the sharing of CFRA resources among UEs being served by cells in different source gNB-DUs. In other words, it is preferable that the CFRA resources are only shared by UEs which are served by cells in same gNB-DU. In this way, by implementation, a gNB-DU can only allocate one CFRA resource to a UE at a given time and avoid PRACH collision without the need of extra coordination between multiple gNB-DUs. The latter results in extra signalling overhead and handover latency. On this matter, RAN3 had the following working assumption, which we propose is turned into an agreement.

Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

Proposal 7: Turn Working Assumption into agreement that RAN3 restricts the sharing of CFRA resources among UEs being served by cells in same source gNB-DU.

In order to introduce this restriction the source gNB-DU ID is to be signalled as part of UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, and have the candidate gNB-DU allocate CFRA resources based on the received source gNB-DU ID. On this regard, RAN3 had the following working assumption which we propose to turn into an agreement.

WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

Proposal 8: Turn Working Assumption into agreement that Source gNB-DU ID is provided as part of UE CONTEXT SETUP REQUEST message to enable source gNB-DU specific CFRA allocation by the candidate gNB-DU.

Early Synchronization

- Early TA Acquisition

At last RAN3 meeting, the following working assumption was reached, yet remains pending conclusion whether to utilize (a) the same new procedure to deliver the TA from Candidate gNB-DU to gNB-CU, and subsequently from gNB-CU to Source gNB-DU, or (b) introduce one new procedure to deliver the TA from Candidate gNB-DU to gNB-CU, and another new procedure to deliver the TA from gNB-CU to Source gNB-DU.

WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

One argument for two new procedures over one was that these messages may contain different information depending on the receiver. Similarly, whether the gNB-CU could modify the information received prior to passing it to the Source gNB-DU. In our view, the information sent does not need to differ or be modified by the gNB-CU. Hence, introducing one new procedure to be used both toward the gNB-CU and toward the Source gNB-DU with same contents is sufficient.

Proposal 9: Turn working assumption into agreement that the same new “TA NOTIFICATION” non-UE associated procedure is used to deliver the TA Information and preamble related information from the Candidate gNB-DU to the Source gNB-DU via the gNB-CU.

Proposal 10: The new “TA NOTIFICATION” procedure includes TA Information (TA Value, and preamble related information).

In addition, there was discussion at last RAN3 meeting on whether the “Source gNB-DU-ID” needs to be signaled to the gNB-CU to differentiate the gNB-DU that should receive the TA values sent by the Candidate gNB-DU. In our view, this is not needed, since the gNB-CU during LTM configuration is already aware of the CFRA resource pool that is to be shared with a given gNB-DU. Thus, a gNB-CU should be able to differentiate the Source gNB-DU based on the CFRA resources used and associated to the TA value that was received.

Proposal 11: There is no need for a Candidate gNB-DU to signal “Source gNB-DU-ID” to a gNB-CU as part of the TA NOTIFICATION procedure.

LTM Execution Phase

- Signal the Activated TCI state to Candidate gNB-DU after LTM

As indicated in LS from RAN2 in R3-233719, RAN2 has agreed that the Candidate gNB-DU should be aware of the beam information (i.e., the indicated TCI state) that the UE will utilize in the target cell. For this to be supported, this information needs to be signaled (a) from Source gNB-DU to gNB-CU firstly, and then (b) from gNB-CU to the Candidate gNB-DU.

- Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

Similarly, RAN3 agreed to use the LTM CELL CHANGE NOTIFICATION procedure as follows.

The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU.

However, there were some open items, as it was raised concern that RAN3 should evaluate whether the information from Source gNB-DU to gNB-CU, and the information from gNB-CU to Candidate gNB-DU would be consistent enough to benefit from utilizing a single message. The following was captured in the chair notes.

FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID. Option 1: Two new class 2 procedures with different name. Option 2: same new class 2 procedure Option 3: class 1 procedure. Down selection among those 3 options at next meeting.

In our view, there is no difference in the required information to be received at the gNB-CU, and at the Candidate gNB-DU. Further, in regard to the beam selection, this information is mainly of use to the Candidate gNB-DU. Hence, the gNB-CU’s role is to forward the information to the Candidate gNB-DU. Therefore, the LTM CELL CHANGE NOTIFICATION message is sufficient for both Source gNB-DU to gNB-CU, and for gNB-CU to Candidate gNB-DU directions.

In addition to the beam selection, at prior meetings (R3-233828) there is also proposed to forward the TA retrieved values from the Source gNB-DU to the Candidate gNB-DU, which has benefit of speeding up the LTM procedures as known TAs can be re-used at the Candidate gNB-DU, also reducing signaling. Passing of the retrieved TAs was also proposed to be carried out via the LTM CELL CHANGE NOTIFICATION message. As with the beam selection, the LTM CELL CHANGE NOTIFICATION message is sufficient for both Source gNB-DU to gNB-CU, and for gNB-CU to Candidate gNB-DU directions.

Proposal 12: The LTM CELL CHANGE NOTIFICATION procedure is sufficient for transferring the selected beam (TCI state) from Source gNB-DU to gNB-CU, and subsequently from gNB-CU to Candidate gNB-DU.

# 3	Conclusions

Proposal 1: Each Candidate gNB-DU signals the SSB related configuration for its cells via legacy cell activation procedures over F1 (F1 Setup, gNB-DU Configuration Update), incurring no specification changes.

Proposal 2: There is no need for gNB-CU to explicitly request a Candidate gNB-DU to provide its SSB related configuration and this takes place implicitly as part of the F1 Setup and gNB-DU Configuration Update procedures.

Proposal 3: Replace “RS Configuration” with “CSI Resource Configuration” IE in UE Context Modification Response message.

Proposal 4: Remove “RS configuration” IE from TS 38.473 and TS 38.401 BL CRs in UE Context Setup Response message.

Proposal 5: There is no need for gNB-CU to explicitly request a Candidate gNB-DU to provide its CSI Resource Configuration and this takes place implicitly as part of the UE Context Modification procedure.

Proposal 6: There is no need for gNB-CU to explicitly request a Candidate gNB-DU to provide its TCI state configuration.

Observation 1: In scenarios where many UEs are configured with LTM, reserving unique CFRA preamble for each UE will limit the network flexibility in assigning CFRA preambles.

Observation 2: In case of shared CFRA resources, different source gNB-DUs may decide to use the same CFRA resource for different UEs which may result in PRACH collision.

Proposal 7: Turn Working Assumption into agreement that RAN3 restricts the sharing of CFRA resources among UEs being served by cells in same source gNB-DU.

Proposal 8: Turn Working Assumption into agreement that Source gNB-DU ID is provided as part of UE CONTEXT SETUP REQUEST message to enable source gNB-DU specific CFRA allocation by the candidate gNB-DU.

Proposal 9: Turn working assumption into agreement that the same new “TA NOTIFICATION” non-UE associated procedure is used to deliver the TA Information and preamble related information from the Candidate gNB-DU to the Source gNB-DU via the gNB-CU.

Proposal 10: The new “TA NOTIFICATION” procedure includes TA Information (TA Value, and preamble related information).

Proposal 11: There is no need for a Candidate gNB-DU to signal “Source gNB-DU-ID” to a gNB-CU as part of the TA NOTIFICATION procedure.

Proposal 12: The LTM CELL CHANGE NOTIFICATION procedure is sufficient for transferring the selected beam (TCI state) from Source gNB-DU to gNB-CU, and subsequently from gNB-CU to Candidate gNB-DU.

# References

- RP-221799 Further NR Mobility Enhancements, MediaTek

- R3-233719 LS on Early TA and RACH-less, RAN2 LS

- R3-233720 LS on L1 measurements for LTM, RAN2 LS

- R3-234800 (BLCR to 38.401) for L1L2Mob, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, ZTE

- R3-233828, TP (BL CR TS 38.473) Additional Implications to L1/2 Triggered Mobility (LTM), Nokia, Nokia Shanghai Bell

- RAN3_121_agenda_20230825_EOM, Chair’s notes

# Annex: TP for TS 38.401

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result FFS) to the gNB-DU containing  measurements of neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID. The gNB-CU requests PRACH resources from the gNB-DU.

- If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations  (e.g., TCI state configuration and RACH configuration) and the CSI Resource configuration for the accepted target candidate cell.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- Early synchronization is performed as specified in TS 38.300 [x].

- The UE sends the lower layer measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

- The gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the selected beam.

- FFS: How the target gNB-DU detects the UE access.

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The gNB-CU may send the UE CONTEXT MODIFICATION message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result FFS) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to initiate LTM configuration.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the Candidate gNB-DU.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration (e.g., TCI state configuration and RACH configuration) for the accepted target candidate cell.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected RS configuration, TCI state configuration and RACH configuration for the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which may include the generated CSI resource configuration.

7. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing  the cell ID(s) of the prepared candidate cell(s) and associated RS configuration for each candidate cell in other candidate gNB-DU(s).

Note: The candidate cell may be the same cell as source cell.

8. The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated CSI resource configuration.

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. Early synchronization is performed as specified in TS 38.300 [x].

14. The candidate gNB-DU sends a TA NOTIFICATION message including the TA and the associated CFRA resource information (e.g., preamble related information) to the source gNB-DU via the gNB-CU.

15. The UE sends the lower layer measurement result to the source gNB-DU.

16. The source gNB-DU decides to execute LTM to a candidate target cell.

FFS: Notifying the LTM triggering decision to the other nodes as well.

17. The source gNB-DU sends LTM command to the UE.

Editor’s note: The LTM command needs to be updated according to RAN2’s discussion.

18. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the selected beam information.

19. The gNB-CU sends LTM CELL CHANGE NOTIFICATION message including the target cell ID and the selected beam information to the candidate gNB-DU.

FFS on details of the selected beam information.

20. FFS:  how the target gNB-DU detects the UE access.

21. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

22. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

23. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

---------------------------------------------------------next change -------------------------------------------------------------

#### 8.2.1.Z	LTM with gNB-CU-UP change

Figure 8.2.1.z-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.z LTM with the change of gNB-CU-UP

0. The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1. The gNB-CU-CP decides to initiate LTM configuration.

2. The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4 - 5. F1 UE context setup procedure is performed to setup one or more bearers in the gNB-DU.

6. The gNB-CU-CP signals the collected TCI state information and RS configuration of the candidate cells to the Source gNB-DU and Candidate gNB-DU via the UE Context Modification procedure.

7 - 8.The gNB-CU-CP sends the RRC Reconfiguration message to the UE.

9.  The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

10. The source gNB-DU sends the LTM CELL CHANGE NOTIFICATION message to the gNB-CU-CP with the target cell ID and selected beam information.

11. The gNB-CU sends the LTM CELL CHANGE NOTIFICATION message to the candidate gNB-DU with the target cell ID and selected beam information.

12-13. The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers.

14-15.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UP/DL status to the target gNB-CU-UP.

16.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

17.  The target gNB-DU detects the UE in the target cell.

18. The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

19 - 21. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

22-23.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-DU.

--------------------------------------------------------- the end of changes -------------------------------------------------------------