3GPP TSG-RAN WG3 #121bis	R3-235138

Xiamen, China, 9th – 13th October 2023

Source:	CATT

Title:	(TP to BLCR of 38.423) Discussion on UE performance feedback

Agenda Item:	12.2.2.1

Document for:	Discussion and decision

- Introduction

This discussion paper discusses two topics:

- Cause value related to time issue

- Reporting Duration for feedback and UE trajectory

- Indication for the case that node cannot provide feedback as configured

- Periodic reporting of the UE Performance Feedback

- Discussion

  - Cause value related to time issue

Agree to introduce cause value(s) indicating failures due to timing issues. Further discussions are needed on which timing issues to address.

Last meeting we decided to focus on introducing cause values indicating failures due to timing issues, e.g., not supported periodicity, or not supported prediction time. However, the requesting node cannot get aware when to send the request again by receiving the “not support” cause value.

To make request messages more efficient, one possible way is that  the requested node also provides available time as assistance information along with the failure cause value such as “Measurement Temporarily not Available”. The failure may be caused by the prediction time or the periodicity. By this way, the requested node can inform the requesting node when it can resend the orginal request again for the failed measurements.

Proposal 1: Introduce avaliable time information with the failure cause value “Measurement Temporarily not Available” in response message to inform the requesting node when to resend the request for the failed measurements.

  - Indication for UE handover to other cell

If the UE moves to Idle/Inactive or it hands over to another cell during the Reporting Duration, collection of UE Performance Feedback is terminated. Whether an explicit or implicit indication on the latter events or any other events is signalled to the requesting node needs further discussions.

For both one-shot reporting and periodic reporting, it is reasonable that the requested node sends the feedbacks to the requesting node immediately once the collection of UE performance is terminated, although sending it later also works. We do not see any necessity for the requesting node to distinguish whether the feedback collection is terminated due to the expiry of the prior-configured timer, or the expiry of prior-configured counter, or a controlled RRC state transition, or a HO. So, there is no need to introduce an enhancement for identify UE’s normal action (move to idle/inactive or HO to other cell).

Nevertheless, there is one case worthy to distinguish: A RLF or other abnormal behaviour, if happens, implies that the previous AI-based decision is not appropriate. This information is useful for the requesting node to optimize the model and to make better decision.

Proposal 2: No need to introduce an indicator for UE normal RRC state transfer or HO.

Proposal 3: Introduce an indicator for indicate UE abnormal behaviour, e.g., RLF.

  - Reporting Duration for feedback and UE trajectory

The use of the time duration, configured by the source node, is almost the same for UE performance feedback and UE trajectory feedback: indicating for how long the requesting node collects the requested information (i.e. when the timer expires the collection stops). So we suggest merging the discussions of the two topics. The next question is whether we need two separate IEs for UE performance feedback and UE trajectort respectively. We think including one unified IE is sufficient enough, for a simiar reason explained in Section 2.4.

Proposal 4: Introduce a “Reporting Duration” IE in to define the duration for UE performance feedback and UE trajectory feedback.

  - Periodic reporting of the UE Performance Feedback

It needs to be further discussed whether to reuse Reporting Periodicity or to introduce a new dedicated Reporting Period for periodic reporting of the UE Performance Feedback

In current mechanism, each DATA COLLECTION REQUEST message is identified by a unique “Measurement ID” pair. For convenience we assume that one data collection context is established and maintained for each “Measurement ID” pair. All of the data collection objects configured by the Report Characteristics IE, associated to the same “Measurement ID” pair, follow the same report Periodicity.

The feedbacks are triggerd by the HANDOVER REQUEST message: when a “Measurement ID” pair is include the HANDOVER REQUEST message, the requested node will carry out the feedback reporting to the requesting node according to the configuration provided in the initiation procedure.

In previous meetings, two issues were raised about how to make the configuration suitable for feedback:

- How to configure the reporting periodic for UE performance feedback, that means, whether the predicity of feedback and other predicated indicate in the report charateristics is always the same. One potential solution is to introduce a new dedicated Reporting Period for periodic reporting of the UE Performance Feedback.

- How to request different feedback metrics for different UE/HO. Different feedback may be needed for different UEs/HOs due to different service types or other requirement. One potential solution is to add a Report Characteristics IE in the HANDOVER REQUEST message.

We try to design a unified solution for the above two issues: triggering multiple Data Collection Reporting Initiation procedure to establish multiple data collection context, and including a list of measurement ID pairs in the HANDOVER REQUEST message.

By sending multiple DATA COLLECTION REQUEST message using differenet measurement ID pairs, the requesting node can configure the Report Characteristics with different combaination and different Reporting Periodicity. An extreme example is that the requesting node configures eight report characteristics by eight DATA COLLECTION REQUEST message. By this way, each characteristic has its own Reporting Periodicity.

What’s more, the above solution can also be used when a UE subject of multiple AI/ML models. Thus, in one HANDOVER REQUEST message, it should include the needed feedback for all the AI/ML models, that to say, feedback with different pair of measurement ID.

The whole procedure as below:

Step 1-3: Node A configures Report Characteristics Average UE Throughput UL, Average Packet Delay, Average Packet Loss, and the predicetion mertices by 4 DATA COLLECTION REQUEST message.

Step 4: Node A decided to HO the UE to the node B, as UE is a URLLC UE, Node A is concerned about the average pack delay and the average date packet loss.

Step 5: Node A sends HANDOVER REQUEST message to the Node B, carried the measure ID pairs of the average pack delay and the average date packet loss, that is, measurement ID pair 2 and measurement ID pair 3.

Step 6-7: Node B sends the DATA COLLECTION UPDATE message to the node A including the average pack delay and the average date packet loss sperately, respectively.

Observation 1: Using multiple DATA COLLECTION REQUEST message and including a list of measurement ID pairs in the HANDOVER REQUEST message can realise the two requirements:

- Configure a reporting periodicity for UE performance feedback different from other requested characters.

- Request different feedback metrics for different UE/HO, including the case that one UE/HO is subject of multiple AI/ML models.

Proposal 5: Introduce a list of measurement ID pairs, rather than only one pair, into the Handover Request message.

  - Semantic Description of Requested Prediction Time

The use of the Requested Prediction Time IE is quite complex and was ever one key issue subject of clarification. In the current BL CR its semantic description is in the tabular, but we consider it better to be moved into the procedure text, with one purpose to clarify its interaction with the update procedure, and the other purpose to make the tabular shorter.

Proposal 6: Converse the semantic description of the Requested Prediction Time IE in the tabular to procedure description in the procedure text.

- Conclusion

Proposal 1: Introduce avaliable time information with the failure cause value “Measurement Temporarily not Available” in response message to inform the requesting node when to resend the request for the failed measurements.

Proposal 2: No need to introduce an indicator for UE normal RRC state transfer or HO.

Proposal 3: Introduce an indicator for indicate UE abnormal behaviour, e.g., RLF.

Proposal 4: Introduce a “Reporting Duration” IE in to define the duration for UE performance feedback and UE trajectory feedback.

Observation 1: Using multiple DATA COLLECTION REQUEST message and including a list of measurement ID pairs in the HANDOVER REQUEST message can realise the two requirements:

- Configure a reporting periodicity for UE performance feedback different from other requested characters.

- Request different feedback metrics for different UE/HO, including the case that one UE/HO is subject of multiple AI/ML models.

Proposal 5: Introduce a list of measurement ID pairs, rather than only one pair, into the Handover Request message.

Proposal 6: Converse the semantic description of the Requested Prediction Time IE in the tabular to procedure description in the procedure text.

- Annex: TP for TS 38.423 on R3-233535

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

8.4.AA	Data Collection Reporting Initiation (FFS on the name)

8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the DATA COLLECTION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all of the requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable to provide some but not all of the requested information, it shall initiate the AI/ML related information reporting for the admitted requested information and include the Node Measurement Initiation Result IE or the Per Cell Measurement Initiation Result IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the DATA COLLECTION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements or prediction on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Radio Resource Status” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”. FFS on the details of Predicted Radio Resource Status IE.

-	the Predicted Number of Active UEs IE, if the second bit, “Predicted Number of Active UEs” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”;

-	the Predicted RRC Connections IE, if the third bit, “Predicted RRC Connections” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average UE Throughput DL IE, if the fourth bit, “Average UE Throughput DL” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average UE Throughput UL IE, if the fifth bit, “Average UE Throughput UL” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average Packet Delay IE, if the sixth bit, “Average Packet Delay” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Average Packet Loss IE, if the seventh bit, “Average Packet Loss” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

-	the Energy Cost IE, if the eighth bit, “Energy Cost” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

If the Requested Prediction Time IE is included in the DATA COLLECTION REQUEST message, and the Reporting Periodicity IE is not included in the DATA COLLECTION REQUEST message, the predicted information included in the DATA COLLECTION UPDATE message shall be the prediction for the point of time one Requested Prediction Time later than the the reception of the DATA COLLECTION REQUEST message.

If the Requested Prediction Time IE is included in the DATA COLLECTION REQUEST message, and the Reporting Periodicity IE is included in the DATA COLLECTION REQUEST message, the predicted information included in the first DATA COLLECTION UPDATE message sent according to the DATA COLLECTION REQUEST message shall be the prediction for the point of time one Requested Prediction Time later than the the reception of the DATA COLLECTION REQUEST message, whereas the predicted information included in the DATA COLLECTION UPDATE message other than the first one shall be the prediction for the point of time one Reporting Periodicity later than the point of time for which the prediction in the last DATA COLLECTION REQUEST message is.

9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x |  | YES | ignore |
| Data Collection Measurement ID To Setup(FFS on the name) | O |  | 9.2.3.M |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

9.1.3.CC	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (0..60, ...) | Indicates the point in time for which predictions are provided. | YES | Ignore |
| Time Duration | O |  | ENUMERATED (0, 1s, 2s, 5s, 10s, 20s, 30s, 40s, 50s, 60s, ...) | Measurements should be reported at/before the Time Duration IE indicate.. | YES | Ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.2.3.M	Data Collection Measurement ID To Setup List (FFS on the name)

This IE indicates the NG-RAN Node Measurement IDs which identify an AI/ML Information Reporting (name is FFS) context.

| IE/Group Name | Presence | Range |  | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| Data Collection Measurement ID To Setup Item |  | 1..<maxnoofDataCollection> |  |  |  |
| >NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) |  | Together with NG-RAN node2 Measurement ID, identifies an AI/ML Information Reporting (name is FFS) context. |
| >NG-RAN node2 Measurement ID (FFS on the name) | M |  |  | INTEGER (1..4095,...) | Together with NG-RAN node1 Measurement ID, identifies an AI/ML Information Reporting (name is FFS) context. |

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

HashedUEIdentityIndexValue,

CellBasedUETrajectoryPrediction,

DataCollectionMeasID-ToSetup-List,

RequestedPredictionTime,

NodeMeasurementInitiationResult-List,

CellMeasurementInitiationResult-List,

UEAssociatedInfoResult,

EnergyCost,

TimeDuration

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-HashedUEIdentityIndexValue,

id-CellBasedUETrajectoryPrediction,

id-DataCollectionMeasID-ToSetup-List,

id-RequestedPredictionTime,

id-NodeMeasurementInitiationResult-List,

id-CellMeasurementInitiationResult-List,

id-UEAssociatedInfoResult,

id-EnergyCost,

id-TimeDuration,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional }|

{ ID id-CHOinformation-Req					CRITICALITY reject	TYPE CHOinformation-Req							PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized				CRITICALITY ignore	TYPE NRV2XServicesAuthorized					PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized			CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional }|

{ ID id-PC5QoSParameters					CRITICALITY ignore	TYPE PC5QoSParameters							PRESENCE optional }|

{ ID id-MobilityInformation					CRITICALITY ignore	TYPE MobilityInformation						PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE			PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject	TYPE IABNodeIndication							PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore	TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional }|

{ ID id-QMCConfigInfo						CRITICALITY ignore	TYPE QMCConfigInfo								PRESENCE optional }|

{ ID id-FiveGProSeAuthorized				CRITICALITY ignore	TYPE FiveGProSeAuthorized						PRESENCE optional }|

{ ID id-FiveGProSePC5QoSParameters			CRITICALITY ignore	TYPE FiveGProSePC5QoSParameters				PRESENCE optional }|

{ ID id-CellBasedUETrajectoryPrediction		CRITICALITY ignore	TYPE CellBasedUETrajectoryPrediction		PRESENCE optional }|

{ ID id-DataCollectionMeasID-ToSetup-List	CRITICALITY ignore	TYPE DataCollectionMeasID-ToSetup-List	PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- DATA COLLECTION REQUEST

--

-- **************************************************************

DataCollectionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionRequest-IEs}},

...

}

DataCollectionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID				CRITICALITY ignore	TYPE Measurement-ID					PRESENCE conditional}|

-- This IE shall be present if the Registration Request IE is set to the value "stop".

{ ID id-RegistrationRequest					CRITICALITY reject	TYPE RegistrationRequest			PRESENCE mandatory}|

{ ID id-ReportCharacteristics					CRITICALITY reject	TYPE ReportCharacteristics			PRESENCE conditional}|

-- This IE shall be present if the Registration Request IE is set to the value "start".

{ ID id-CellToReport						CRITICALITY ignore	TYPE CellToReport					PRESENCE optional}|

{ ID id-ReportingPeriodicity					CRITICALITY ignore	TYPE ReportingPeriodicity			PRESENCE optional}|

{ ID id-RequestedPredictionTime					CRITICALITY ignore	TYPE RequestedPredictionTime		PRESENCE optional}|

{ ID id-TimeDuration					CRITICALITY ignore	TYPE TimeDuration		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Active-MBS-SessionInformation ::= SEQUENCE {

mBS-QoSFlowsToAdd-List								MBS-QoSFlowsToAdd-List,

mBS-ServiceArea										MBS-ServiceArea												OPTIONAL,

mBS-MappingandDataForwardingRequestInfofromSource	MBS-MappingandDataForwardingRequestInfofromSource		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Active-MBS-SessionInformation-ExtIEs} }		OPTIONAL,

...

}

Active-MBS-SessionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofNeighbour-NG-RAN-Nodes,

maxnoofSRBs,

maxnoofSMBR,

maxnoofNSAGs,

maxnoofRBsetsPerCell1,

maxnoofTargetSNsMinusOne,

maxnoofThresholdsForExcessPacketDelay,

maxnoofCellsTrajectoryPredict,

maxFailedMeasObjects,

maxFailedMeasPerNode,

maxnoofUEReports,

maxnoofDataCollection

FROM XnAP-Constants

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- D

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

DataCollectionMeasID-ToSetupList ::= SEQUENCE (SIZE(1..maxnoofDataCollection)) OF DataCollectionMeasID-ToSetup-Item

DataCollectionMeasID-ToSetup-Item ::= SEQUENCE {

nG-RAN-Node1MeasID				INTEGER (1..4095),

nG-RAN-Node2MeasID				INTEGER (1..4095),

iE-Extensions					ProtocolExtensionContainer { { DataCollectionMeasID-ToSetup-Item-ExtIEs} } 	OPTIONAL,

...

}

DataCollectionMeasID-ToSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TimeDuration ::= ENUMERATED {ms0, ms10000, ms20000, ms50000, ms100000, ms200000, ms300000, ms400000, ms500000, ms600000, ...}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- N

NG-RANnode2SSBOffsetsModificationRange ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBOffsetModificationRange

NG-RANnodeUEXnAPID ::= INTEGER (0.. 4294967295)

NumberofActiveUEs::= INTEGER(0..16777215, ...)

NodeMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxFailedMeasPerNode)) OF NodeMeasurementInitiationResult-Item

NodeMeasurementInitiationResult-Item ::= SEQUENCE {

measurementFailedReportCharacteristics			BIT STRING(SIZE(128)),

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { { NodeMeasurementInitiationResult-Item-ExtIEs} } OPTIONAL,

...

}

NodeMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NoofRRCConnections ::= INTEGER (1..65536,...)

9.3.7	Constant definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-dataCollectionReportingInitiation							ProcedureCode ::= xx

id-dataCollectionReporting										ProcedureCode ::= yy

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofSMBR									INTEGER ::= 8

maxnoofNSAGs								INTEGER ::= 256

maxnoofTargetSNsMinusOne					INTEGER ::= 7

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofCellsTrajectoryPredict				INTEGER ::= FFS

maxFailedMeasObjects						INTEGER ::= 124

maxFailedMeasPerNode						INTEGER ::= 124

maxnoofUEReports							INTEGER ::= FFS

maxnoofDataCollection						INTEGER ::= FFS

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																		ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-CellBasedUETrajectoryPrediction																	ProtocolIE-ID ::= xx1

id-DataCollectionMeasID-ToSetup-List										ProtocolIE-ID ::= xx2

id-RequestedPredictionTime																			ProtocolIE-ID ::= xx3

id-NodeMeasurementInitiationResult-List																ProtocolIE-ID ::= xx4

id-CellMeasurementInitiationResult-List																ProtocolIE-ID ::= xx5

id-UEAssociatedInfoResult																			ProtocolIE-ID ::= xx6

id-EnergyCost																						ProtocolIE-ID ::= xx7

id-TimeDuration																						ProtocolIE-ID ::= xx8

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--