3GPP TSG-RAN WG3 Meeting #121bis	R3-235176

Xiamen, China, 09-13 Oct, 2023

Title: 	(TP to Netw_Energy_NR BLCR for TS 38.473, 38.413, 38.470 and 38.300) Network energy saving techniques

Source: 	Huawei

Agenda item:	24.2

Document Type:	Other

# 1. Introduction

At the last RAN3#121 meeting, the following agreements and remaining issues on NES were captured in [2]:

- Cause value for SSB activation failure: Introduce a new cause value “SSB not Available” in XnAP and F1AP to indicate that none of the requested SSB beam(s) can be successfully activated. Editor notes in F1AP and XnAP - Maximum SSB areas is 64 - In F1AP BLCR, keep the criticality “reject” for the SSBs within the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message, and remove the FFS. Paging enhancements: To support CN paging enhancement, the recommended SSB list for Paging could be transferred over NG in the UE context release message? Work on reply LS to SA2. Take the texts in the SoD as baseline, and send the draft reply LS to SA2 in Oct. meeting when we receive RAN2 reply LS. Working assumption: To support CN paging enhancement, the recommended SSB list for Paging could be transferred in the NGAP UE context release complete message, and sent back in the NGAP Paging message in the transparent container Cell DTX/DRX WA: Support the exchange of the Cell DTX/DRX configuration over Xn. FFS on the activation/deactivation status exchange.

In this paper, we continue to discuss the remaining open issues for NES. The TPs and reply LS are provided in the Annex.

# 2. Discussion

## 2.1 Inter-node beam activation

- Beams deactivation: Whether to introduce a cause for beams deactivation (e.g. ‘energy saving’)

This was discussed at previous RAN3-121 meeting, but without conclusion. In current F1AP specification, the DU will send the SSB Coverage State to CU (through Coverage Modification Notification IE in GNB-DU CONFIGURATION UPDATE message) when modifying the coverage of an SSB beam. Similarly, over Xn, a NG-RAN node will send the SSB Coverage State to the peer node (through Coverage Modification List IE in NG-RAN NODE CONFIGURATION UPDATE message) when modifying the coverage of an SSB beam. In both of the above cases, if the SSB beam is deactivated, the corresponding SSB Coverage State is set to “0”.

The modification of SSB beam coverage could be due to e.g. CCO or NES. It would be beneficial to indicate the detailed cause over the network interfaces. For example, if the SSB beam deactivation (i.e. SSB Coverage State = “0”) is due to NES, the peer node may request the activation of this SSB beam when the its load becomes high; while if the SSB beam deactivation is due to CCO (high interference), the peer node will not request the activation of this SSB beam, as this will increase the interference and reduce the overall capacity. Hence, a cause value e.g. “network energy saving” is beneficial.

To indicate the “network energy saving” cause, there are two signalling options in F1AP.

- Option 1: updating the Coverage Modification Notification to include the new cause.  This cause could only be used for the deactivation case (i.e. SSB Coverage State = “0”), but not for other cases (i.e. SSB Coverage State = 1, 2, …).

- Option 2: introducing a new IE containing the new cause value and a list SSBs suggested to be deactivated.

Option 1 is preferred since it is simple to update the exiting IE, and notify the cause in one unified IE.

While for XnAP, the existing Coverage Modification Cause can be enhanced to indicate “network energy saving” for SSB beam deactivation.

Introduce a new cause in the Coverage Modification Notification for F1AP and in the Coverage Modification List for XnAP to indicate “network energy saving” for SSB beam deactivation.

- SSB bitmap update: Whether to update the SSB bitmap when SSB coverage status changed

It was discussed whether to update the SSB bitmap over F1AP, upon SSB (de) activation and/or SSB coverage state update.

Whether there is a need to always update SSB bitmap (sent by gNB-DU in Served Cell Information and then broadcasted in SIB1) upon every SSB (de) activation. The following options can be considered: • Option 1: SSB bitmap is always updated upon SSB (de)activation along with updating SSB Coverage State and SSBs Activated List • Option 2: There is no need to always update SSB bitmap upon every SSB (de)activation. Just changing the “SSB coverage state” or “SSB Activated List” is sufficient

First this seems not a Rel-18 NES issue, given that the CCO feature was supported even from the previous release.

Second, there should not have any impact on the UE side. That is, each time the SSB (de)activation and/or SSB coverage state changes (especially for SSB inactive case), the SIB1 information should be updated.  This allows the consistency between the UE and network avoid any measurement failure or signal receiving errors.

Then over F1 interface, we think both options could work, as long as it is clear for the CU to be aware of the SSB activation/deactivation state. It can be either up to the DU to report both, or only the SSB coverage state/SSB activated list. Option 1 is slightly preferred to have the safest solution. And we don’t see the need to specify in the specification.

Both could work, option 1 can be considered the safest case, i.e., to update the SSB bitmap when SSB coverage status/SSB activation changed.

## 2.2 Paging enhancements

In last meeting, the questions proposed in the replied SA2 LS were discussed. Due to the fact that RAN2 has not replied the LS, the initial answers to SA2 are captured as baseline in the following [3].

[Answer to the assumption]: RAN3 confirms and expects the AMF can do the paging escalation based on the existing mechanisms. – to be revised during the draft view. [Answer to Q1]: NG-RAN can determine the UE mobility state at least based on the subscription information i.e., the "Expected UE behaviour" IE from the AMF. [Answer to Q2]: RAN3 considers that the AMF itself can decide the validity condition of the potential “List of recommended beams”, e.g., the same as the handling of the existing Recommended Cells for Paging, Last Visited Cell Information in TS 38.413. It is up to the AMF implementation to determine to delete it when the UE moves to the connected state, or under other conditions. –can be revisited during the draft view.

Generally, the legacy mechanism can be utilized to solve the issues proposed by SA2. If there are no problems about paging in a limit SSBs from RAN2’s perspective, RAN3 can reply to SA2 with the above baseline answers.

Once RAN2 finds no issues about the paging enhancement, RAN3 can reply to SA2 with the above baseline answers.

CN paging

The work assumption about CN paging was made in last meeting.

Working assumption: To support CN paging enhancement, the recommended SSB list for Paging could be transferred in the NGAP UE context release complete message, and sent back in the NGAP Paging message in the transparent container

In order to maximize the NES gain, the paging enhancement should also be applied for CN paging. As a reminder, the questions in the LS sent to RAN2 was not related to the issues about CN paging problem. Since we have agreed RAN paging enhancement and SA2 found no risks about CN paging, the CN paging enhancement should be supported. In order to support CN paging, turn the work assumption to agreement.

In order to support the SSB level paging enhancement, i.e., allow the exchange of Recommended SSBs for Paging over NG. Specifically, Information on Recommended Cells and RAN Nodes for Paging and Assistance Data for Paging could include the info of Recommended SSBs, including: 1) a list of visited and non-visited SSBs, and 2) the time a UE stayed in a visited SSB.

In the disaggregated gNB case, upon the reception of the recommended SSBs for Paging from the AMF, the gNB-CU could inform the gNB-DU of the paging SSBs in PAGING, which includes: a list of paging SSBs.

Turn the WA to agreement and the information of recommended SSBs for Paging could be transferred over NG. Specifically,

- Over NG, Information on Recommended Cells and RAN Nodes for Paging and Assistance Data for Paging to include 1) a list of visited and non-visited SSBs, and 2) the time a UE stayed in a visited SSB.

To enable the NG-RAN node to get knowledge of the information of the visited SSBs, the UE History Information transferred over NG and Xn could be extended to include the Last Visited SSB Information, which is similar with the legacy Last Visited Cell Information.

RAN3 to discuss whether the last visited SSB information could be transferred over NG and Xn. Specifically, the stage 3 NG and Xn impacts include:

- UE History Info to include 1) a list of visited SSBs, and 2) the time UE stayed in a visited SSB.

## 2.3 Cell DTX/DRX

RAN2 has agreed that Cell DTX/DRX could be periodic, and the Cell DTX/DRX configuration could contain the periodicity, start slot/offset and on duration. The specific value ranges of cell DTX/DRX configuration parameters have been agreed in last RAN2#123 meeting. The periodicity and on duration time have the same value range as UE C-DRX parameters. Noted that this Cell DTX/DRX is lower layer per cell information and C-DRX parameters are determined by the DU, it follows the same mechanism as C-DRX to enable DU to decide these parameters. As the detailed parameter presence and format are still under RAN2’s discussion, we will update them in the BLCR based on RAN2’s progress.

In the CU/DU split architecture, the Cell DTX/DRX parameters including the periodicity, start slot/offset, on duration are decided by the gNB-DU.

In addition, the working assumption was made at the previous meeting.

- WA: Support the exchange of the Cell DTX/DRX configuration over Xn

It is beneficial to exchange the cell DTX/DRX configurations over Xn, including on-duration time, periodicity and start offset. The information can be used to do the load balance and mobility management. For example, source cell can take the information into consideration when selecting the cell DTX/DRX cell as a candidate cell. Moreover, as we talked before, the configuration parameters are decided by gNB-DU. Thus, the information exchange between CU and DU over F1AP is necessary to support the information exchange over Xn.

Cell DTX/DRX can be activated/deactivated by RRC signalling or L1 group common signalling. We think the necessity and feasibility of the exchange of activation/deactivation status over network interfaces highly depend on how dynamically the cell DTX/DRX configuration could be activated/deactivated. If the activation/deactivation is frequent, it would be problematic to timely notify the activation/ deactivation info to peer node. If the group common L1 signalling is used to lower signalling overhead, and is not dynamic, e.g., in terms of hundreds of microseconds, then the exchange is needed.

Hence, we think it will be finally up to the operator policy and detailed RRM algorithm.

Turn the WA to agreement to exchange the Cell DTX/DRX configuration per cell over Xn and F1 interface. How to use it can be left to the operator policy.

## 2.4 NES mode

For cell selection/re-selection and CHO mechanisms discussed in RAN2, a cell may enter the “NES mode”. In RAN2#122 and RAN2#123 meetings, the following agreements were made:

RAN2#122

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

RAN2#123

Agreements

-	One single bit in SIB1 is introduced for controlling all “NES-capable UEs” to access a cell.  FFS what “NES capable UE” bit means.  The NES UE always follows the NES bit used for barring, if present.  If not present the UE shall follow legacy barring.

-	No new cell baring techniques for non-NES UEs will be specified.

-	No new cell re-selection techniques will be considered in this Rel-18

Agreements

1	We will support the CHO triggers for the use case of turning off the cell

2	(At least for cell DTX/DRX) Time-based CHO is not to be considered in NES.

3	Do not consider using an indication in SIB1 for triggering NES CHO execution condition

When a cell enters or leaves the “NES mode”, it may need to indicate or update its current NES mode to neighbour nodes, which would be helpful for target cell selection during HO. However, so far other group may not consider to introduce the explicit “NES mode” definition.

The NES mode exchange over network interfaces are pending to other groups.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Introduce a new cause in the Coverage Modification Notification for F1AP and in the Coverage Modification List for XnAP to indicate “network energy saving” for SSB beam deactivation.

- Both could work, option 1 can be considered the safest case, i.e., to update the SSB bitmap when SSB coverage status/SSB activation changed.

- Once RAN2 finds no issues about the paging enhancement, RAN3 can reply to SA2 with the above baseline answers.

- Turn the WA to agreement and the information of recommended SSBs for Paging could be transferred over NG. Specifically,

- Over NG, Information on Recommended Cells and RAN Nodes for Paging and Assistance Data for Paging to include 1) a list of visited and non-visited SSBs, and 2) the time a UE stayed in a visited SSB.

- RAN3 to discuss whether the last visited SSB information could be transferred over NG and Xn. Specifically, the stage 3 NG and Xn impacts include:

- UE History Info to include 1) a list of visited SSBs, and 2) the time UE stayed in a visited SSB.

- In the CU/DU split architecture, the Cell DTX/DRX parameters including the periodicity, start slot/offset, on duration are decided by the gNB-DU.

- Turn the WA to agreement to exchange the Cell DTX/DRX configuration per cell over Xn and F1 interface. How to use it can be left to the operator policy.

- The NES mode exchange over network interfaces are pending to other groups.

- The corresponding TP to TS 38.300, TS 38.470, TS 38.473 and TS 38.413 and the draft LS are provided in Annexes.

# 4. Reference

- 3GPP RAN3#120 Chairman’s notes (RAN3_110_agenda_20230530_EOM1.doc)

- 3GPP RAN3#121 Chairman’s notes ( RAN3_121_agenda_20230825_EOM.doc)

- R3-234614, Summary of offline discussions on CB: # R18ES

# Annex 1: TP to 38.300(based on BLCR R3-234825)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 15.4	Support for Energy Saving

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 15.4.2	Solution description

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 15.4.2.aa	Inter-node beam activation/deactivation

The solution builds upon the possibility for the NG-RAN node to request neighboring NG-RAN nodes, e.g., in the capacity layer, to switch on some SSB beams within the cell which are deactivated. The receiving NG-RAN node acts upon the request accordingly. This is achieved via the Cell Activation procedure. The NG-RAN node can be informed by neighbour NG-RAN nodes owning the concerned cell about the switch-off SSB beam actions over the Xn interface, by means of the NG-RAN node Configuration Update procedure, before triggering the beam activation procedure.

### 15.4.2.bb	Restricting Paging in a limited area

The solution builds upon the possibility for the NG-RAN node to page e.g. stationary UEs in certain SSB(s) instead of all the SSBs within a cell. This mechanism is applicable for UEs in both RRC inactive and idle state. It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement mechanism.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 2: TP to BLCR for 38.470

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 5.1	General

The following clauses describe the functions supported over F1-C and F1-U.

## 5.2	F1-C functions

### 5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, may activate or deactivate cells, or activate SSBs of cells. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. With the gNB-CU Configuration Update function, energy saving with cell activation/deactivation can be supported as defined in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.2.5	Paging function

The gNB-CU is responsible for filtering target cells for paging based on the UE Radio Capability for Paging. The gNB-CU may filter SSBs of the target cells for paging for network power saving.

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters provided. The gNB-DU also takes the UE paging capability into account for paging when provided by the gNB-CU.

The gNB-CU provides paging information to enable the gNB-DU to calculate the exact PH, if the eDRX is configured, PO and PF. The gNB-CU determines the PA. The gNB-DU consolidates all the paging records for a particular PH, PO, PF and PA, and encodes the final RRC message and broadcasts the paging message on the respective PH, PO, PF in the PA.

The paging function also supports CN controlled subgrouping paging for UE Power Saving.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 3: TP to 38.473 (based on BLCR R3-234784)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the F1 SETUP REQUEST message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the RedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the Cell DTX/DRX configuration IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may take this IE into account for network energy saving.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization or Network Energy Saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the Cell DTX/DRX configuration IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may take this IE into account for network energy saving.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination and/or network energy saving. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE and Cell DTX/DRX configuration IE are absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI. If the Cell DTX/DRX configuration IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall, if supported, use the received information to determine its cell DTX/DRX configuration for network energy saving.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.3.3	UE Context Release (gNB-CU initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. If the CG-SDT Kept Indicator IE is contained in the UE CONTEXT RELEASE COMMAND message and set to "true", the gNB-DU shall, if supported, consider that the UE is sent to RRC_INACTIVE state with CG-SDT configuration and store the configured CG-SDT resources, C-RNTI, CS-RNTI, the CG-SDT related RLC configurations and F1-U connections associated with the SDT bearers while releasing the UE context.

If the old gNB-DU UE F1AP ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall additionally release the UE context associated with the old gNB-DU UE F1AP ID.

If the UE CONTEXT RELEASE COMMAND message contains the RRC-Container IE, the gNB-DU shall send the RRC container to the UE via the SRB indicated by the SRB ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider that the gNB-CU is cancelling only the conditional handover or conditional PSCell addition or conditional PSCell change associated to the cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

If the Recommended SSBs for Paging List IE is included in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store it and may use it as assistance information for subsequent paging.

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List |  | 0 .. < maxnoofSSBAreas> |  | List of SSB beams within the cell requested to be activated | EACH | reject |
| >>>SSB Index | M |  | INTEGER (0..63) | Identifier of SSB beam requested to be activated | - |  |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >> E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>> E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| >>Cell DTX/DRX configuration | O |  | 9.3.1.aaa |  | YES | ignore |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | Ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| <<<<<<<<<<<<<<<<<<<< Unmodified Texts Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| Cell DTX/DRX configuration | O |  | 9.3.1.aaa |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  | - |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>SSB Inactivation Cause | O |  | ENUMERATED (network energy saving, …) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aaa	Cell DTX/DRX configuration

This IE contains the Cell DTX/DRX configuration information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CellDTX-Config | O |  | OCTET STRING | Includes the CellDTX-Config IE, as defined in TS 38.331 [8]. |
| CellDRX config | O |  | OCTET STRING | Includes the CellDRX-Config IE, as defined in TS 38.331 [8]. |

Editor’s Note: The presence and format of Cell DTX/DRX configuration will be updated based on RAN2’s progress.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

CellGroupConfig ::= OCTET STRING

CellCapacityClassValue ::= INTEGER (1..100,...)

CellDTXDRXConfiguration	::= SEQUENCE {

cellDTXConfig				CellDTX-Config		OPTIONAL,

cellDRXConfig				CellDRX-Config		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CellDTXDRXConfiguration-ExtIEs} }	OPTIONAL,

}

CellDTXDRXConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellDTX-Config	::= OCTET STRING

CellDRX-Config	::= OCTET STRING

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

cellCoverageState					CellCoverageState,

sSBCoverageModificationList			SSBCoverageModification-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

sSBInactivationCause			SSBInactivationCause 	OPTIONAL,

...

}

CellCoverageState ::= INTEGER (0..63, ...)

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional }|

{	ID id-CellDTXDRXConfiguration		CRITICALITY ignore	EXTENSION CellDTXDRXConfiguration 	PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SSBAreaRadioResourceStatusItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBInactivationCause  ::= ENUMERATED { network-energy-saving,...}

SSBInformation ::= SEQUENCE {

sSBInformationList	SSBInformationList,

iE-Extensions		ProtocolExtensionContainer { { SSBInformation-ExtIEs } }	OPTIONAL

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-CellDTXDRXConfiguration 							ProtocolIE-ID ::= xxx

id-SSBInactivationCause								ProtocolIE-ID ::= yyy

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 4. TP to TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.5.1	Paging

#### 8.5.1.1	General

The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

If the Paging Cause IE is included in the PAGING message, the NG-RAN node shall, if supported, transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the PEIPS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [8].

If the Assistance Data for Recommended SSBs IE is included in the Assistance Data for Paging IE, the NG-RAN node may take it into account when determining the cells and SSBs where paging will be performed.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.69	Assistance Data for Paging

This IE provides assistance information for paging optimisation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Assistance Data for Recommended Cells | O |  | 9.3.1.70 |  | - |  |
| Paging Attempt Information | O |  | 9.3.1.72 |  | - |  |
| NPN Paging Assistance Information | O |  | 9.3.1.183 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| Assistance Data for Recommended SSBs | O |  | 9.3.1.aaa |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| Last Visited SSB List |  | 0..< maxnoofSSBsinUEHistoryInfo > |  | List of visited SSBs. Most recent SSB related information is added to the top of the list. | YES | ignore |
| >Last Visited SSB Information | M |  | 9.3.1.ccc | The SSB related information | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |
| maxnoofSSBsinUEHistoryInfo | Maximum number of last visited SSB information records that can be reported in the IE. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.100	Information on Recommended Cells and RAN Nodes for Paging

This IE provides information on recommended cells and NG-RAN nodes for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Cells for Paging | M |  | 9.3.1.71 |  |
| Recommended RAN Nodes for Paging | M |  | 9.3.1.101 |  |
| Recommended SSBs for Paging | O |  | 9.3.1.bbb |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aaa	Assistance Data for Recommended SSBs

This IE provides assistance information for paging in recommended SSBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended SSBs for Paging | M |  | 9.3.1.bbb | Includes visited and non-visited SSBs, where visited SBBs are listed in the order the UE visited them with the most recent SSB being the first in the list. Non-visited SSBs are included after the visited SSB they are associated with. |

#### 9.3.1.bbb	Recommended SSBs for Paging

This IE contains recommended SSBs for paging.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended SSB List |  | 1 |  |  |
| >Recommended SSB Item |  | 1..<maxnoofRecommendedSSBs> |  | Includes visited and non-visited SSBs, where visited SBBs are listed in the order the UE visited them with the most recent SSB being the first in the list. Non-visited SSBs are included after the visited SSB they are associated with. |
| >>NG-RAN CGI | M |  | 9.3.1.73 |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |
| >>Time Stayed in SSB | O |  | INTEGER (0..4095) | This is included for visited SSBs and indicates the time a UE stayed in an SSB in seconds. If the UE stays in an SSB more than 4095 seconds, this IE is set to 4095. (Value FFS) |

| Range bound | Explanation |
| --- | --- |
| maxnoofRecommendedSSBs | Maximum no. of recommended SSBs. Value is 64. |

#### 9.3.1.ccc	Last Visited SSB Information

The Last Visited SSB Information may contain SSB specific information of a cell.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SSB Index | M |  | INTEGER (0..63) |  |
| Time Stay | O |  | INTEGER (0..4095) | This is included for visited SSBs and indicates the time a UE stayed in an SSB in seconds. If the UE stays in an SSB more than 4095 seconds, this IE is set to 4095 (Value FFS). |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 5. Draft LS

# Title:	[Draft] Reply LS on the enhancements to restricting paging in a limited area

# Response to:	S2-2307984/R3-233729

# Release:	Rel-18

# Work Item:	Netw_Energy_NR-Core

Source:	Huawei [will be RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Feng Han

E-mail Address:	hanfeng3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:        -

1. Overall Description:

RAN3 would like to thank SA2 for their reply LS on the enhancements to restricting paging in a limited area.

RAN3 discussed the questions and would like to provide the following answers:

Assumption validation: SA2 would also like to validate the following assumption: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how paging escalation happen will be controlled by the AMF based on existing procedures.

Answer: RAN3 confirms and expects the AMF can do the paging escalation based on the existing mechanisms.

Q1: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation?

Answer: NG-RAN can determine the UE mobility state at least based on the subscription information i.e., the "Expected UE behaviour" IE from the AMF.

Q2: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"?

Answer: RAN3 considers that the AMF itself can decide the validity condition of the potential “List of recommended beams”, e.g., the same as the handling of the existing Recommended Cells for Paging, Last Visited Cell Information in TS 38.413. It is up to the AMF implementation to determine to delete it when the UE moves to the connected state, or under other conditions.

2. Actions:

To SA2

ACTION: 	RAN3 kindly asks SA2 to take above information into account.

3. Date of Next TSG RAN3 Meetings:

3GPP TSG RAN WG3#122	13 – 17 November, 2023		Chicago, United States

3GPP TSG RAN WG3#123	26 February – 1 March, 2024		Athens, Greece