3GPP TSG-RAN WG3 Meeting #121bis	R3-235183

Xiamen, China, 09-13 Oct, 2023

Title: 	(TPs to MBS BL CRs) MBS reception in RAN sharing scenario

Source: 	Huawei, CBN

Agenda item:	15.2

Document Type:	other

# 1. Introduction

In RAN3#121 meeting, the following agreements/WAs/FFSs were achieved:

Introduce Broadcast session transport request/response/failure procedure to support NG-RAN triggered NG-U tunnel establishment.

CU-CP makes decision on whether to establish NG-U tunnel.

For MOCN scenario, only one set of shared F1-U tunnels is established and kept for MOCN scenario as long as there is one PLMN keeping the MBS service.

For multiple cell-ID broadcast scenario, the entity controlling logical DUs decides how many F1-U tunnels to be set up. The decision of CU-CP on establishment of NG-U tunnel takes the feedback of DU on establishment of a set of F1-U tunnels into account.

FFS whether the set of F1-U tunnel(s) and NG-U tunnel are always one to one mapping or not.

Adopt the option of Multiple F1AP contexts/messages, one per PLMN, for MOCN scenario.

Associated session ID and MBS service area should be introduced in E1AP for shared NG-U termination scenario (regardless of whether we have one or multiple broadcast bearer context on E1 for RAN sharing).

Multiple E1AP context/message, one per PLMN, for MOCN scenario.

Identify the issue first, then discuss whether both of the options are supported

Option 1: OAM configures the MRB-PDCP-ConfigBroadcast of each MRB.

Option 2: DU makes decision on which MRB-PDCP-ConfigBroadcast it used.

For option 2: whether to provide the used MRB-PDCP-ConfigBroadcast to CU and whether CU could reject and fall back to Rel-17 behaviour.

In this contribution, we will further analysis on the support of MBS reception in RAN sharing efficiency optimization for the broadcast MBS service from different perspectives and provide the corresponding TPs to the BL CRs.

# 2. Discussion

## 2.1 Details of Associated Session ID

In the previous meetings, it has been agreed to add Associated Session ID IE in the same level of MBS Session ID IE in the related NG/F1 messages, but the detailed encoding of the Associated Session ID IE is still FFS.

As defined in TS23.247, the Associated Session ID includes two IP addresses:

6.5.5 Associated Session ID In the case of network sharing, an Associated Session ID may be used as specified in clause 6.17. When the AF creates multiple broadcast MBS Sessions via different CNs to deliver the same content, it may provide the Associated Session ID which enables the NG-RAN to identify the multiple MBS Sessions delivering the same content. Source Specific IP Multicast Address specified in clause 6.5.3 may be used as Associated Session ID.

### 6.5.3 Source Specific IP Multicast Address The source specific IP multicast address is used to identify an Multicast MBS session and consists of two IP addresses, one is an IP unicast address used as source address in IP packets for identifying the source of the multicast service (e.g. AF/AS), the other is an IP multicast address used as destination address in related IP packets for identifying a multicast communication service associated with the source.

Proposal 1: Define the Associated Session ID IE as a sequence type including two IP addresses, i.e. source address (IP unicast address) and destination address (IP multicast address).

## 2.2 Further consideration in case of RAN Sharing with multiple cell-ID broadcast

### Mapping relationship between the set of F1-U tunnel and NG-U tunnel

In the last meeting, it has been agreed that CU-CP makes decision on whether to establish NG-U tunnel, and for multiple cell-ID scenario, the entity controlling logical DUs decides how many F1-U tunnels to be set up. There was an issue to be continued marked in the last meeting:

FFS whether the set of F1-U tunnel(s) and NG-U tunnel are always one to one mapping or not.

For non-sharing scenario, in order to send the broadcast data in the air interface successfully, the NG-U tunnel and F1-U tunnel shall be setup in the Broadcast MBS Session Setup procedure according to TS 38.401. For RAN Sharing with multiple cell-ID broadcast scenario, there may be two kinds of relationship between the set of NG-U tunnel and F1-U tunnel as follows.

Option 1: F1-U tunnel and NG-U tunnel are always established together, i.e. one to one mapping.

Option 2: Up to gNB-DU and gNB-CU implementation

For option 1, once the Broadcast MBS Session Setup procedure starts, the NG-U tunnel and F1-U tunnel will be established together except the failure case. In other words, if the gNB DU decides to establish the F1-U tunnel, the gNB (gNB CU-CP) would probably decide to establish the corresponding NG-U tunnel. If the gNB-DU decides not to establish the F1-U tunnel, the gNB needs to take this into account and should decide not to establish the corresponding NG-U tunnel.

For option 2, considering that it’s the gNB DU’s responsibility to decide whether to establish the F1-U tunnel and it’s the gNB-CU’s (gNB CU-CP) responsibility to decide whether to establish the NG-U tunnel, there may be a case that the gNB DU decides not to establish the F1-U tunnel with a gNB-CU, but that gNB CU still decides to establish the corresponding NG-U tunnel. In this case, the gNB DU only needs to handle broadcast data from the gNB-CU which has established both the NG-U and F1-U tunnels, and the other gNB-CUs will need to discard the received packets from their CN.

As option 1 is a subset of option 2, and option 2 allows flexibility for these two nodes’ implementation, we slightly prefer option 2.

Proposal 2: In case of RAN Sharing with multiple cell-ID broadcast, F1-U tunnel and NG-U tunnel mapping is left to gNB-DU and gNB-CU implementation, i.e. the establishment of F1-U tunnel is determined by the gNB-DU, and the establishment of NG-U tunnel is determined by the gNB-CU.

### PDCP configuration

In case of network sharing with multiple cell-ID broadcast, each logical gNB-DU will receive different request messages from the each gNB-CU. Without any coordination/configuration, the received PDCP configurations from different gNB-CUs are probably different. Considering how to decide the MRB PDCP configuration, there was an FFS left:

Identify the issue first, then discuss whether both of the options are supported

Option 1: OAM configures the MRB-PDCP-ConfigBroadcast of each MRB.

Option 2: DU makes decision on which MRB-PDCP-ConfigBroadcast it used.

For option 2: whether to provide the used MRB-PDCP-ConfigBroadcast to CU and whether CU could reject and fall back to Rel-17 behaviour.

From our view, upon receiving different PDCP configurations from different gNB-CUs, the entity controlling multiple logical gNB-DUs chooses the  PDCP configuration to be broadcasted, and it will also use the downlink data from the same gNB-CU over the radio. For example, the gNB-DU can broadcast the PDCP configuration provided by the first gNB-CU and use the data received via the shared F1-U tunnel with the first gNB-CU.

Proposal 3: In case of RAN sharing with multiple Cell ID broadcast, the gNB-DU chooses one of the PDCP configurations from different gNB-CUs, and transmit the DL data received from the related gNB-CU to the UEs.

### Packet Discard issue

Figure 1: Data path change in case of different MRB configurations from different CUs

Based on the above analyses, so far there is no real issue identified in case different PDCP configurations are provided by different gNB-CUs, as shown in the Figure 1 above, different MRB configurations are provided from different gNB-CUs (gNB-CU1 provided MRB configuration A, gNB-CU2 provided MRB configuration B), when we need to change the path, e.g. in case CN 1 triggers BC release, the DU will change the MRB configuration over the MCCH from A to B, and in the meanwhile start to transmit the data received from gNB-CU2 over the radio. From UE point of view, upon receiving the updated MRB configuration, it will release and setup the MRB and receives the data by using the new configuration.

Observation 1: Upon receiving the updated MRB configuration over MCCH, the UE will release and setup the MRB, and therefore it can receive the data using the new configuration correctly.

According to TS 38.323, at the beginning of broadcast service reception, the UE would initiate the PDCP window according to the SN of the first received PDCP Data PDU:

For broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU.

But during the reception of a broadcast MRB, in case the received PDCP Packets are outside of the receiving window (RCVD_COUND < RX_DELIV, or if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before), the UE will discard the received PDCP Data PDUs.

Observation 2: During the reception of a broadcast MRB, the PDCP Data PDU outside receiving window will be discarded by the UE.

In RAN3#121 meeting, it was agreed that “For MOCN scenario, only one set of shared F1-U tunnels is established and kept for MOCN scenario as long as there is one PLMN keeping the MBS service.”, therefore in case new shared NG-U tunnel is established, the gNB-CU can decide to keep the MRB configurations, and uses a continuous PDCP SN for the packets received from the new NG-U tunnel.

However, in case of network sharing with multiple cell-ID broadcast, each gNB-CU will receive broadcast data from their own CN. Then, the PDCP SN determined by different gNB-CUs are probably different, in case the gNB-DU change the F1-U path from gNB-CU1 to gNB-CU2, if different MRB configurations are used by different gNB-CUs, upon receiving the different MRB Configuration over MCCH, the UE will be able to “release and add” the MRB, therefore, it can receive the data correctly. But if same MRB configurations are used before and after path change, the UE will not be able to release and setup the MRB, it will not be able to adjust the initial value of the window based on the newly received PDCP Data, therefore, if the PDCP SN for the packets received via the new path are outside of the receiving window, packet loss will happen.

Figure 2: packet loss in case of data path change without MRB configuration change

Proposal 4:  Confirm the issue about in case same MRB Configurations is used before and after path change, the UE will not be able to “release and add” the broadcast MRB and initialize the receiving window, if the PDCP SN of the packets received via the new path are outside of the receiving window, these packets will be discarded by the UE.

From our understanding, there are several ways to avoid/solve this issue:

Solution 1: keep the PDCP SN continuity

- there are probably no Xn interface between these gNB-CUs belongs to different PLMNs, the MBS QFI SN from different CN are probably different.

- we may need to further consider how to coordinate among these gNB-CUs via the gNB-DU, F1/E1 impacts foreseen?

Solution 2: use different MRB Configuration

- the higher layer MRB configuration parameters only includes few parameters, even without inter gNB-CU coordination, different gNB-CUs may or may not provide different higher layer MRB configuration parameters.

- lower layer MRB configuration parameters are controlled by the gNB-DU, maybe the DU can choose to use different lower layer MRB configuration parameters in case the higher layer MRB configuration parameters are unchanged?

Proposal 5: Further discuss how to avoid/solve the identified issue in case of data path change, either try to keep the PDCP SN continuity, or make sure to use different MRB configurations.

## 2.3 other F1/E1 impacts

The following agreements were made last meeting:

CU-CP makes decision on whether to establish NG-U tunnel.

For MOCN scenario, only one set of shared F1-U tunnels is established and kept for MOCN scenario as long as there is one PLMN keeping the MBS service.

For multiple cell-ID broadcast scenario, the entity controlling logical DUs decides how many F1-U tunnels to be set up. The decision of CU-CP on establishment of NG-U tunnel takes the feedback of DU on establishment of a set of F1-U tunnels into account.

Adopt the option of Multiple F1AP contexts/messages, one per PLMN, for MOCN scenario.

Multiple E1AP context/message, one per PLMN, for MOCN scenario.

In order to support “not establish shared NG-U resources” and “not establish shared F1-U resources”, it is needed for the gNB-CU-CP to indicate such information of the gNB-CU-UP. Considering of the proposal 2 that the establishment of F1-U tunnel is determined by the gNB-DU, and the establishment of NG-U tunnel is determined by the gNB-CU, these indications should be included in a list of F1/E1 messages.

Proposal 6: Introduce a Shared F1-U not Established IE in

- F1AP: BROADCAST CONTEXT SETUP RESPONSE message

- F1AP: BROADCAST CONTEXT MODIFICATION RESPONSE message

- E1AP: BC BEARER CONTEXT MODIFICATION REQUEST message

Proposal 7: Introduce a Shared NG-U not Established IE in:

- E1AP: BC BEARER CONTEXT SETUP REQUEST message

- E1AP: BC BEARER CONTEXT MODIFICATION REQUEST message

2.4 correction to NGAP BL CR

In the latest version of NGAP BL CR, in the Broadcast Transport Response Transfer IE, the MBS Session TNL Information 5GC IE is optionally provided, but considering that this new Broadcast Session Transport procedure is only be used for unicast transport, there is no need to have this optional IE.

Proposal 8: In the NGAP BL CR, remove the MBS Session TNL Information 5GC IE in the Broadcast Transport Response Transfer IE.

# 3. Conclusion

Based on the discussion in this paper, we got the following observations and proposals:

Proposal 1: Define the Associated Session ID IE as a sequence type including two IP addresses, i.e. source address (IP unicast address) and destination address (IP multicast address).

Proposal 2: In case of RAN Sharing with multiple cell-ID broadcast, F1-U tunnel and NG-U tunnel mapping is left to gNB-DU and gNB-CU implementation, i.e. the establishment of F1-U tunnel is determined by the gNB-DU, and the establishment of NG-U tunnel is determined by the gNB-CU.

Proposal 3: In case of RAN sharing with multiple Cell ID broadcast, the gNB-DU chooses one of the PDCP configurations from different gNB-CUs, and transmit the DL data received from the related gNB-CU to the UEs.

Observation 1: Upon receiving the updated MRB configuration over MCCH, the UE will release and setup the MRB, and therefore it can receive the data using the new configuration correctly.

Observation 2: During the reception of a broadcast MRB, the PDCP Data PDU outside receiving window will be discarded by the UE.

Proposal 4:  Confirm the issue about in case same MRB Configurations is used before and after path change, the UE will not be able to “release and add” the broadcast MRB and initialize the receiving window, if the PDCP SN of the packets received via the new path are outside of the receiving window, these packets will be discarded by the UE.

Proposal 5: Further discuss how to avoid/solve the identified issue in case of data path change, either try to keep the PDCP SN continuity, or make sure to use different MRB configurations.

Proposal 6: Introduce a Shared F1-U not Established IE in

- F1AP: BROADCAST CONTEXT SETUP RESPONSE message

- F1AP: BROADCAST CONTEXT MODIFICATION RESPONSE message

- E1AP: BC BEARER CONTEXT MODIFICATION REQUEST message

Proposal 7: Introduce a Shared NG-U not Established IE in:

- E1AP: BC BEARER CONTEXT SETUP REQUEST message

- E1AP: BC BEARER CONTEXT MODIFICATION REQUEST message

Proposal 8: In the NGAP BL CR, remove the MBS Session TNL Information 5GC IE in the Broadcast Transport Response Transfer IE.

Based on these proposals and the previous agreements, the TPs to BL CRs are provided in the following sections.

-----------------End of the Change------------------

# 4. TP to TS 38.401 BL CR

-----------------Start of the Change-------------------

## 7.7	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

### 7.7.1	Support of dynamic PTP and PTM switching

For UEs in RRC_CONNECTED, NG-RAN supports dynamic switch between PTP and PTM for MBS as specified in TS 38.300 [2].

In case of split gNB architecture, for a MRB with common PDCP involving both PTP (RLC leg) and PTM (RLC leg), upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg) or both.

For UEs in RRC_INACTIVE only PTM is supported as specified in TS 38.300 [2].

### 7.7.x	Support of resource efficiency for RAN Sharing

#### 7.7.x.1	General

Editor’s Note:	TBD

#### 7.7.x.2	Support of resource efficiency for MOCN

Information enabling identifying broadcast MBS sessions for which RAN resources could be shared by the involved gNB-DUs and the gNB-CU-UP is provided by the gNB-CU-CP. Only one set of shared F1-U resources is established and kept as long as there is one PLMN keeping the Multicast service.

#### 7.7.x.3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content.

Applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	resolve different QoS requirements received from the participating 5GCs in an implementation specific way.

-	F1-U resources are established towards either all involved gNB-CUs or only some of them which is decided by the entity controlling the involved gNB-DUs sharing the same physical cell resources.

-	the gNB-CU-CP takes into account the decision F1-U resources have been established or not to decide whether to establish NG-U resources.

### 7.7.y	Support of Multicast reception for UEs in RRC_INACTIVE

F1AP supports enabling of multicast reception for UEs in RRC_INACTIVE.

The gNB-CU decides whether multicast reception in RRC_INACTIVE is applied.

Editor’s Note:	FFS whether additional assistance information is sent by gNB-DU to gNB-CU to help gNB-CU's decision making.

----------------- Start of the Change------------------

## 8.15	Overall procedures for NR MBS

### 8.15.1	General

The following clauses describe the overall procedures for NR MBS involving E1 and F1.

#### 8.15.1.1	Broadcast MBS Session Setup

Figure 8.15.1.1-1 illustrates an exemplified interaction of NGAP, E1AP, F1AP and RRC protocol functions at Broadcast MBS Session Setup.

Figure 8.15.1.1-1: Broadcast MBS Session Setup

1.	The 5GC starts the broadcast session by sending the NGAP Broadcast Session Setup Request message to the gNB containing the TMGI, S-NSSAI, 5G QoS Profile, area information and transport information (for NG-U multicast transport it provides the IP multicast address and the IP source specific multicast address).

2/3. The gNB-CU-CP sets up the broadcast bearer context, providing NG-U transport information from the 5GC to the gNB-CU-UP and receiving from the gNB-CU-UP the NG-U GTP DL TEID in case NG-U unicast transport was selected and an F1-U GTP UL TEID per MRB.

4.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

5/6.	The gNB-CU-CP establishes the Broadcast MBS Session Context at the gNB-DU, providing MRB configuration, other relevant session parameters and F1-U GTP UL TEID information, and receiving F1-U GTP DL TEID information.

7/8.	The gNB-CU-CP triggers BC Bearer Context Modification Request towards the gNB-CU-UP to provide the F1-U GTP DL TEID information.

9.	The gNB-DU configures broadcast resources and provides broadcast configuration information to the UEs by means of MCCH.

10.	The gNB-CU CP successfully terminates the NGAP broadcast Session Setup procedure. In case the gNB has chosen NG-U unicast transport, NG-U GTP DL TEID information is provided to the 5GC.

11.	The broadcast MBS media stream is provided to the UEs.

On NG-U, in case of location dependent broadcast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the BC bearer context in the gNB-CU-UP, the gNB-CU-CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.

#### 8.15.1.1a	Broadcast MBS Session Setup for resource efficiency for network sharing

This clause describes necessary additions as compared to the Broadcast MBS Session Setup as defined in clause 8.15.1.1, to support resource efficiency for MOCN or for RAN sharing with multiple cell-ID broadcast.

In addition to clause 8.15.1.1, the following enhancements applies:

- Step 1: the 5GC includes the associated session ID in the NGAP Broadcast Session Setup Request message.

- Step 2: if the gNB-CU-CP determines that there is already an existing broadcast MBS session delivering the same content based on the received Associated Session ID, the gNB-CU-CP may decide to not establish the shared NG-U resources and then send shared NG-U resources not to establish indication to the gNB-CU-UP.

- Step 3: in case shared NG-U resources not to establish indication is received in Step 2, the gNB-CU-UP shall, if supported, not allocate shared NG-U resources accordingly.

- Step 4: in case shared NG-U resources not to establish indication is received in Step 2, the step 4 is skipped.

- Step 5: the gNB-CU-CP includes the Associated Session ID in the Broadcast Context Setup Request to the gNB-DU.

- Step 6: if the gNB-DU determines that there is already an existing broadcast MBS session delivering the same content based on the received Associated Session ID, the gNB-DU may decide to not establish shared F1-U resources and provide F1-U resources not to establish indication to the gNB-CU-CP, but the F1-U GTP DL TEID information will not be provided.

- Step 7: if the gNB-CU-CP receives the shared F1-U resources not to establish indication in step 6, the gNB-CU-CP may decide to not establish the shared NG-U resources and send shared NG-U resources not to establish indication to the gNB-CU-UP. The gNB-CU-CP also forwards the shared F1-U resources not to establish indication to the gNB-CU-UP, if received in step 6.

- Step 8: upon receiving the shared NG-U resources not to establish indication in step 7, the gNB-CU-UP leaves the IGMP/MLD if jointed in step 4, and releases the shared NG-U resources allocated in step 3 if any. Upon receiving the shared F1-U resources not to establish indication, the gNB-CU-UP releases the shared F1-U resources allocated in step 3 if any.

- Step 9/10: in case of RAN Sharing with multiple cell-ID broadcast, for the broadcast sessions with the same associated session ID, the gNB-DU chooses one set of broadcast MRB configurations from a gNB-CU, provides the selected set of broadcast MRB configurations over the MCCH for these broadcast sessions, and transmits the received DL data from the same gNB-CU over the radio.

In case of shared NG-U termination,

-	the gNB-CU-CP may include the Associated Session ID and the MBS Service Area in the BC Bearer Context Setup Request in case of shared NG-U termination.

-----------------End of the Change------------------

# 5. TP to TS 38.413 BL CR

-----------------Start of the Change-------------------

#### 9.3.1.y	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |
| IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |

----------------- Start of the Change------------------

#### 9.3.5.y	Broadcast Transport Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  |

----------------- Start of the Change------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

// skip unchanged part

AssociatedQosFlowList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF AssociatedQosFlowItem

AssociatedQosFlowItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowMappingIndication		ENUMERATED {ul, dl, ...}							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssociatedQosFlowItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedQosFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex	PRESENCE optional	},

...

}

AssociatedSessionID ::= SEQUENCE {

iP-MulticastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

iE-Extensions				ProtocolExtensionContainer { {AssociatedSessionID-ExtIEs} } 	OPTIONAL,

...

}

AssociatedSessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AuthenticatedIndication ::= ENUMERATED {true, ...}

// skip unchanged part

BroadcastPLMNItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-NPN-Support					CRITICALITY reject	EXTENSION NPN-Support				PRESENCE optional}|

{ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList 	PRESENCE optional}|

{ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION TAINSAGSupportList		PRESENCE optional},

...

}

BroadcastTransportFailureTransfer ::= SEQUENCE {

mBS-SessionID 				MBS-SessionID,

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics															OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastTransportFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

BroadcastTransportFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastTransportRequestTransfer ::= SEQUENCE {

mBS-SessionID 				MBS-SessionID,

mBS-SessionTNLInfoNGRAN 	MBS-SessionTNLInfoNGRAN,

iE-Extensions				ProtocolExtensionContainer { {BroadcastTransportRequestTransfer-ExtIEs} }		OPTIONAL,

...

}

BroadcastTransportRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastTransportResponseTransfer ::= SEQUENCE {

mBS-SessionID 			MBS-SessionID,

iE-Extensions			ProtocolExtensionContainer { {BroadcastTransportResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

BroadcastTransportResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig				BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList											OPTIONAL,

bt-rssi							ENUMERATED {true, ...}												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } 	OPTIONAL,

...

}

BluetoothMeasurementConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothMeasConfigNameItem

BluetoothMeasConfigNameItem ::= SEQUENCE {

bluetoothName		BluetoothName,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasConfigNameItem-ExtIEs } } 	OPTIONAL,

...

}

BluetoothMeasConfigNameItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-----------------End of the Change------------------

# 6. TP to TS 38.473 BL CR

-----------------Start of the Change-------------------

## 8.14	NR MBS Procedures

### 8.14.1	Broadcast Context Setup

#### 8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

If

-	either the MBS Service Area IE was included in the BROADCAST CONTEXT SETUP REQUEST message,

-	or the the MBS Service Area IE was not included in the BROADCAST CONTEXT SETUP REQUEST message and the gNB-DU was not able to establish MBS Session Resources in all cells served by the gNB-DU,

the Broadcast Area Scope IE shall be included in the BROADCAST CONTEXT SETUP RESPONSE message to indicate the cells where MBS Session resources have been successfully established in the gNB-DU.

If the Associated Session ID IE is contained in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.401 [4].

If the Shared F1-U not established IE is provided in the Broadcast MRB Setup Item IE in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the shared F1-U resources is not established for the MRB, ignore the BC Bearer Context F1-U TNL Info at DU IE provided for that MRB, and release the F1-U resources corresponding to that MRB allocated in the BROADCAST CONTEXT SETUP REQUEST message.

For a MBS area session in case of location dependent scenario, if the Shared F1-U not Established IE is provided in the BC Bearer Context F1-U TNL Info at DU IE in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the shared F1-U resources is not established for the MRB for the MBS area session, ignore the MBS F1-U Information IE in the BC Bearer Context F1-U TNL Info at DU IE provided for that MRB for the MBS area session, and release the F1-U resources corresponding to that MRB for the MRB area session allocated in the BROADCAST CONTEXT SETUP REQUEST message.

----------------- Start of the Change------------------

### 8.14.4	Broadcast Context Modification

#### 8.14.4.1	General

The purpose of the Broadcast Context Modification procedure is to modify an established MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.4.2	Successful Operation

Figure 8.14.4.2-1: Broadcast Context Modification procedure. Successful operation

The BROADCAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the BROADCAST CONTEXT MODIFICATION RESPONSE message.

If the Broadcast MRB To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicate the reason for the failure with an appropriate cause value for each MRB failed to be setup.

If the Broadcast MRB To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-DU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Broadcast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Broadcast MRB Failed To Be Modified List IE;

If the Broadcast MRB Failed To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be modified with an appropriate cause value for each Broadcast MRB failed to modify.

If the MBS Service Area IE is included in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall

-	release MBS Session Resources within cells not contained in the MBS Service Area IE, if any;

-	establish MBS Session Resources within cells which have not been contained in MBS Service Area information previously received;

-	replace MBS Service Area information previously received with information received in the MBS Service Area IE included in the BROADCAST CONTEXT MODIFICATION REQUEST message;

-	include the Broadcast Area Scope IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message to indicate the cells where MBS Session resources are currently established in the gNB-DU.

If the MBS Service Area IE was not included in the BROADCAST CONTEXT MODIFICATION REQUEST message and the gNB-DU has released MBS Session Resources within at least one cell or has established MBS Session Resources within at least one cell the gNB-DU shall include the Broadcast Area Scope IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message to indicate the cells where MBS Session resources are currently established in the gNB-DU.

If the Shared F1-U not established IE is provided in the Broadcast MRB Setup Item IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the shared F1-U resources is not established for the MRB, ignore the BC Bearer Context F1-U TNL Info at DU IE provided for that MRB, and release the F1-U resources corresponding to that MRB allocated in the BROADCAST CONTEXT MODIFICATION REQUEST message.

For a MBS area session in case of location dependent scenario, if the Shared F1-U not Established IE is provided in the BC Bearer Context F1-U TNL Info at DU IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the shared F1-U resources is not established for the MRB for the MBS area session, ignore the MBS F1-U Information IE in the BC Bearer Context F1-U TNL Info at DU IE provided for that MRB for the MBS area session, and release the F1-U resources corresponding to that MRB for the MRB area session allocated in the BROADCAST CONTEXT MODIFICATION REQUEST message.

----------------- Start of the Change------------------

#### 9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. This IE is ignored if the Shared F1-U not Established IE is present for this MRB. | - |  |
| >>Shared F1-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish shared F1-U resources for this MRB. | YES | ignore |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

----------------- Start of the Change------------------

#### 9.2.13.7	BROADCAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. This IE is ignored if the Shared F1-U not Established IE is present for this MRB. | - |  |
| >>Shared F1-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish shared F1-U resources for this MRB. | YES | ignore |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | Updated gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

----------------- Start of the Change------------------

#### 9.3.2.7	BC Bearer Context F1-U TNL Info

This IE contains F1-U TNL information for an MBS Session. In case of location dependent MBS sessions, it also contains per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS F1-U Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS F1-U Information |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.221 |  |
| >>>MBS F1-U Information | M |  | UP Transport Layer Information 9.3.2.1 | This IE is ignored if the Shared F1-U not Established IE is included for this MRB for this MBS Area Session. |
| >>Shared F1-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish the shared F1-U resources. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

----------------- Start of the Change------------------

#### 9.3.1.x 	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Multicast Address | M |  | Transport Layer Address 9.3.2.3 |  |
| IP Source Address | M |  | Transport Layer Address 9.3.2.3 |  |

----------------- Start of the Change------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

// skip unchanged part

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-SharedF1UnotEstablished,

maxNRARFCN,

maxnoofErrors,

// skip unchanged part

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoofSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (1..3)

Associated-SCell-Item ::= SEQUENCE {

sCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Associated-SCell-ItemExtIEs } }	OPTIONAL

}

Associated-SCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= SEQUENCE {

iP-MulticastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

iE-Extensions				ProtocolExtensionContainer { {AssociatedSessionID-ExtIEs} } 	OPTIONAL,

...

}

AssociatedSessionID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailablePLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF AvailablePLMNList-Item

AvailablePLMNList-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { AvailablePLMNList-Item-ExtIEs} } OPTIONAL

}

AvailablePLMNList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

// skip unchanged part

BroadcastMRBs-Setup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-Setup-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedF1UNotEstablished				CRITICALITY ignore	EXTENSION SharedF1UNotEstablished			PRESENCE optional},

...

}

BroadcastMRBs-SetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-SetupMod-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-SetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedF1UNotEstablished				CRITICALITY ignore	EXTENSION SharedF1UNotEstablished			PRESENCE optional},

...

}

// skip unchanged part

LocationDependentMBSF1UInformation ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformation-Item

LocationDependentMBSF1UInformation-Item ::= SEQUENCE {

mbsAreaSession-ID				MBS-Area-Session-ID,

mbs-f1u-info-at-CU				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformation-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformation-Item-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedF1UNotEstablished				CRITICALITY ignore	EXTENSION SharedF1UNotEstablished			PRESENCE optional},

...

}

LocationMeasurementInformation ::= OCTET STRING

LocationUncertainty ::= SEQUENCE {

horizontalUncertainty		INTEGER (0..255),

horizontalConfidence		INTEGER (0..100),

verticalUncertainty			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { LocationUncertainty-ExtIEs} } OPTIONAL

}

LocationUncertainty-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

// skip unchanged part

MBS-Flows-Mapped-To-MRB-Item 	::= SEQUENCE {

mBS-QoSFlowIdentifier						QoSFlowIdentifier,

mbs-QoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions								ProtocolExtensionContainer { { MBS-Flows-Mapped-To-MRB-Item-ExtIEs} } OPTIONAL

}

MBS-Flows-Mapped-To-MRB-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformation ::= SEQUENCE {

mbs-f1u-info				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformation-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformation-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

// skip unchanged part

Service-Status ::= SEQUENCE {

service-state				Service-State,

switchingOffOngoing			ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Service-Status-ExtIEs } }	OPTIONAL,

...

}

Service-Status-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativeTime1900 ::= 	BIT STRING (SIZE (64))

SharedF1UNotEstablished ::= ENUMERATED {true, ...}

ShortDRXCycleLength ::=  ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ...}

ShortDRXCycleTimer ::= INTEGER (1..16)

SIB1-message ::= OCTET STRING

SIB10-message ::= OCTET STRING

SIB12-message ::= OCTET STRING

SIB13-message ::= OCTET STRING

SIB14-message ::= OCTET STRING

// skip unchanged part

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

// skip unchanged part

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-AssociatedSessionID									ProtocolIE-ID ::= 998 -- to be allocated

id-SharedNGUNotEstablished								ProtocolIE-ID ::= 999 -- to be allocated

id-MBS-AssistanceInformation							ProtocolIE-ID ::= 1000 -- to be allocated

id-BroadcastTransportFailureTransfer					ProtocolIE-ID ::= FFS

id-BroadcastTransportRequestTransfer					ProtocolIE-ID ::= FFS

id-BroadcastTransportResponseTransfer					ProtocolIE-ID ::= FFS

id-SharedF1UNotEstablished								ProtocolIE-ID ::= FFS

END

-- ASN1STOP

-----------------End of the Change------------------

# 7. TP to TS 37.483 BL CR

-----------------Start of the Change-------------------

#### 9.2.5.1.1	BC BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup MBS session resources for a broadcast MBS session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.106 |  | YES | reject |
| Global MBS Session ID | M |  | 9.3.1.108 |  | YES | reject |
| BC Bearer Context To Setup | M |  | 9.3.3.26 |  | YES | reject |
| Associated Session ID | O |  | 9.3.3.x |  | YES | ignore |
| MBS Service Area | O |  | 9.3.3.y |  | YES | ignore |

----------------- Start of the Change------------------

#### 9.3.1.112	BC Bearer Context NG-U TNL Info at 5GC

This IE contains TNL information for an MBS Session as provided by the 5GC for both, shared NG-U multicast and unicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type |  |  |  |  |
| >location independent |  |  |  |  |
| >>MBS NG-U Information at 5GC | M |  | 9.3.1.113 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS NG-U Information at 5GC |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  |
| >>MBS NG-U Information at 5GC | M |  | 9.3.1.113 | This IE is ignored if the Shared NG-U not Established IE is included for this MBS area session. |
| >>Shared NG-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish shared NG-U resources for this MBS area session. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

----------------- Start of the Change------------------

#### 9.3.1.119	BC Bearer Context F1-U TNL Info at DU

This IE contains CU F1-U TNL information for an MBS Session. It may also contain per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS F1-U Information at DU |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 | This IE is ignored if the Shared F1-U not Established IE is included for this MBS Area Session. |
| >>Shared F1-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish shared F1-U resources for this MBS area session. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

----------------- Start of the Change------------------

#### 9.3.3.26	BC Bearer Context To Setup

This IE contains MBS session resource related information used to request BC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  |
| BC Bearer Context NG-U TNL Info at 5GC | O |  | 9.3.1.112 | This IE is ignored If the Shared NG-U not Established IE is included for this MBS Session. |
| BC MRB To Setup List | M |  | BC MRB Setup Configuration 9.3.1.114 |  |
| Requested Action for Available Shared NG-U Termination | O |  | 9.3.1.115 |  |
| Shared NG-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish shared NG-U resources for this MBS session. |

----------------- Start of the Change------------------

#### 9.3.3.28	BC Bearer Context To Modify

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify | O |  | BC Bearer Context NG-U TNL Info at 5GC 9.3.1.112 | This IE is ignored If the Shared NG-U not Established IE is included for this MBS Session. |
| BC MRB To Setup List | O |  | BC MRB Setup Configuration 9.3.1.114 |  |
| BC MRB To Modify List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >BC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.119 | This IE is ignored If the Shared F1-U not Established IE is included for this MRB. |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 |  |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |
| >>Shared F1-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish shared F1-U resources for this MRB. |
| BC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| Shared NG-U not Established | O |  | ENUMERATED (true, ...) | Indicates to not establish shared NG-U resources for this MBS session. |

----------------- Start of the Change------------------

#### 9.3.3.x Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |
| IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y	MBS Service Area

This IE contains the MBS service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS Service Area Information | M |  | 9.3.3.z |  |
| >location dependent |  |  |  |  |
| >>MBS Service Area Information Location Dependent List |  | 1..maxnoofMBSServiceArea Information |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  |
| >>>MBS Service Area Information | M |  | 9.3.3.z |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSServiceAreaInformation | Maximum no. of MBS Service Area Information elements in the MBS Service Area Information Location Dependent List IE. Value is 256 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.z	MBS Service Area information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.14 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >PLMN-Identity | M |  | 9.3.1.7 |  |
| >5GS TAC | M |  | 9.3.3.w |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 512. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 512. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.w	5GS TAC

This information element is used to identify Tracking Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5GS TAC | M |  | OCTET STRING (SIZE (3)) |  |

----------------- Start of the Change------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

// skip unchanged part

id-MCForwardingResourceReleaseIndication,

id-PDCP-COUNT-Reset,

id-MBSSessionAssociatedInfoNonSupportToSupport,

id-VersionID,

id-MBSAreaSessionID,

id-Secondary-PDU-Session-Data-Forwarding-Information,

id-SharedF1UNotEstablished,

id-SharedNGUNotEstablished,

maxnoofMBSAreaSessionIDs,

// skip unchanged part

AlternativeQoSParaSetItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= SEQUENCE {

iP-MulticastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

iE-Extensions				ProtocolExtensionContainer { {AssociatedSessionID-ExtIEs} } 	OPTIONAL,

...

}

AssociatedSessionID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

// skip unchanged part

##### -- BCBearerContextToSetup

BCBearerContextToSetup ::= SEQUENCE {

snssai									SNSSAI,

bcBearerContextNGU-TNLInfoat5GC			BCBearerContextNGU-TNLInfoat5GC				OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration,

requestedAction							RequestedAction4AvailNGUTermination			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedNGUNotEstablished			CRITICALITY ignore	EXTENSION SharedNGUNotEstablished			PRESENCE optional},

...

}

// skip unchanged part

##### -- BCBearerContextToModify

BCBearerContextToModify ::= SEQUENCE {

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC	OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration			OPTIONAL,

bcMRBToModifyList					BCMRBModifyConfiguration			OPTIONAL,

bcMRBToRemoveList					BCMRBRemoveConfiguration			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedNGUNotEstablished			CRITICALITY ignore	EXTENSION SharedNGUNotEstablished			PRESENCE optional},

...

}

BCBearerContextNGU-TNLInfoat5GC::= CHOICE {

locationindependent				MBSNGUInformationAt5GC,

locationdependent				LocationDependentMBSNGUInformationAt5GC,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextNGU-TNLInfoat5GC-ExtIEs}}

}

BCBearerContextNGU-TNLInfoat5GC-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBSetupConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupConfiguration-Item

BCMRBSetupConfiguration-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedF1UNotEstablished			CRITICALITY ignore	EXTENSION SharedF1UNotEstablished			PRESENCE optional},

...

}

// skip unchanged part

LocationDependentMBSNGUInformationAt5GC ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSNGUInformationAt5GC-Item

LocationDependentMBSNGUInformationAt5GC-Item ::= SEQUENCE {

mbsAreaSession-ID							MBSAreaSessionID,

mbsNGUInformationAt5GC						MBSNGUInformationAt5GC,

iE-Extensions							ProtocolExtensionContainer	{ { LocationDependentMBSNGUInformationAt5GC-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSNGUInformationAt5GC-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedNGUNotEstablished			CRITICALITY ignore	EXTENSION SharedNGUNotEstablished			PRESENCE optional},

...

}

LocationDependentMBSF1UInformationAtCU ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformationAtCU-Item

LocationDependentMBSF1UInformationAtCU-Item ::= SEQUENCE {

mbsAreaSession-ID				MBSAreaSessionID,

mbs-f1u-info-at-CU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformationAtCU-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformationAtCU-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationDependentMBSF1UInformationAtDU ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformationAtDU-Item

LocationDependentMBSF1UInformationAtDU-Item ::= SEQUENCE {

mbsAreaSession-ID				MBSAreaSessionID,

mbs-f1u-info-at-DU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformationAtDU-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformationAtDU-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedF1UNotEstablished			CRITICALITY ignore	EXTENSION SharedF1UNotEstablished			PRESENCE optional},

...

}

LocationDependentMBSNGUInformationAtNGRAN ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSNGUInformationAtNGRAN-Item

LocationDependentMBSNGUInformationAtNGRAN-Item ::= SEQUENCE {

mbsAreaSession-ID							MBSAreaSessionID,

mbsNGUInformationAtNGRAN						MBSNGUInformationAtNGRAN,

iE-Extensions							ProtocolExtensionContainer	{ { LocationDependentMBSNGUInformationAtNGRAN-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSNGUInformationAtNGRAN-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

// skip unchanged part

SecurityResult ::= SEQUENCE {

integrityProtectionResult			IntegrityProtectionResult,

confidentialityProtectionResult		ConfidentialityProtectionResult,

iE-Extensions						ProtocolExtensionContainer { {SecurityResult-ExtIEs} }	OPTIONAL,

...

}

SecurityResult-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

SharedF1UNotEstablished ::= ENUMERATED {

true,

...

}

SharedNGUNotEstablished ::= ENUMERATED {

true,

...

}

// skip unchanged part

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

// skip unchanged part

id-PDCP-COUNT-Reset												ProtocolIE-ID ::= 184

id-MBSSessionAssociatedInfoNonSupportToSupport					ProtocolIE-ID ::= 185

id-VersionID													ProtocolIE-ID ::= 186

id-InactivityInformationRequest									ProtocolIE-ID ::= 187

id-UEInactivityInformation										ProtocolIE-ID ::= 188

id-MBSAreaSessionID												ProtocolIE-ID ::= 189

id-Secondary-PDU-Session-Data-Forwarding-Information			ProtocolIE-ID ::= 190

id-AssociatedSessionID											ProtocolIE-ID ::= xxx

id-MBS-ServiceArea												ProtocolIE-ID ::= xxx

id-SharedF1UNotEstablished 										ProtocolIE-ID ::= FFS

id-SharedNGUNotEstablished 										ProtocolIE-ID ::= FFS

END

-- ASN1STOP

-----------------End of the Change------------------