3GPP TSG-RAN WG3 Meeting #121bis	R3-235191

Xiamen, China, 09-13 Oct, 2023

Title: 	(TP to R3-234736 to TS 38.300) Correction on large size UL SDT data in case of without anchor relocation

Source: 	Huawei, China Telecom, Ericsson, Qualcomm Incorporated, China Unicom

Agenda item:	31.2

Document Type:	other

# 1	Introduction

As discussed in [1], in case of SDT without UE context relocation, the Last serving gNB is not able to receive the BSR from UE, therefore, it cannot determine large size UL SDT Data Arrival. But in the technically endorsed stage2 draftCR [2], the following Note was added:

## 18.3 SDT without UE context relocation … NOTE x: The last serving gNB may terminate the SDT procedure and direct the UE to continue in RRC_INACTIVE state by sending the RRCRelease message based on the BSR received from the UE in case of uplink SDT data exceeding the data size threshold.

This paper provides the TP to fix this issue.

# 2	Reference

[1] R3-235189 Consideration on large size UL SDT data in case of without anchor relocation, Huawei, China Telecom, Ericsson

[2] R3-234736 Support of oversize UL SDT Data Arrival [Large SDT Uplink Data], ZTE, Nokia, Nokia Shanghai Bell, Huawei, Qualcomm Incorporated, CATT, LG Electronics, China Telecom, Samsung, Ericsson

# 3	TP to technically endorsed TEI18 draft CR R3-234736 to TS38.300

------------------Start of the Change-----------------

## 18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2. The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides not to relocate the full UE context for SDT.

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context, and may also indicate an SDT volume threshold.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address. The UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB. Then UL/DL GTP-U tunnels are established for DRBs configured for SDT, if any, and the UL SDT data and/or signalling, if any, are forwarded to the last serving gNB, and then delivered to the core network.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers it to the UE.

6.	The receiving gNB detects the end of SDT session and sends the RETRIEVE UE CONTEXT CONFIRM message including whether this is a "normal" end of SDT transaction or a radio link problem. Or if the UL SDT data size in the BSR received from the UE is larger than the SDT volume threshold indicated in Step 4, the receiving gNB sends the RETRIEVE UE CONTEXT CONFIRM message including SDT volume threshold crossed indication.

7.	Upon receiving the RETRIEVE UE CONTEXT CONFIRM message and deciding to terminate the SDT, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The receiving gNB shall release the established partial UE context.

NOTE 2:	Void..

NOTE 3:	Void..

8.	The receiving gNB sends the RRCRelease message to the UE.

NOTE 4:	In case DL non-SDT data or DL non-SDT signalling arrives, or receives UE assistance information (i.e. UL non-SDT data arrival indication) from the UE, the last serving gNB completes the SDT procedure and directs the UE to continue in RRC_INACTIVE state by sending the RRCRelease message.

NOTE x:	The last serving gNB may terminate the SDT procedure and direct the UE to continue in RRC_INACTIVE state by sending the RRCRelease message upon receiving the indication about uplink SDT data exceeding the data size threshold from the receiving gNB in step 6.

9.	The UE moves to RRC_INACTIVE state if the suspend indication is included in the RRCRelease message. Or else, the UE moves to RRC_IDLE state.

------------------End of the Change------------------