3GPP TSG-RAN WG3 Meeting #121bis	R3-235211

Xiamen, China, 9 – 13 October 2023

Agenda item:	 10.2.2

Source:		Samsung

Title:	 (TP for SON BLCR for 38.423 and 37.340) SON enhancements for CPAC

Document for:		Discussion and Decision

1	Introduction

On MR-DC for CPAC, RAN3 has achieved some agreements at previous meetings. This contribution continued the discussion on the specification impacts to support this feature. The discussion mainly focused on NR-NR DC. Inter-RAT CPAC can be discussed after the NR-NR DC solution is clear.

2	Discussion

The agreements achieved at RAN3#117-e and RAN3#117bis-e meeting:

MRO for CPC and CPA:

MRO for CPC and CPA based on the R17 NR-DC MRO solution

Not consider too late CPA.

CPA Execution to wrong PSCell will be considered, e.g. UE receives CPA configuration and CPA execution condition is satisfied, CPA execution fails or an SCG failure occurs shortly after a successful CPA execution; a suitable PSCell different with target PSCell is found based on the measurements reported from the UE.

Too Late CPC Execution, Too Early CPC Execution and CPC Execution to wrong PSCell will be considered:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different with source PSCell is found based on the measurements reported for the UE.

-	Too Early CPC Execution: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	CPC Execution to wrong PSCell: UE receives CPC configuration and CPC execution condition is satisfied, CPC execution fails or an SCG failure occurs shortly after a successful CPC execution; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

For MRO for CPAC, deprioritize Case i/ii/iii/iv:

-	Case i: mixed scenarios of legacy PA and CPA, i.e. UE receives CPA configuration, a legacy PSCell addition is performed but fails, or a legacy PSCell addition is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell addition.

-	Case ii: mixed scenarios of legacy PC and CPC, i.e. UE receives CPC configuration, a legacy PSCell change is performed but fails, or a legacy PSCell change is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell change.

-	Case iii: MCG RLF or handover failure or CHO execution failure before CPA/CPC execution.

-	Case iv: CHO-CPC coexistence scenarios with low priority.

The agreements achieved at RAN3#118 meeting:

MRO for CPC and CPA:

Too Early CPA Execution will be considered. FFS on the naming

The agreements achieved at RAN3#119 meeting:

Naming for too early CPA execution? No need to rename.

Update the definition to wrong PSCell change/addition should be splitting to sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the target node.

Information available in the network nodes should not be included in the SCGFailureInformation.

Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN.

For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events.

The agreements achieved at RAN3#119bis-e meeting:

Definitions of MRO events for CPAC will be introduced in TS 37.340 in a new chapter (based on [1226], CPA needs to be added).

During CPAC configuration, the value of the Time Stay IE for the source PSCell UHI, sent in S-NODE ADDITION REQUEST message, does not reflect the exact time the UE stayed in the source PSCell.

Support CPAC UHI?

The agreements achieved at RAN3#120 meeting:

Include CPC candidate cell list and CPC execution condition(s) to the message from the MN to the source SN.

The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

The agreements achieved at RAN3#121 meeting:

Include CPC candidate cell list and CPC execution condition(s) to the message from the MN to the source SN.

The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

RAN2#123

Agreements:

3   For CPAC MRO, UE logs the below information in SCGFailureInformation:

the type of the first triggered CPAC event if multiple events are configured

the time duration between the two triggered CPAC events if multiple events are configured

4   For CPAC MRO, RAN2 discuss which of below measurement information is included in SCGFailureInformation (should further check whether something is already existed):

Latest radio measurements of neighbour cell(s) if available, reusing existing fields.

Source PSCell info (cell ID, measurement result) if available, reusing existing fields.

Target PScell info (cell ID, measurement result) if available, reusing existing fields.

The overall procedures on MRO for CPAC were offline discussed at last RAN3#121 meeting. It was agreed that “The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.” For the subsequent signalling flow, there are different options.

For CPA or MN initiated CPC, there are two options:

Option 1:

  - if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - FFS: Indication whether a candidate PSCell is accepted by the (candidate) target (The candidate PSCell list recommended by the initiating node is already agreed).

  - Otherwise, the MN makes the optimization

Option 2:

  - MN always send the SCG Failure Information Report message to the last serving SN.

  - The last serving SN feedback if the problem is not brought by itself.

  - the MN makes the optimization

The reason of Option 2 is that the MN doesn’t know whether there is intra-SN PSCell change trigger by the last serving SN. So the MN needs to firstly contact with the last serving SN. The story has been changed comparing with Rel-17. At the end of Rel-17, the IE “SCG Reconfiguration Notification” has been added to S-NODE MODIFICATION REQUIRED message in order to coordinate MN triggered CPC procedure and SN triggered CPC procedure. The IE can indicate to the MN there is intra-SN PSCell change.

In case there is intra-SN PSCell, the MN will know it is the SN triggered CPC, not MN triggered. For the SN triggered CPC, it will be discussed later.

For the MN triggered CPC, Option 1 is enough.

Proposal 1: For CPA or MN initiated CPC, agree Option 1 as way forward.

Option 1:

  - if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - FFS: Indication whether a candidate PSCell is accepted by the (candidate) target (The candidate PSCell list recommended by the initiating node is already agreed).

  - Otherwise, the MN makes the optimization

Indication whether a candidate PSCell is accepted by the (candidate) target is needed. With this IE, the (candidate) target can know whether a candidate PSCell recommended by the source is accepted by the target. So the target can make opitmsiation on the candidate cell list.

Proposal 2: Include 	“Suitable PSCell ID” and “Indication whether a candidate PSCell is accepted by the (candidate) target” in the SCG Failure Information Report procedure from the MN to the (candidate) target SN:

The TP for TS38.423 is provided in Annex 1 to capture the proposal 2.

Proposal 3: Agree the TP for TS38.423 in Annex 1.

For SN initiated CPC, there are two options:

Option 1:

  - The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

  - if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

  - Otherwise, the source SN makes the optimization

    - SCG FAILURE TRANSFER is used from the source SN to the MN

    - SCG Failure Information Report message is used from the MN to the (candidate) target SN

Option 2: the same as the option 2 for MN initiated CPC.

Similar as MN initiated CPC, the reason of Option 2 is that the MN doesn’t know whether there is intra-SN PSCell change trigger by the last serving SN. Actually, the MN can know whether there is intra-SN CPC based on “SCG Reconfiguration Notification” IE.

In case there is intra-SN PSCell, the MN will know the last serving SN is the source SN. With option 1 “The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis”, the message is actually sent to the last serving SN. So intra-SN PSCell change/CPC has been considered in option 1.

Proposal 4: For SN initiated CPC, agree Option 1 as way forward.

Option 1:

  - The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

  - if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

  - Otherwise, the source SN makes the optimization

The TP with failure definitions on MRO for CPAC has been agreed at RAN3#120 meeting.

Currently in stage 2, there is description on the signalling from MN to the source SN or the last serving SN. The overall procedure should be captured in stage 2. A TP is included in Annex 2.

Proposal 5: Agree the TP for TS37.340 in Annex 2.

3	Conclusion

This contribution discussed SON for CPAC. We have the following observations and proposals. The TPs for TS37.340 and TS38.423 are provided in the Annexs.

Proposal 1: For CPA or MN initiated CPC, agree Option 1 as way forward.

Option 1:

  - if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - FFS: Indication whether a candidate PSCell is accepted by the (candidate) target (The candidate PSCell list recommended by the initiating node is already agreed).

  - Otherwise, the MN makes the optimization

Proposal 2: Include 	“Suitable PSCell ID” and “Indication whether a candidate PSCell is accepted by the (candidate) target” in the SCG Failure Information Report procedure from the MN to the (candidate) target SN:

Proposal 3: Agree the TP for TS38.423 in Annex 1.

Proposal 4: For SN initiated CPC, agree Option 1 as way forward.

Option 1:

  - The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

  - if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

  - Otherwise, the source SN makes the optimization

Proposal 5: Agree the TP for TS37.340 in Annex 2.

Reference

[1] TS37.340

[2] TS38.423

[3] R3-226587, Discussion on MR-DC SCG failure and inter-RAT CPAC, Samsung

Annex 1: TP for TS38.423

#### 9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node   S-NG-RAN node .

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | The SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.xy |  | YES | ignore |
| Suitable PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | CGI of suitable PSCell to be candidate PSCell for CPAC procedure | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.xy	CPAC Configuration

This IE contains the CPC or CPA configuration information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CPAC Candidate Cell List |  | 1 |  |  |
| >CPAC Candidate Cell Item |  | 1 .. <maxnoofPSCellsinCPAC> |  |  |
| >>CPAC Candidate Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  |
| >> Accepted by the target | O | ENUMERATED (True, …) |  |  |
| >>CPAC Execution Condition List |  | 1 |  |  |
| >>>CPAC Execution Condition Item |  | 1 .. <maxnoofCPACexecutioncond> |  |  |
| >>>>MeasObject Container | M |  | OCTET STRING | Includes the MeasObjectToAddMod IE contained in the RRCReconfiguration message as specified in TS 38.331 [10], which is configured for the CPAC candidate cell |
| >>>>ReportConfig Container | M |  | OCTET STRING | Includes the ReportConfigToAddMod IE contained in the RRCReconfiguration message as specified in TS 38.331 [10], which is configured for the CPAC candidate cell |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsinCPAC | Maximum no. cells that can be prepared for a CPAC handover. Value is 8. |
| maxnoofCPACexecutioncond | Maximum no. execution conditions for a conditional handover. Value is 2. |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

Annex 2: TP for TS37.340

### 10.18.1	General

For analysis of PSCell change failure, the UE makes the SCG Failure Information available to the MN.

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

### 10.18.2	PSCell change failure

One of the functions of self-optimization for PSCell change is to detect PSCell change failures that occur due to Too late PSCell change or Too early PSCell change, or Triggering PSCell change to wrong PSCell. These problems are defined as follows:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

In the definition above, the "successful PSCell change" refers to the UE state, namely the successful completion of the RA procedure.

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

### 10.18.A	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN, it is wrong target PSCell selection at the (candidate) target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

For CPA or MN initiated CPC, if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell is selected by the (candidate) target SN via SCG Failure Information Report message. Otherwise, the MN makes the optimization.

For SN initiated CPC, the MN sends SCG failure information report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends SCG FAILURE TRANSFER message to the MN, then the MN sends SCG Failure Information Report message to the (candidate) target SN. Otherwise, the source SN makes the optimization.