3GPP TSG-RAN WG3 Meeting #121bis	R3-235212

Xiamen, China, 9 – 13 October 2023

Agenda item:	 10.2.2

Source:		Samsung

Title:		(TP for SON BLCR for 38.413) MRO for the fast MCG recovery and inter-system handover for voice fallback

Document for:		Discussion and Decision

1	Introduction

On MRO for the fast MCG recovery and inter-system HO for voice fallback, RAN3 has achieved some agreements at previous meetings. This contribution discussed the leftover open issues on the two features.

2	Discussion

2.3	 MRO for the fast MCG recovery

Two scenarios for fast MCG failure recovery has been agreed at RAN3#117-e. Other problems are not precluded if legacy MRO mechanism cannot cope with it.

MRO for the fast MCG recovery:

SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure)

the signalling delay is longer than the time the UE waits for the response (T316 expired);

other problems are not precluded if legacy MRO mechanism cannot cope with it.

No agreement can be achieved at RAN3#117bis-e meeting.

A minimum set of parameters reported from the UE has been agreed at last RAN3#118 meeting:

It is beneficial for the UE to report at least the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure) and also, if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

Agreement achieved at RAN3#119 meeting:

Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure.

It is beneficial for the UE to report at least PSCell where SCG failure happened, the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure, SCG was deactivated or other cases that SCG is not available), and also if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

Agreement achieved at RAN3#119bis-e meeting:

Case f1, where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed.

Scenario ‘a’ is redefined: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running).

Agreement achieved at RAN3#120 meeting:

Agree to define case c in R18. Solution can be further discussed.

Agreement achieved at RAN2#122 meeting:

Agreements:

1	RAN2 confirms scenario of near failure fast MCG recovery.

2	RAN2 confirms scenario f1, i.e., SCG fails or is deactivated before the UE sends the MCGFailureInformation. FFS RAN2 impact.

Agreement achieved at RAN2#123 meeting:

Agreements:

1	UE reports the elapsed T316 between the transmission of MCGFailureInformation and receiving RRC reconfiguration or RRC release message.

2	No T316 related triggering threshold is introduced.

3	Reuse existing RLF report to capture fast MCG recovery related information.

Agreements:

1	RAN2 confirms the “SCG deactivation during fast MCG recovery” is not a valid scenario, therefore would not be considered in fast MCG MRO.

2   UE logs the new information for fast MCG link recovery optimziation, only when AS security has been activated.

Regard to fast MCG link recovery optimization, it is beneficial for network to understand the situation of SCG and do further optimization of fast MCG recovery based on the time between MCG failure and SCG failure.

The time can be used to indicate whether the coverage problem of the MN and SN, then some optimization can be performed. Therefore, for case a and case f1, UE reports the time between MCG failure and SCG failure.

Proposal 1: It is beneficial for the UE to report the time between MCG failure and SCG failure for fast MCG failure recovery.

2.3	 MRO for inter-system handover for voice fallback

The following scenarios were agreed to be considered on MRO enhancement for inter-system handover for voice fallback at RAN3#117 meeting:

Consider Case 1-2 for MRO enhancements for inter-system inter-RAT handover for voice fallback:

-	Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell.

-	Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

At RAN3#117bis-e meeting, it was agreed to introduce stage 2 description of failure type definition.

Introduce stage 2 descriptions of failure type definition for inter-system inter-RAT HO from NR to E-UTRA for voice fallback. The detailed descriptions are FFS.

At RAN3#119 meeting, the stage 2 TP for capturing the definition was agreed in R3-230993.

At RAN3#119bis-e meeting, the detection mechanism was agreed to stage 2:

Stage-2 description of the detection mechanism will be introduced in TS 38.300 (based on R3-231585).

At RAN3#120 meeting, the following working assumption was agreed:

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

RAN2#122 agreements:

Agreements:

1	Introduce a new indication in the LTE RLF report for the case an RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback.

2	UE to log the time until reconnection during RRC connection establishment to the acceptable cell and reconnection cell ID in is absent, which will reuse the legacy field.

At RAN3#121 meeting, the following was agreed:

Convert the WA into agreement “Define a new handover report type in the Inter-system HO Report over S1 and NG”.

The corresponding TP for TS38.413 was agreed. Some FFSs are left in the BLCR.

| >Inter-system Mobility Failure for Voice Fallback |  |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >>Source Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for HO |  |  |
| >>Failure Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the failure cell for HO |  |  |
| >>Re-connect Cell ID (FFS on the name) | O |  | FFS | FFS |  |  |
| >>UE RLF Report Container | O (FFS on the presence) |  | 9.3.3.41 |  |  |  |

For the IE name “Re-connect Cell ID”, it can be kept, or alternatively “Suitable Cell ID” can be used because this cell should be a suitable target cell to handover the UE from the source.

For the IE type and reference of “Re-connect Cell ID”, it is beneficial to include both E-UTRA cell and NR cell for future proof.

For the presence of UE RLF Report Container, the source NR node may not decode the LTE UE RLF Report Container, it is write as Option for this reason. For simplicity, the IE can also be defined as Mandatory. We have no strong view.

A TP is provided in Annex to fix those FFSs in Annex.

Proposal 2: Agree the TP for TS38.413 in Annex.

3	Conclusion

In this contribution, the stage 3 impacts on MRO for inter-system inter-RAT handover for voice fallback are discussed. We have the following observations and proposals.

Proposal 1: It is beneficial for the UE to report the time between MCG failure and SCG failure for fast MCG failure recovery.

Proposal 2: Agree the TP for TS38.413 in Annex.

Reference

[1] Chairman notes for RAN3#120 meeting

Annex: TP for TS38.413

#### 9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |  |  |
| >Too early Inter-system HO |  |  |  |  | - |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |  |  |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |
| >Inter-system Unnecessary HO |  |  |  |  | - |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |  |  |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |  |  |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |  |  |
| >>Candidate Cell List |  | 1 |  |  |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |  |  |
| >>>>>Candidate PCI |  |  |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |  |  |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |  |  |
| >Inter-system Mobility Failure for Voice Fallback |  |  |  |  | YES | ignore |
| >>Source Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for HO |  |  |
| >>Failure Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the failure cell for HO |  |  |
| >>Re-connect Cell ID | O |  | CGI 9.3.1.xx | CGI of a suitable E-UTRA cell or NR Cell |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |

#### 9.3.1.xx	CGI

This IE is used to globally identify a cell in NG-RAN or E-UTRAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE CGI | M |  |  |  |
| >NR |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  |
| >E-UTRA |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  |

**************************** Next Change *************************

9.4.5	Information Element Definitions

****** skip unchanged part******

id-IncludeBeamMeasurementsIndication,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-intersystemMobilityFailureforVoiceFallback,

id-LastEUTRAN-PLMNIdentity,

****** skip unchanged part******

CEmodeBrestricted ::= ENUMERATED {

restricted,

not-restricted,

...

}

CGI ::= CHOICE {

nR-CGI			NR-CGI,

eUTRA-CGI		EUTRA-CGI,

choice-Extensions		ProtocolIE-SingleContainer { { CGI-ExtIEs} }

}

CGI-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

CNAssistedRANTuning ::= SEQUENCE {

expectedUEBehaviour					ExpectedUEBehaviour							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {CNAssistedRANTuning-ExtIEs} }	OPTIONAL,

...

}

****** skip unchanged part******

InterSystemHandoverReportType ::= CHOICE {

tooearlyIntersystemHO				TooearlyIntersystemHO,

intersystemUnnecessaryHO			IntersystemUnnecessaryHO,

choice-Extensions		ProtocolIE-SingleContainer { { InterSystemHandoverReportType-ExtIEs} }

}

InterSystemHandoverReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-intersystemMobilityFailureforVoiceFallback  CRITICALITY ignore	TYPE  IntersystemMobilityFailureforVoiceFallback 	PRESENCE mandatory },

...

}

IntersystemMobilityFailureforVoiceFallback ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

reconnectCellID	CGI                   OPTIONAL,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureforVoiceFallback-ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureforVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- S