3GPP TSG-RAN WG3 Meeting #121bis	R3-235230

Xiamen, China, 9th – 13th October 2023

Agenda item:	 10.2.2

Source:		Lenovo

Title:		(TP for SON BLCR to TS38.423) MRO for CPAC

Document for:		Discussion and Decision

1	Introduction

RAN3#119 meeting [1] achieved agreements on MRO for CPAC as below:

Update the definition to wrong PSCell change/addition should be splitting to sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the target node.

Information available in the network nodes should not be included in the SCGFailureInformation.

Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN.

For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events.

- RAN3#121 meeting [2] achieved further agreements as below:

Include CPC candidate cell list and CPC execution condition(s) to the message from the MN to the source SN.

The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

In this paper, we would further discuss the MRO for CPAC.

2	Discussion

In R17 MRO for PSCell change failure, MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

The mechanism of R17 MRO for PSCell change failure can be taken as baseline for MRO for CPAC, i.e. after MN receives CPAC failure related information from the UE, MN performs initial analysis to identify the node that caused the failure in the CPAC procedure, the SCG FAILURE INFORMATION REPORT message can be used to transfer CPC failure related information from MN to the last serving SN to verify whether intra-SN PSCell change has been triggered in the last serving SN, the SCG FAILURE TRANSFER can be used to inform MN that the root cause of CPC failure may have occurred in other nodes e.g. source SN. Then, MN may forward the SCG Failure Information to the source SN for root cause analysis.

RAN3#121 meeting agreed the MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated. The detailed solution of MRO analysis at network side for CPA or CPC is FFS.

For CPA or MN initiated CPC, as summarized in [3], there are two options on table:

Option 1:

  - if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - FFS: Indication whether a candidate PSCell is accepted by the (candidate) target (The candidate PSCell list recommended by the initiating node is already agreed).

  - Otherwise, the MN makes the optimization

Option 2:

  - MN always send the SCG Failure Information Report message to the last serving SN.

  - The last serving SN feedback if the problem is not brought by itself.

  - the MN makes the optimization

Option 1 is raised to reflect RAN3# 119 meeting’s agreements about updates of definition to wrong PSCell change/addition to split it to sub cases: 1) the wrong candidate cells list provided by the initiating node or 2) the wrong candidate cell is selected by the target node, which has been captured in BLCR to TS 37.340 [4] as following:

CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN, it is wrong target PSCell selection at the (candidate) target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

We can find that Option 2 totollay follows the principle of R17 MRO for PSCell change failure, however, since CPAC candidate PSCell(s) are selected by (candidate) target SN(s) during CPAC preparation phase, compared with legacy PSCell addition or change procedure, besides the source SN, the last serving SN or the MN, (candidate) target SN(s) should also be considered as one node caused the failure as Option 1 illustrated.

In case of CPA or MN initiated CPC, MN performs initial analysis to identify the node e.g. (candidate) target SN(s) that caused the failure, because MN knows the CPAC candidate PSCells recommended by MN, the CPAC candidate PSCells selected by the (candidate) target SN(s) and the suitable PSCell which is selected by MN based on measurements reported from the UE, it is reasonable of MN to perform further root cause analysis, i.e. MN decides whether CPAC candidate PSCell(s) provided by MN to candidate SN(s) are suitable, or whether CPAC candidate PSCell(s) selected by the (candidate) target SN(s) are suitable, for example:

- if the selected suitable PSCell is not included in the CPAC candidate PSCells provided by MN to (candidate) target SN(s), MN detects the CPAC candidate PSCells provided by MN are unsuitable, so MN may modify the list of CPAC candidate PSCells;

- if the selected suitable PSCell is included in the CPAC candidate PSCells provided by MN to (candidate) target SN(s), but not included in the CPAC candidate PSCells selected by (candidate) target SN, MN detects the wrong candidate PSCell is selected by the (candidate) target SN; then, MN may transmit the analysis result to the (candidate) target SN, e.g. MN informs the (candidate) target SN that wrong CPAC candidate PSCell is selected by the (candidate) target SN e.g. via the SCG Failure Information Report message.

Proposal 1: For CPA or MN initiated CPC, if the suitable PSCell is one of the candidate target PSCells provided by the MN to a (candidate) target SN, but not one of the candidate target PSCells selected by the (candidate) target SN, MN may detect that wrong candidate target PSCell is selected by the (candidate) target SN, and, MN may inform the (candidate) target SN that wrong candidate target PSCell is selected by the (candidate) target SN, e.g. via the MCG Failure Information Report message.

In such a way, MN does not need to transfer suitable PSCell ID to the (candidate) target SN because it is better for MN rather than (candidate) target SN to perform root cause analysis.

Proposal 2: MN does not need to transfer suitable PSCell ID to the (candidate) target SN.

Furthermore, because (candidate) target SN decides whether a candidate PSCell is accepted or not, of course the (candidate) target SN knows whether a candidate PSCell is accepted or not by itself, we don’t see the necessity of MN to transfer an indication on whether a candidate target PSCell is accepted by the (candidate) target SN to the (candidate) target SN, additionally, it is not correct for the MN to decide such an indication.

Proposal 3: MN does not need to transfer an indication on whether a candidate target PSCell is accepted by the (candidate) target SN to the (candidate) target SN.

Similarly, in case of SN initiated CPC, MN performs initial analysis, following the principle of R17 MRO for PSCell change failure, MN may send SCG failure information report message to source SN, and source SN performs root cause analysis:

- if the selected suitable PSCell is not included in the CPAC candidate PSCells provided by S-SN to (candidate) target SN(s), S-SN knows the CPAC candidate PSCells provided by S-SN are unsuitable, S-SN may modify the list of CPAC candidate PSCell(s);

- if the selected suitable PSCell is included in the CPAC candidate PSCells provided by S-SN to (candidate) target SN(s) but not included in the CPAC candidate PSCells selected by (candidate) target SN(S), S-SN knows wrong candidate PSCell(s) is selected by the (candidate) target SN(s); if so, S-SN informs (candidate) target SN that wrong candidate PSCell is selected directly or via MN (S-SN may first indicate the ID of (candidate) target SN where wrong candidate PSCell is selected to the MN e.g. via the SCG FAILURE TRANSFER message, then MN can inform to the corresponding (candidate) target SN that wrong candidate PSCell is selected e.g. via the MCG Failure Information Report message), then (candidate) target SN may modify the selected candidate PSCell(s);

Proposal 4: For SN initiated CPC, if the suitable PSCell is one of the candidate target PSCells provided by the source SN to a (candidate) target SN, but not one of the candidate target PSCells selected by the (candidate) target SN, source SN may detect that wrong candidate target PSCell is selected by the (candidate) target SN,

- S-SN may indicate the ID of (candidate) target SN where wrong candidate PSCell is selected to the MN, e.g. via the SCG FAILURE TRANSFER message;

- MN may inform to the corresponding (candidate) target SN that wrong candidate PSCell is selected, e.g. via the MCG Failure Information Report message.

3	Conclusion

In this contribution, MRO enhancements for CPAC are further discussed. We have the following proposals:

Proposal 1: For CPA or MN initiated CPC, if the suitable PSCell is one of the candidate target PSCells provided by the MN to a (candidate) target SN, but not one of the candidate target PSCells selected by the (candidate) target SN, MN may detect that wrong candidate target PSCell is selected by the (candidate) target SN, and, MN may inform the (candidate) target SN that wrong candidate target PSCell is selected by the (candidate) target SN, e.g. via the MCG Failure Information Report message.

Proposal 2: MN does not need to transfer suitable PSCell ID to the (candidate) target SN.

Proposal 3: MN does not need to transfer an indication on whether a candidate target PSCell is accepted by the (candidate) target SN to the (candidate) target SN.

Proposal 4: For SN initiated CPC, if the suitable PSCell is one of the candidate target PSCells provided by the source SN to a (candidate) target SN, but not one of the candidate target PSCells selected by the (candidate) target SN, source SN may detect that wrong candidate target PSCell is selected by the (candidate) target SN,

- S-SN may indicate the ID of (candidate) target SN where wrong candidate PSCell is selected to the MN, e.g. via the SCG FAILURE TRANSFER message;

- MN may inform to the corresponding (candidate) target SN that wrong candidate PSCell is selected, e.g. via the MCG Failure Information Report message.

Reference

[1] Chairman notes for RAN3#119 meeting

[2] Chairman notes for RAN3#121 meeting

[3] R3-234540, Summary_of_offline_disc_CB # SONMDT2_MRO, Samsung

[4] R3-234789, (BLCR to 37.340) Addition of SON Rel.18 features, Nokia

Annex: TP for TS38.423

### 8.3.17	SCG Failure Information Report

#### 8.3.17.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to the S-NG-RAN node or (candidate) T-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.17.2	Successful Operation

Figure 8.3.17.2-1: SCG Failure Information Report, successful operation

The M-NG-RAN node initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the S-NG-RAN node or (candidate) T-NG-RAN node. Upon receiving the message, the S-NG-RAN node shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the S-NG-RAN node;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the S-NG-RAN node;

-	the Wrong Target PSCell Selection Indication IE, if M-NG-RAN node detects it is wrong candidate target PSCell selection at the (candidate) T-NG-RAN;

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If received, the S-NG-RAN node uses the above information for SCG failure reason detection and optimisation.

#### 8.3.17.3	Unsuccessful Operation

Not applicable.

#### 8.3.17.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.18	SCG Failure Transfer

#### 8.3.18.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the M-NG-RAN node that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

#### 8.3.18.2	Successful Operation

Figure 8.3.18.2-1: SCG Failure Information Transfer, successful operation

S-NG-RAN node initiates the procedure by sending the SCG FAILURE TRANSFER message to M-NG-RAN node.

If the SCG FAILURE TRANSFER message includes the Target S-NG-RAN Node List IE, the M-NG-RAN node shall, if supported, store the information.

If received, M-NG-RAN node uses the information according to TS 38.300 [9].

#### 8.3.18.3	Unsuccessful Operation

Not applicable.

#### 8.3.18.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node or (candidate) T-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node or (candidate) T-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | The SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.xy |  | YES | ignore |
| Wrong Target PSCell Selection Indication | O |  |  | ENUMARATED (true, …) | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.30	SCG FAILURE TRANSFER

This message is sent by the S-NG-RAN node to the M-NG-RAN node to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Target S-NG-RAN Node List |  | 0 .. <maxnoofTargetSNs> |  |  | YES | ignore |
| >Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – | – |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

HashedUEIdentityIndexValue,

RaReportIndicationList,

SuccessfulPSCellChangeReportInformation,

CPACConfiguration,

WrongTargetPSCellSelectionIndication,

TargetS-NG-RANNodeList

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-HashedUEIdentityIndexValue,

id-RaReportIndicationList,

id-SuccessfulPSCellChangeReportInformation,

id-CPACConfiguration,

id-WrongTargetPSCellSelectionIndication,

id-TargetS-NG-RANNodeList,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- SCG FAILURE INFORMATION REPORT

--

-- **************************************************************

ScgFailureInformationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureInformationReport-IEs}},

...

}

ScgFailureInformationReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-SourcePSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-FailedPSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-SCGFailureReportContainer					CRITICALITY ignore		TYPE SCGFailureReportContainer					PRESENCE mandatory }|

{ ID id-SNMobilityInformation				CRITICALITY ignore		TYPE SNMobilityInformation						PRESENCE optional }|

{ ID id-CPACConfiguration					CRITICALITY ignore		TYPE CPACConfiguration				         PRESENCE optional}|

{ ID id-WrongTargetPSCellSelectionIndication								CRITICALITY ignore		TYPE WrongTargetPSCellSelectionIndication							PRESENCE optional },

...

}

-- **************************************************************

--

#### -- SCG FAILURE TRANSFER

--

-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureTransfer-IEs}},

...

}

ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-TargetS-NG-RANNodeList								CRITICALITY ignore		TYPE TargetS-NG-RANNodeList								PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- T

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

TargetS-NG-RANNodeList ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF TargetS-NG-RANNodeList-Item

TargetS-NG-RANNodeList-Item ::= SEQUENCE {

target-S-NG-RANnodeID					GlobalNG-RANNode-ID,

iE-Extensions							ProtocolExtensionContainer { { TargetS-NG-RANNodeList-Item-ExtIEs} } OPTIONAL

}

TargetS-NG-RANNodeList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-WrongTargetPSCellSelectionIndication						ProtocolIE-ID ::= xyz

id-TargetS-NG-RANNodeList									ProtocolIE-ID ::= xyz

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>