3GPP TSG-RAN WG3 Meeting #121bis	R3-235237

Xiamen, China, 9th – 13th October 2023

Agenda item:	  11.4

Source:	  Lenovo

Title:	Remaining issues on QoE measurement during intra-5GC inter-RAT handover

Document for:		Approval

1	Introduction

This contribution discusses the potential issues on QoE measurement during intra-5GC inter-RAT handover.

2	Discussion

- In RAN3#119bis-e meeting, it was agreed and FFSs:

WA: For HO from LTE/5GC to NR, there is no impacts to RAN3.

For HO from NR to LTE/5GC case, identify the impact on RAN3

- In RAN3#120 meeting, it was agreed that:

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

- In RAN3#121, it was further agreed that:

QoE measurement continuity from NR to LTE/5GC and vice versa can be supported by the current Area Scope for QMC IE.

The scenario where a UE is handed over from NR to LTE/5GC and then back to NR is not considered in the context of continuity of QoE measurements during intra-5GC inter-RAT HO.

HO from LTE/5GC to NR can be supported without introducing any new IEs.

However, there are following remaining issues:

#### 5.4.3.4 Successful completion of the mobility from NR Upon successfully completing the handover, at the source side the UE shall: 1> reset MAC; 1> stop all timers that are running except T325, T330 and T400; 1> release ran-NotificationAreaInfo, if stored; 1> release the AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if stored; 1> release all radio resources, including release of the RLC entity and the MAC configuration; 1> release the associated PDCP entity and SDAP entity for all established RBs; NOTE : PDCP and SDAP configured by the source RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure. 1> if the targetRAT-Type is set to eutra and the nas-SecurityParamFromNR is included: or 1> if the targetRAT-Type is set to utra-fdd: 2> indicate the release of the RRC connection to upper layers together with the release cause 'other'.

For handover from gNB to ng-eNB, only one QoE measurement configuration can be kept and other QoE measurement configurations (if any) should be released. However, according to current TS 38.331, subclause 5.4.3.4, the other QoE measurement configurations are not released during handover from gNB to ng-eNB. How UE releases the other QoE measurement configurations in source side needs to be addressed. One solution is the gNB releases the other QoE measurement configurations before handover by RRC Reconfiguration message. But this solution requires the gNB to perform additional RRC Reconfiguration procedure that causes extra delay for handover.

Observation 1: For handover from gNB to ng-eNB the other QoE measurement configurations are not released during handover procedure according to existing TS 38.331. How UE releases these QoE measurement configurations in source side needs to be addressed.

#### 5.3.5.13d Application layer measurement configuration 1> if measConfigAppLayerToAddModList is included in appLayerMeasConfig within RRCReconfiguration or RRCResume: 2> for each measConfigAppLayerId value included in the measConfigAppLayerToAddModList: 3> if measConfigAppLayerContainer is included for the corresponding MeasConfigAppLayer configuration: 4> forward the measConfigAppLayerContainer, the measConfigAppLayerId and the serviceType to upper layers considering the serviceType; 3> consider itself to be configured to send application layer measurement report for the measConfigAppLayerId in accordance with 5.7.16;

According to TS 38.331, subclause 5.3.5.13d, whenever the UE receives a new QoE measurement configuration from the serving gNB, the UE will forward the received measConfigAppLayerContainer to the upper layers. Although the subclause 5.3.5.13d is for QoE measurement configuration within NR, the target gNB needs to provide the same configuration in handover command during handover from ng-eNB to gNB according to the discussion in previous RAN3 meeting.

For handover from ng-eNB to gNB, the target gNB needs to provide for the QoE measurement configuration to continue the measConfigAppLayerContainer in the handover command to UE. And the UE shall forward the received measConfigAppLayerContainer to the upper layers. If the container received previously from source side is not released, the UE application layer will receive a duplicated application layer container.  The consequence is that the UE application layer’s behaviour is unclear whether the first received application layer measurement configuration is replaced by the followed received application layer measurement configuration or not.

Observation 2: For handover from ng-eNB to gNB, the UE application layer may receive a duplicated measConfigAppLayerContainer. How UE application layer handles the duplicated container for Application Layer Measurement Configuration needs to be checked with SA4.

Since RAN3 is the leading group of the WI, it is RAN3’s responsibility to ensure the end to end solution works well for QoE measurements during IRAT handover. We think an LS to RAN2 and CT4 to check their views on the issues in observation 1&2 are needed.

Proposal: Send an LS to RAN2&SA4 for checking their views on the potential issues:

- For handover from gNB to ng-eNB the other QoE measurement configurations are not released during handover procedure according to existing TS 38.331. How UE releases the QoE measurement configurations in source side needs to be addressed.

- For handover from ng-gNB to gNB, the UE application layer may receive a duplicated measConfigAppLayerContainer. How UE application layer handles the duplicated container for Application Layer Measurement Configuration needs to be checked with SA4.

3	Conclusion

This contribution discusses the potential issues on QoE measurement during intra-5GC inter-RAT handover.

Observation 1: For handover from gNB to ng-eNB the other QoE measurement configurations are not released during handover procedure according to existing TS 38.331. How UE releases these QoE measurement configurations in source side needs to be addressed.

Observation 2: For handover from ng-eNB to gNB, the UE application layer may receive a duplicated measConfigAppLayerContainer. How UE application layer handles the duplicated container for Application Layer Measurement Configuration needs to be checked with SA4.

Proposal: Send an LS to RAN2&SA4 for checking their views on the potential issues:

- For handover from gNB to ng-eNB the other QoE measurement configurations are not released during handover procedure according to existing TS 38.331. How UE releases these QoE measurement configurations in source side needs to be addressed.

- For handover from ng-gNB to gNB, the UE application layer may receive a duplicated measConfigAppLayerContainer. How UE application layer handles the duplicated container for Application Layer Measurement Configuration needs to be checked with SA4.

Reference