3GPP TSG-RAN WG3 #121bis	R3-235240

Xiamen, China, 9th – 13th Octorber 2023

Agenda Item:	12.2.2.2

Source:	Lenovo

Title: 		Left issues related to mobility enhancement use case

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss the left issues related to mobility enhancement use case.

# 2	Discussion

## 2.1	FFSs from last meeting

The presence of time of stay in the Measured UE Trajectory is FFS. Whether to re-use the UE History Information IE or introduce a new IE or re-use the IE defined for predicted UE Trajectory is FFS. Whether the predicted UE Trajectory report needs to be indicated in the Report Characteristics IE of DATA COLLECTION REQUEST message is FFS.

# Presence of time of stay in measured UE trajectory

As discussed in the previous RAN3 meeting, the measured UE trajectory can be used as training input or as feedback for AI model. Currently, the UE trajectory prediction may or may not include predicted time UE stays in cell (the IE is optional), meaning AI model at source gNB may or may not be able to predict the time of stay, which lead to different requirements for the training input.

- For AI model that can predict the time of stay, it requires measured time of stay for each visited cell as input.

- For AI model that does not predict the time of stay, it does not require measured time of stay for each visited cell as input.

Depending on if source gNB wants to train an AI model capable of predicting time of stay, the time of stay in measured UE trajectory may or may not be needed.

To have a design that can work for both possibilities (i.e., source gNB wants to train an AI model which may or may not predict time of stay), RAN3 could either:

- Option 1: Agree the presence of time of stay in measured UE trajectory is mandatory, then it is upon source gNB to decide whether/how to use it.

- Option 2: Introduce additional indicator for source gNB to indicate target gNB whether time of stay for each visited cell is required

In our understanding Option 1 would be easier.

RAN3 agrees the presence of time of stay in measured UE trajectory is mandatory. Otherwise, RAN3 introduces additional indicator for source gNB to indicate target gNB whether time of stay for each visited cell is required.

# UHI or new IE

Another question is whether UHI can be reused to carry the measured UE trajectory from the target gNB to source gNB, or a new IE should be used. After checking the spec, irrelevant elements in UHI are optional, thus, it seems ok to reuse UHI. On the other hand, we understand it could be cleaner to use a new IE carrying only necessary information. Without a strong preference, RAN3 can simply pick one according to majority view.

#### 9.3.1.97 Last Visited NG-RAN Cell Information This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Global Cell ID; M; NG-RAN CGI 9.3.1.73; -. Cell Type; M; 9.3.1.98; -. Time UE Stayed in Cell; M; INTEGER (0..4095); The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095.; -. Time UE Stayed in Cell Enhanced Granularity; O; INTEGER (0..40950); The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950.; -. HO Cause Value; O; Cause 9.3.1.2; The cause for the handover.; -. Last Visited PSCell List; 0..<maxnoof PSCellsPer PrimaryCellin UEHistoryInfo>; List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list.; YES; ignore. >Last Visited PSCell Information; M; 9.3.1.235; The PSCell related information.; -.

RAN3 is suggested to follow majority view and use either UHI or new IE to carry measured UE trajectory from target gNB to source gNB.

# Request of UE trajectory prediction

Some companies raised the issue that the source gNB should provide the UE trajectory prediction only if the target gNB requests. However, in our understanding, RAN3 has been discussing the UE trajectory prediction transfer all along based on a different motivation, that if source gNB has UE trajectory prediction available anyway, it could provide the available prediction result to target gNB as additional information for target gNB to consider in future handover. In other words, source gNB makes UE trajectory prediction for its own good to make proper handover decision, while providing to target gNB is something extra without complexity. At this late stage, we prefer to follow the original RAN3 understanding and do not introduce the request of UE trajectory prediction upon request from target gNB.

RAN3 does not support UE trajectory prediction request mechanism in release 18.

## 2.2	Common issues as for UE performance feedback

The design of measured UE trajectory collection after handover has similar issues as UE performance feedback design. We have analysed the details in our paper [1]. And here we briefly explain the applicability for measured UE trajectory collection.

Until now, the DATA COLLECTION REQUEST message can be used to configure and request the following data from the neighbour gNB:

- Measurements/Prediction, e.g., EC, predicted resource status.

- UE performance after handover, e.g., UE throughput, latency, packet loss rate.

- UE trajectory after handover, e.g., visited cells.

It is suggested to group the configurations for different data request purposes into different IEs or IE groups. Since each data request configuration may contain different elements with different meanings.

RAN3 is suggested to convey the configuration for prediction request, configuration for UE performance feedback, and configuration for measured UE trajectory after handover as three separate IEs or IE groups.

For UE performance feedback, RAN3 agreed that the UE performance feedback reporting should occur no later than the expiration of the reporting duration. Similar as UE performance feedback reporting, any UE trajectory reporting after handover should occur no later than the expiration of reporting duration. Besides, if the number of handovers is configured, any UE trajectory reporting after handover should occur no later than the configured number of handovers is exceeded.

Any UE Performance Feedback reporting should occur no later than the expiration of the Reporting Duration. The Measured UE Trajectory reporting is one-time report. The conditions triggering the one-time reporting are at least: - Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE); - Condition B: UE leaves the target node; - Condition C: Expiration of the report time duration; - Condition D: The configured number of hand-overs within the same target node is reached. Report time duration and configured number of intra-NG-RAN node inter-cell handovers are included in DATA COLLECTION REQUEST message.

If report time duration is configured for the measured UE trajectory report, any UE trajectory reporting after handover should occur no later than the expiration of reporting duration.

If the number of handovers is configured for the measured UE trajectory report, any UE trajectory reporting after handover should occur no later than the configured number of handovers is exceeded.

On the other hand, in our understanding there are two cases, measured UE trajectory could be reported before report duration expiry:

- Case#1: Due to interruption and UE trajectory can be no longer measured by the target gNB

- Case#2: Due to signalling efficiency reason, gNB can report the measured trajectory of many UEs in the same DATA COLLECTION UPDATE message even if the configured report duration for some UEs is not reached.

For Case#1, the interruption could be any of the following:

- Radio link failure happens

- The UE enters RRC inactive/idle state

- The UE is handed over to another gNB

In our understanding, it is beneficial to indicate the source gNB the cause of interruption of measured UE trajectory considering they are related to the UE trajectory prediction made by source gNB as well considering:

- If it is because of UE being handed over to another gNB, itself is useful UE trajectory information and can be used as training input data or feedback data.

- If it is because of RLF in short time after handover, it implies suboptimal handover decision.

- If it is because of RRC state change, then the interruption should not be interpreted as relevant to a UE trajectory prediction or suboptimal handover decision as above.

There are three existing cause values could be used with extended meaning:

- User Inactivity,

- Radio Connection With UE Lost,

- Failure in the Radio Interface Procedure

If target gNB can no longer provide measured UE trajectory for a particular UE (e.g., due to RLF, RRC state, HO), the target gNB indicates to the source gNB via a cause value associated with the UE in the DATA COLLECTION UPDATE message.

RAN3 further discusses if legacy cause values can be reused with extended meaning or new cause values are needed.

For UE performance feedback, RAN3 has agreed that if Reporting Duration is configured, it should start at successful handover execution. First, we believe similar agreement applies to measured UE trajectory too.

A new IE is introduced in the Data Collection Request, indicating the Reporting Duration for the UE Performance Feedback, starting at successful Handover Execution.

RAN3 agrees that if Reporting Duration is configured for the measured UE trajectory, it starts at successful handover execution.

However, if target gNB strictly follow the time configuration, and start counting report duration immediately after the handover, it may lead to a sub-optimal situation. Since handovers of different UEs cannot occur at exactly the same time, eventually the target gNB may trigger many DATA COLLECTION UPDATE messages each carries the measured UE trajectory for one particular UE. At the end, one DATA COLLECTION UPDATE message will carry measured trajectory for only one UE even if it is designed to carry multiple UEs’ at the same time.

To avoid above mentioned sub-optimal situation, and use DATA COLLECTION UPDATE message to carry measured trajectory for many UEs at the same time, RAN3 is suggested to support gNB to report the measured trajectory of one UE before the report duration expiry proactively.

gNB can report the measured UE trajectory of many UEs in the same DATA COLLECTION UPDATE message even if the configured periodicity/report duration for some UEs is not reached.

RAN3 agrees the relevant TP in R3-235239.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Depending on if source gNB wants to train an AI model capable of predicting time of stay, the time of stay in measured UE trajectory may or may not be needed.

Based on the discussion above, we propose:

Proposal 1	RAN3 agrees the presence of time of stay in measured UE trajectory is mandatory. Otherwise, RAN3 introduces additional indicator for source gNB to indicate target gNB whether time of stay for each visited cell is required.

Proposal 2	RAN3 is suggested to follow majority view and use either UHI or new IE to carry measured UE trajectory from target gNB to source gNB.

Proposal 3	RAN3 does not support UE trajectory prediction request mechanism in release 18.

Proposal 4	RAN3 is suggested to convey the configuration for prediction request, configuration for UE performance feedback, and configuration for measured UE trajectory after handover as three separate IEs or IE groups.

Proposal 5	If report time duration is configured for the measured UE trajectory report, any UE trajectory reporting after handover should occur no later than the expiration of reporting duration.

Proposal 6	If the number of handovers is configured for the measured UE trajectory report, any UE trajectory reporting after handover should occur no later than the configured number of handovers is exceeded.

Proposal 7	If target gNB can no longer provide measured UE trajectory for a particular UE (e.g., due to RLF, RRC state, HO), the target gNB indicates to the source gNB via a cause value associated with the UE in the DATA COLLECTION UPDATE message.

Proposal 8	RAN3 further discusses if legacy cause values can be reused with extended meaning or new cause values are needed.

Proposal 9	RAN3 agrees that if Reporting Duration is configured for the measured UE trajectory, it starts at successful handover execution.

Proposal 10	gNB can report the measured UE trajectory of many UEs in the same DATA COLLECTION UPDATE message even if the configured periodicity/report duration for some UEs is not reached.

Proposal 11	RAN3 agrees the relevant TP in R3-235239.