3GPP TSG-RAN WG3 Meeting #121-bis	R3-235247

09 – 13 October 2023, Xiamen, China

Title: 	Discussion on L1/L2 based inter-cell mobility

Source: 	Lenovo

Agenda item:	14.2

Document Type:	Discussion and Approval

1. Introduction

The signalling support for LTM was discussed at the RAN3#121 with the following achievements. The summary of offline discussion could be found in [1].

Turn the WA to agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch. Issue 1: L1 measurement configuration and reporting For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU. Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3. FFS on the details of Stage 2 and stage3. The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations. FFS whether CU needs to request the DU to provide the RS configuration. RS configuration sends from C-DU to S-DU via CU or CU request. Need to discuss detail. Issue 2: TCI state configuration The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM. After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE. FFS on the need of an explicit request from CU Issue 3: Selected beam transfer FFS for the detail of selected beam information. For the part of CU to target DU is FFS of which procedure can be used. FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID. Option 1: Two new class 2 procedures with different name. Option 2: same new class 2 procedure Option 3: class 1 procedure. Down selection among those 3 options at next meeting. Issue 4: TA acquisition RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue. CU shall send the TA value to the source DU. In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure. Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2. WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU. WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU. How the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU). Source DU is in charge of checking the TA validity. Issue 6: LTM and L3 handover collision In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority. In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority, WA: In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority. Issue 7: Reference configuration Prioritize the reference configuration on F1 at next meeting. List all options and down selection.

In this contribution, we will further discuss the remaining issues of LTM related to RAN3 impact.

2. Discussion

Part A: HO collision

At the RAN3#121 meeting, the HO collision between the LTM and L3 HO was discussed, and a working assumption was made:

- WA: In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

- Case 2: The CU receives the LTM notification from the DU after the CU has started preparing (inter-CU) L3 HO but has not triggered L3 HO. In this case, the CU may need to cancel the prepared L3 HO over Xn interface, or the CU may coordinate with the DU before the L3 HO preparation to avoid such collision.

- Case 3: The DU receives the L3 HO command while the DU has already triggered the LTM. In this case, the UE may have already switched to another cell, and the L3 HO is no longer applicable. Therefore, the DU may fail the L3 HO with necessary indication about the cause.

However, in the current BLCR to TS 38.473, the cause is captured with value “LTM command triggered”, which means the action failed because the LTM command has been triggered. In our view, the current BLCR already covers the case of cross signalling on F1, and no further discussions are needed for the HO collision between the LTM and L3 HO. Therefore, we propose to turn the WA into agreement.

Proposal 1: Confirm a cause value meaning LTM has high priority in case that LTM and L3 handover are triggered almost simultaneously (cross signalling on F1).

Part B: TA acquisition

Regarding early TA acquisition without RAR, after UE receives the PDCCH order from source cell, UE transmits preamble to the corresponding candidate cell based on the RACH resource for early TA acquisition. Once the candidate cell receives the preamble, the candidate cell will calculate TA value and transmit early TA value to source cell/DU via CU. If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell can determine the failure of early TA acquisition. The candidate cell/DU transmits the message and indicates this to the source cell (DU). This will enable the source cell to determine if it would like the UE to re-attempt Early TA acquisition and when so, source cell may request dedicated PRACH resources from the candidate cell (DU).

Proposal 2: If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU).

Source cell (DU) may store early TA value list for some candidate cells. Source cell will transmit LTM command MAC CE to UE once the source cell determines to switch to a candidate cell which may be located in another DU. After candidate cell receives the RRC reconfiguration complete message from UE, the candidate cell may transmit indication to source cell. In subsequent-LTM, UE will continue keeping the LTM candidate configuration after completing cell switching. If source cell (DU) does not transfer the stored early TA value list, UE will be expected to get early TA value again based on RACH. Therefore, source cell may provide the early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

Proposal 3: Source cell may provide the stored early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

It was agreed in RAN1 that UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. If UE-based TA measurement is configured to UE, that means UE itself can calculate the TA for LTM. In case of early TA acquisition, CU transmits the request to target DU to configure RACH resource for early TA acquisition. It means CU decides whether to use early TA for LTM. Therefore, CU can decide whether UE based TA measurement can be supported or not. One reason could be CU is aware of the UE capability. In addition, CU can select one of UE based TA measurement and early TA acquisition for RACH-less LTM.

Proposal 4: CU decides whether UE based TA measurement can be supported or not.

If UE based TA measurement is used, target cell may configure CG to UE. If CG is not configured, target cell will transmit PDCCH addressed by C-RNTI to UE once LTM is triggered. Therefore, CU needs to indicate UE based TA measurement to target DU. Then, target DU can decide whether CG is configured or not.

Proposal 5: CU needs to indicate UE based TA measurement to target DU. Then, target DU can decide whether CG is configured or not.

In case of UE based TA measurement, UE will monitor PDCCH via C-RNTI if CG is not configured as RACH-less LTM with early TA. Therefore, target DU should be informed when LTM with UE based TA is triggered as RACH-less LTM with early TA.

Proposal 6: Target DU should be informed when LTM with UE based TA is triggered as RACH-less LTM with early TA.

Part C: Beam alignment

At the RAN3#121 meeting, the group agreed that the LTM Cell Change Notification procedure is reused to transfer the selected beam from the source DU to the target DU via CU. However, there is a remaining FFS for the detailed of selected beam information.

According to the RAN1 agreements [2], the beam indication of target cell and TCI state activation for candidate cell is by indicating the R17 TCI state index.

- Beam indication of target cell(s) RAN1 has made the following agreement in RAN1#112bis-e - Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported), - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE - Alt. 2: By indicating Rel-17 TCI state index This agreement implies that the source cell must be provided with information so that the source cell can send a Rel-17 TCI state index of the target/candidate cell(s) to the UE even when source cell and target/candidate cell(s) belong to different DUs.

Therefore, it is straightforward to use the TCI state index to identify the selected beam in the LTM Cell Change Notification procedure.

Proposal 7: The selected beam in the LTM Cell Change Notification procedure is the TCI state index.

Part D: L1 measurement

In the current BL CR, the lower layer measurement report is used to trigger the inter-cell mobility execution decision. However, it is not clear what is the lower layer measurement report. Since RAN1 and RAN2 already agreed to use the L1 measurement report, we propose to use the L1 measurement report for better understanding.

Proposal 8: L1 measurement report is used to trigger the inter-cell mobility execution decision.

At the RAN3#121 meeting, the group agreed to add the RS configuration in UE Context Setup/Modification in stage 2 and stage 3. However, there is a remaining FFS whether CU needs to request the DU to provide the RS configuration.

The RS configuration is used to generate the CSI measurement configuration, which is used for the L1 measurement and report. Without the RS configuration, the UE and the DU could not perform the L1 measurement towards the candidate cell. That is, the RS configuration of the candidate cell shall always be presented.

Therefore, it is reasonable that the DU provides the RS configuration during the LTM candidate cell configuration preparation, without an explicit request from the CU.

Proposal 9: No need of an explicit request from the CU to the candidate DU for providing the RS configuration.

The group also agreed that the CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source DU and/or other candidate DUs to generates the L1 configurations. However, there is a remaining FFS whether the source DU / candidate DU needs to provide the generated CSI resource configuration.

At the RAN2#122 meeting, RAN2 made the following agreements [3]:

- For L1 measurements for LTM - The RS configuration is provided to the UE per LTM candidate cell. - RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration. - RAN2 assumes that The CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM. - RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated). - The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration. - CSI reports for LTM candidates (neighbour cell reports for the purpose of LTM cell switch) are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted. - RAN2 assumes the following about CSI measurement reporting for LTM (final decision up to RAN1): - a. UE reports all measured LTM candidate cells in a single report; or - b. UE reports one or a subset of measured LTM candidate cell(s) in a report.

The CSI resource configuration is within the ServingCellConfig of the cell in which the report is to be transmitted. That is, the source DU should provide the CSI resource configuration when the UE connects to the source DU. To support subsequent LTM, the candidate DU should also provide the CSI resource configuration when the UE connects to the candidate DU. That is, the source DU and the candidate DU should provide the CSI resource configuration separately.

Therefore, it is proposed to remove the FFS for generated CSI resource configuration in the UE Context Modification Response message to the source DU and the candidate DU.

Proposal 10: Remove the FFS for generated CSI resource configuration in the UE Context Modification Response message to the source DU and the candidate DU.

Part E: TCI state

At the RAN3#121 meeting, the group agreed that after LTM configuration, the CU indicates the collected TCI state configurations of candidate cells to the source DU and all candidate DUs via UE Context Modification procedure for the UE.

However, the agreement is not reflected in current stage 2 BLCR from the candidate DU side. It is proposed to add the TCIs state of candidate cells in the UE Context Modification Request message from the CU to the candidate DU, e.g., in the step 7.

Proposal 11: Update the stage 2 BLCR to capture the agreement “After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE.”

There is a remaining FFS on the need of an explicit request from CU to the candidate DU for the TCI state configuration.

In order to minimize the time needed to acquire the target beam after receiving the LTM cell switch command, the network can provide the UE the candidate cell beam indication in the form of TCI states associated with the candidate cell. Without the TCI state configuration, the UE could not obtain the beam indication of target cell and perform TCI state activation for candidate cells.

In addition, according to the RAN1 agreements [2], the source CU must be provided with the TCI state information.

RAN1 has made the following agreement in RAN1#112bis-e - Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported), - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE - Alt. 2: By indicating Rel-17 TCI state index This agreement implies that the source cell must be provided with information so that the source cell can send a Rel-17 TCI state index of the target/candidate cell(s) to the UE even when source cell and target/candidate cell(s) belong to different DUs.

Therefore, the TCI state configuration of the candidate cell shall always be presented. It is reasonable that the candidate DU provides the TCI state configuration during the LTM candidate cell configuration preparation, without an explicit request from the CU.

Proposal 12: No need of an explicit request for the TCI state provision from the CU to the candidate DU.

Part F: Reference configuration

For the delta configuration, RAN2 made the following agreements:

- Whether the Reference configuration is a complete configuration or not is up to the network implementation. - Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. - The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

From the RAN2 agreements, the reference configuration is managed separately and explicitly signalled, i.e., not automatically derived from any other configuration, e.g., current configuration. There are two cases of the reference configuration generation:

- Case 1: The reference configuration is generated by the source DU or the CU. In this case, the CU has the reference configuration, and the CU may provide the reference configuration to the candidate DU. The candidate DU takes the reference configuration into account when generating the LTM candidate cell configuration. Since RAN3 agreed that the UE Context Setup procedure is reused for the LTM preparation for the inter-DU LTM, it is reasonable that the CU provides the reference configuration to the candidate DU using the UE Context Setup Request message.

Proposal 13: If the CU has the LTM reference configuration, the CU may provide the LTM reference configuration to the candidate DU in the UE Context Setup Request message.

- Case 2: The CU doesn’t have the reference configuration. In this case, the CU can’t provide the reference configuration to the candidate DU in the UE Context Setup Request message. Since the delta configuration is useful for service continuity and signalling overhead reduction, the candidate DU may provide the reference configuration to the CU during the UE Context Setup procedure, e.g., using the UE Context Setup Response message.

Proposal 14: If the CU doesn’t provide the LTM reference configuration, the candidate DU may provide the LTM reference configuration.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

HO collision

Proposal 1: Confirm a cause value meaning LTM has high priority in case that LTM and L3 handover are triggered almost simultaneously (cross signalling on F1).

TA acquisition

Proposal 2: If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU).

Proposal 3: Source cell may provide the stored early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

Proposal 4: CU decides whether UE based TA measurement can be supported or not.

Proposal 5: CU needs to indicate UE based TA measurement to target DU. Then, target DU can decide whether CG is configured or not.

Proposal 6: Target DU should be informed when LTM with UE based TA is triggered as RACH-less LTM with early TA.

Beam alignment

Proposal 7: The selected beam in the LTM Cell Change Notification procedure is the TCI state index.

L1 measurement

Proposal 8: L1 measurement report is used to trigger the inter-cell mobility execution decision.

Proposal 9: No need of an explicit request from the CU to the candidate DU for providing the RS configuration.

Proposal 10: Remove the FFS for generated CSI resource configuration in the UE Context Modification Response message to the source DU and the candidate DU.

TCI state

Proposal 11: Update the stage 2 BLCR to capture the agreement “After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE.”

Proposal 12: No need of an explicit request for the TCI state provision from the CU to the candidate DU.

Reference configuration

Proposal 13: If the CU has the LTM reference configuration, the CU may provide the LTM reference configuration to the candidate DU in the UE Context Setup Request message.

Proposal 14: If the CU doesn’t provide the LTM reference configuration, the candidate DU may provide the LTM reference configuration.

4. Reference

- R3-234574, Summary of Offline Discussion: CB: # MobilityEhn_L1L2Mobility, Huawei (moderator).

- R3-232569, LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, RAN1.

- R3-233720, LS on L1 measurements for LTM, RAN2.