3GPP TSG-RAN WG3 Meeting #121-bis	R3-235264

Xiamen, China, Oct 9th - Oct 13th, 2023

Agenda item:	14.2

Source:	Qualcomm Incorporated

Title:	Signalling Support for LTM

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #121 [1]), solutions were agreed for the following: signalling of TCI state configurations and RS configurations for L1 measurement configuration and reporting as part of LTM preparation and execution, Early TA acquisition procedure, and coexistence of LTM and legacy (L3) handover. The following agreements were made during the meeting.

RAN3 #121 agreements:

Notification of LTM triggering to gNB-CUs

Turn the WA to agreement that a new Class-2 procedure is introduced to notify gNB-CU about the LTM cell switch.

TCI state configuration

The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM.

After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE.

Selected beam transfer

The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU.

TA management

RAN3 prioritizes specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as open issue.

CU shall send the TA value to the source DU.

L1 measurement configuration and reporting

For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU.

Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3.

The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations.

TA acquisition

In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure.

Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

WA: Indicate the source gNB-DU ID when request the CFRA resources to the candidate DU.

WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

How the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU).

Source DU is in charge of checking the TA validity.

LTM and L3 handover collision

In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority.

In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority,

WA: In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

In this contribution, we continue the discussion on the following:

- Early TA acquisition and delivery of TA information.

- Enhancements to the LTM preparation procedure.

# 2 Early TA acquisition and delivery of TA information

It was agreed in a previous RAN2 meeting (RAN2 #122 [2]) that Early TA acquisition without RAR is the solution to be specified in Rel-18. In the last RAN3 meeting (RAN3 #121 [1]), RAN3 proceeded with this assumption and developed some aspects of the design of the Early TA acquisition procedure.

An unresolved issue was the details of the delivery of estimated TA information from a candidate DU to the source DU via the CU. We briefly recall the following understanding based on RAN1 and RAN2 agreements in the past several meetings regarding the configuration for Early TA acquisition, which aims to efficiently utilize the available CFRA resources at the candidate DUs (as well as at the serving DU in case of intra-DU LTM). For the remainder of the discussion in this section, we consider only the case of inter-DU LTM.

- A candidate DU provides to the serving DU a pool of RA preamble IDs (RAP IDs) per candidate cell. This set of RAP IDs can be assigned by the serving DU to a set of UEs that are connected to it and that are configured for LTM.

- If the serving DU determines that a UE can potentially perform LTM cell switch to a candidate cell, the serving DU transmits a PDCCH order to the UE to trigger the UE to perform Early TA acquisition on the candidate cell. The PDCCH order contains:

  - The candidate cell ID,

  - SSB index, and associated RAP ID and RACH occasion. The RAP ID is selected by the serving DU from the assigned pool of RAP IDs for the candidate cell.

Based on the CFRA parameters received in the PDCCH order as discussed above, the UE transmits a RACH preamble on the candidate cell including the provided RAP ID.

The candidate DU estimates the TA value based on the received RACH preamble from the UE. However, the candidate DU cannot determine the UE ID from the received RACH preamble since the RAP ID is from the pool of assigned RAP IDs for the candidate cell. Consequently, a Working Assumption was made in the last RAN3 meeting that non-UE associated F1-AP Class 2 procedure signalling is used to deliver the estimated TA value from a candidate DU to the serving DU via the CU.

Proposal 1. Agree the Working Assumption from the last RAN3 meeting that new Class 2 non-UE associated F1 signalling are used to deliver the TA information from a candidate DU to the source DU via CU.

Furthermore, since CFRA resources can be shared only among UEs in a single DU to avoid RACH access conflict between UEs from different DUs, a candidate DU should assign a different pool of RAP IDs to each serving DU which initiates an LTM procedure with it. For this, the candidate DU needs to be made aware during CFRA configuration for Early TA acquisition regarding the source DU. A related Working Assumption was made in the last RAN3 meeting.

Proposal 2. Agree the Working Assumption from the last RAN3 meeting that the source DU ID is indicated when CU requests CFRA resources for Early TA acquisition to a candidate DU.

The candidate DU transmits the estimated TA value to the serving DU. The serving DU then transmits the estimated TA value to the UE in the LTM cell switch MAC CE.

Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate DU determines the serving DU based on the RAP ID. The candidate DU then transmits to the CU the following:

- Estimated TA value,

- Detected RAP ID,

- Serving gNB-DU ID,

- Candidate cell ID.

The CU forwards the TA information to the serving DU. The serving DU can determine the UE ID from the TA information received from the CU.

Furthermore, since the serving DU transmits the TA value to the UE in the LTM cell switch command, upon receiving the TA value from a candidate DU, the serving DU needs to initiate a TA timer to keep track of the time for which the TA value is valid. The TA timer value should be determined by the candidate DU, and should be provided by the candidate DU to the serving DU along with the estimated TA value. Regarding what should be done on the network side in case the TA timer expires before LTM cell switch occurs, can be left FFS.

Proposal 3. Upon successfully receiving the RACH preamble from the UE on a candidate cell, the candidate DU estimates the TA value and determines the serving DU. The candidate DU transmits to the CU the following TA information (Class 2 non-UE associated F1 signalling):

- Estimated TA value,

- TA timer associated with the TA value,

- Detected RAP ID,

- Serving gNB-DU ID,

- Candidate cell ID.

FFS the signalling message used for this purpose.

Proposal 4. The CU forwards the TA information to the serving DU (Class 2 non-UE associated F1 signalling):

- Estimated TA value,

- TA timer associated with the TA value,

- Detected RAP ID,

- Candidate cell ID.

FFS the signalling message used for this purpose.

The signalling flow in Figure 1 summarizes the various aspects of Early TA acquisition and delivery of TA information discussed above.

Figure 1: Early TA acquisition and delivery of TA information

# 3 Enhancements to the LTM preparation procedure

In the last RAN2 meeting (RAN2 #122 [2]), the following agreements were made on L1 measurements for LTM:

- The RS configuration is provided to the UE per LTM candidate cell.

- RAN2 assumes that each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration.

- RAN2 assumes that the CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM.

- RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated).

- The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g., outside of the LTM candidate configuration.

- CSI reports for LTM candidates (neighbour cell reports for the purpose of LTM cell switch) are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted.

- RAN2 assumes the following about CSI measurement reporting for LTM (final decision up to RAN1):

  - UE reports all measured LTM candidate cells in a single report; or

  - UE reports one or a subset of measured LTM candidate cell(s) in a report.

In this section, we continue the discussion on the RAN3 impacts.

In LTM, the UE performs L1 measurements on candidate cells and transmits the corresponding L1 measurement reports to the serving DU. Based on the L1 measurement reports, the serving DU takes the decision on LTM cell switch for the UE.

Similarly, after performing LTM cell switch to a candidate cell, the UE performs L1 measurements on other candidate cells and transmits the corresponding L1 measurement reports to the candidate DU.

In order to perform L1 measurements on a candidate cell, the UE should be provided with the configuration of the relevant RSs transmitted on that cell, and for L1 measurement reporting, the UE should be provided with the reporting configuration.

Enhancements are required to the current LTM preparation procedure for signalling related to L1 measurement configuration. Some enhancements were agreed in the last RAN3 meeting (RAN3 #121 [1]):

- For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU.

- Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage3.

- The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations.

Figure 2 from the BL CR to TS 38.401 includes the enhancements required to the LTM preparation procedure for signalling related to L1 measurement configuration.

The following are some proposals on some of the remaining aspects of the solution.

Proposal 5. Upon receiving from the CU, the collected RS configuration from the candidate DUs, the source DU responds to the CU using UE Context Modification Response (Step 6 in Figure 2), including the following information:

- Generated LTM L1 measurement configuration, including L1 measurement reporting configuration, to be used by the UE when the UE is in the source cell,

- Prepared candidate cells of the source DU for subsequent LTM (including the source cell), and their RS configurations.

Proposal 6. The CU provides to each candidate DU using UE Context Modification Request (Step 7 in Figure 2) the collected RS configurations of candidate cells belonging to other candidate DUs and the source DU.

Proposal 7. A candidate DU responds, using UE Context Modification Response (Step 8 in Figure 2), providing generated LTM L1 measurement configurations, including L1 measurement reporting configurations, to be used by the UE when the UE is in the cells belonging to the candidate DU.

Proposal 8. The CU includes the LTM L1 measurement configurations provided by the candidate DUs and the source DU in the RRC reconfiguration message containing the LTM configuration provided to the UE (Steps 9, 10, in Figure 2).

Figure 2: Inter-gNB-DU LTM

# 5 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Early TA acquisition and delivery of TA information

Proposal 1. Agree the Working Assumption from the last RAN3 meeting that new Class 2 non-UE associated F1 signalling are used to deliver the TA information from a candidate DU to the source DU via CU.

Proposal 2. Agree the Working Assumption from the last RAN3 meeting that the source DU ID is indicated when CU requests CFRA resources for Early TA acquisition to a candidate DU.

Proposal 3. Upon successfully receiving the RACH preamble from the UE on a candidate cell, the candidate DU estimates the TA value and determines the serving DU. The candidate DU transmits to the CU the following TA information (Class 2 non-UE associated F1 signalling):

- Estimated TA value,

- TA timer associated with the TA value,

- Detected RAP ID,

- Serving gNB-DU ID,

- Candidate cell ID.

FFS the signalling message used for this purpose.

Proposal 4. The CU forwards the TA information to the serving DU (Class 2 non-UE associated F1 signalling):

- Estimated TA value,

- TA timer associated with the TA value,

- Detected RAP ID,

- Candidate cell ID.

FFS the signalling message used for this purpose.

Enhancements to the LTM preparation procedure

Proposal 5. Upon receiving from the CU, the collected RS configuration from the candidate DUs, the source DU responds to the CU using UE Context Modification Response (Step 6 in Figure 2), including the following information:

- Generated LTM L1 measurement configuration, including L1 measurement reporting configuration, to be used by the UE when the UE is in the source cell,

- Prepared candidate cells of the source DU for subsequent LTM (including the source cell), and their RS configurations.

Proposal 6. The CU provides to each candidate DU using UE Context Modification Request (Step 7 in Figure 2) the collected RS configurations of candidate cells belonging to other candidate DUs and the source DU.

Proposal 7. A candidate DU responds, using UE Context Modification Response (Step 8 in Figure 2), providing generated LTM L1 measurement configurations, including L1 measurement reporting configurations, to be used by the UE when the UE is in the cells belonging to the candidate DU.

Proposal 8. The CU includes the LTM L1 measurement configurations provided by the candidate DUs and the source DU in the RRC reconfiguration message containing the LTM configuration provided to the UE (Steps 9, 10, in Figure 2).

# References

[1] Chair notes, RAN3 #121 meeting.

[2] Chair notes, RAN2 #122 meeting.